Progress Inspection checklist template

| Inspector name: | Rick | Date | 15/02/2024 |
| --- | --- | --- | --- |
| Site location | Rockpool Pelican Waters |  |  |
| Trade being inspected | Ceilings & Partitions ITP |  |  |

Area preparation

- Grid lines and datum point identified and pinned. 					Y/N

- Concrete Rectification complete including corrects falls in wet areas. 			Y/N

- Door/ window openings water proofed. (Photos and spec sheet required). 		Y/N

- Sub Sills Installed. 									Y/N

Fire walls If Applicable

- Firewall Pre sheet - service trade sign off acknowledged.			 HOLD POINT

- Metal door frames installed and fire rated (checked ready for fitting of doors) 		Y/N

- Fire wall sheeting complete as per approved system. 				Y/N

- Fire Wall setting complete 								Y/N

- Fire wall mastic complete								Y/N

- Fire walls complete and ready for McNab inspection. 			HOLD POINT

Service trades

- Electrical, comms and security services installed as per latest plan ready to sheet	Y/N

- Plumbing services installed as per current plan ready to sheet			Y/N

- Mechanical services installed as per current plan ready to sheet			Y/N

Wall framing and sheeting

- Set -out in accordance with the latest plans and specifications. 			Y/N

- Studs /Nogs (Min 18mm thick) installed for Joinery, fittings & fixtures- (Photos showing dimensions). 									Y/N

- Access panels located and trimmed. 						Y/N

- Positive framing to all internals. 							Y/N

- Framing complete for all service penetrations.					Y/N

- Box Studs/ timber infills installed to door frame openings. 				Y/N

- All door openings are correct size, plumb and level. 					Y/N

- Ceiling Heights as per plans including bulkheads.					Y/N

- All ceiling frame members avoid lighting locations.					Y/N

- All service trades signed ITP off ready to sheet				HOLD POINT

- Pre sheet inspection With McNab.						HOLD POINT

- Insulation installed as documented.							Y/N

- Correct sheet type as per specification.						Y/N

- Stud & ceiling adhesive applied as per specification.					Y/N

- Fixed in accordance with manufactures recommendations.				Y/N

- All Services brought through sheeting.						Y/N

- Bulkheads installed true and plumb.							Y/N

- Expansion Joints considered and installed in areas where there is potential for cracking and at required intervals.								Y/N

- Area left clean and excess materials removed.					Y/N

- Inspection with McNab ahead of the painter. 					HOLD POINT