CPCCBC4026A Arrange building applications and approvals &

CPCCBC4031A Process client requirements

THEORY EXAM INSTRUCTIONS - This exam will occur under standard assessment conditions. - An oral examination may be allowed under certain circumstances. Answers will be documented and FT003 (Oral examination declaration form) must be completed. - Participants must provide enough detail in their responses to demonstrate their knowledge and skills. - Please fill in the details below, providing your name, signature and date of assessment. - Your signature acknowledges that this is your own work. - If a participant initially answers a question incorrectly but modifies their response, they must follow the instructions below: - If a participant changes their response in writing, they must put their initials next to the written change. - If a participant changes their response verbally, this must be noted on the exam paper by the assessor.

| Participant name | Debbie Cremer |
| --- | --- |
| Participant signature |  |

| Assessor name |  |
| --- | --- |
| Assessor signature |  |

| Date | 25/11/2018 |
| --- | --- |

| Number of questions | Number of questions answered correctly | Satisfactory? |  |
| --- | --- | --- | --- |
| 56 |  | Yes | No |

Identify four (4) details you would try to establish about the client’s design requirements.

| Answer | Client background – culture, family, interests, priorities, goals and future plans. Planned use of the project, expectations of finished product and preferred design styles Client budget & cost expectations or limitations. Contingency plans in case of budgetary overrun. Client timeframe expectations and any time restraints or deadlines to be met. Client expected involvement in the project development. Client specifications and preferences for the project including safety, quality, environment and sustainability, building materials, and anything else important to the client. Details of the site including location, environment, services, ground & soil conditions, flooding and drainage. |  |
| --- | --- | --- |

In what format should the client’s design requirements be recorded?

| Answer | Detailed project brief in the form of a clearly written and easy to understand report discussed with the client allowing input and feedback. Confirm they understand all details in report. |  |
| --- | --- | --- |

Describe how explaining a product’s strengths and limitations can ensure the client’s design needs are met.

| Answer | Client will understand the best products will be sought for their needs. They will get best value and quality for money and be within their budget. Environmentally friendly, innovative and local materials are used where possible. They will be assured the product is suitable for the particular use. |  |
| --- | --- | --- |

Identify two (2) types of technical/product knowledge you should provide to a client who wishes to maximise sustainability.

| Answer | Waste & water management Energy use – climate control stragegies. |  |
| --- | --- | --- |

Which of the following may be considered a restrictive covenant? 
Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Limit on building heights |  |
|  |  | Limit on fence heights |  |
|  |  | Limit on retaining wall heights |  |

Why should your client be notified about restrictive covenants?

| Answer | So they can design and plan within the covenant rules/regulations. Therefore not being in breach and risking legal action. They may need to change certain plans or expectations. |  |
| --- | --- | --- |

What document sets out what a client can build within the Council area?

| Answer | City Plan |  |
| --- | --- | --- |

- Outline the requirements of the Work Health and Safety Act 2011 with regards to the design of buildings.

| Answer | The design should not pose any health or safety risk to any persons using the building. |  |
| --- | --- | --- |

True or False (Tick your response): The client requesting a design from an architect automatically holds copyright of the design.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

Identify five (5) site features that would be identified when preparing a site plan.

| Answer | Services Vegetation Layout Existing buildings Site access |  |
| --- | --- | --- |

Explain the role of a drilling rig in establishing site costs.

| Answer | Drills the earth to test the ground condition for the foundations and identifies and problems that may incur extra cost to the client. |  |
| --- | --- | --- |

Identify three (3) site works activities that may be conducted by the builder.

| Answer | Setting out the site for construction Earthmoving Creating safe access to and from the site for personnel, plant and equipment |  |
| --- | --- | --- |

Identify two (2) documents that a client would receive in relation to site investigations and costs.

| Answer | Site Plan based on the site investigation showing site conditions Site report detailing site conditions and recommendations of adequate foundations and costs involved |  |
| --- | --- | --- |

Identify two (2) disclaimers you may need to give a client with regards to site costs.

| Answer | If ground conditions are found to be poor and require strengthened foundations resulting in increased cost. If block is sloping, additional levelling, cut, fill and retaining may be required at an additional cost |  |
| --- | --- | --- |

Outline why it is important that the requirements (including variation requests) for sketch plans and drawings should be accurately communicated to drafting personnel.

| Answer | To ensure drafting personnel understand the client’s requirements and draft plans and sketches accurately to minimise cost of re-doing drawings and making changes. Saving time and cost. |  |
| --- | --- | --- |

- What might be prepared by the architect to quickly and simply explore initial ideas for designs?

| Answer | Concept drawing |  |
| --- | --- | --- |

- Name three (3) types of detailed plans that may be forwarded to the client for approval.

| Answer | Plans 3D models Elevations |  |
| --- | --- | --- |

- Name three (3) costing details that should be able to be established from sketch plans.

| Answer | Floor area Access Facades |  |
| --- | --- | --- |

True or False (Tick your response): All documents should be sighted by the client, with details initialled.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

- What of the following are preliminary contracts that a builder could ask a client to sign prior to signing the main contract? Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Estimate |  |
|  |  | Handover certificate |  |
|  |  | Quote |  |

True or False (Tick your response): Pre-contract agreements may influence the final contract development.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

- Identify one (1) standardised commercial building contract and explain the maximum value of the work it may be used for.

| Answer | Basic Works Contract maximum $80,000.00 for work performed on a detached dwelling, for example renovations, repair, removal or re-sitting. |  |
| --- | --- | --- |

- Identify an appropriate standardised contract that would be appropriate for a house renovation exceeding $20,000.

| Answer | Level 2 – renovation extension, and repair contract |  |
| --- | --- | --- |

Which of the following parties could preliminary contracts be submitted to for checking and recording? Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Contract association (e.g. Master Builders) |  |
|  |  | Parliament |  |
|  |  | Solicitor |  |

- List two (2) permits that may need to be organised when preparing contract documents.

| Answer | Building Permit Electricity supply |  |
| --- | --- | --- |

- Outline the importance of preparing construction documents according to contract details.

| Answer | To ensure compliance with the contract terms and conditions so as not to incur breach of contract or any regulations and legislation. Services, subcontractors, materials and certifications are arranged in accordance with the contract and client’s expectations and timeframes. |  |
| --- | --- | --- |

For the purpose of development applications, describe how ‘development’ is defined in the legislation.

| Answer | Carrying out building work Carrying out plumbing or drainage work Carrying out operational work Reconfiguring a lot (i.e. subdivision) Making material change of use of premises (i.de. starting a new use, re-establishing a use that has been abandoned or changing the intensity or scale of the use) |  |
| --- | --- | --- |

At which of the following stages would a development approval be sought? Tick your response.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Town planning stage |  |
|  |  | Building approval stage |  |
|  |  | Construction stage |  |
|  |  | Final certification stage |  |

True or False (Tick your response): Building approval is separate to planning approval.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

True or False (Tick your response): Full approval is one type of approval that may be sought for the building approval stage.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

For each of the external specialists categories listed below, identify the information and documentation they may provide for an approval application.

| Answer | External specialist | Information and documentation provided |  |
| --- | --- | --- | --- |
|  | Building surveyors, quantity surveyors and site surveyors | Dial before you dig Costing report Aerial photos Site photos Site plans & drawings Elevation plans Cadastral (boundary) plans & information Topography plan (detailed plan) *Plans may include parts of neighbouring properties which may impact development |  |
|  | Geotechnical and environmental specialists | Soil/compaction test results & reports Geotechnical report & plans for bearing capacity of soil Methodology for footings and extra building supports if required Environmental permits if required Environmental impact study Vegetation and watercourse plans and information including any protected areas or habitats Sustainability features and information |  |
|  | Structural, mechanical and electrical engineers | Building Specifications including but not limited to footings, roof trusses, frames, termite protection, electrical & fire safety etc Scope of works 3D models Floor/building elevation/section plans & drawings Hydraulic/plumbing/drainage/electrical/engineering/detailed plans & drawings |  |

Name two (2) documents that are required under legislation to support a development application.

| Answer | Planning report and other technical reports demonstrating compliance with City Plan and (if required) the State planning Policy & State Development Assessment Provisions Plans of the development |  |
| --- | --- | --- |

What documents are required by Council to support a development application (in addition to those under Q32. )?

| Answer | Development services application Erosion hazard assessment form Land owner’s consent Planning reports including the zone code, neighbourhood plan code & any relevant overlays. Report should demonstrate compliance with City Plan Proposed plans including site plans, demolition plans, floor plans, detailed elevation plans |  |
| --- | --- | --- |

When arranging for building approval through a building certifier, you must give them scaled and detailed plans. Who should these plans be prepared by?

| Answer | Drafts person or architect |  |
| --- | --- | --- |

Name three (3) approvals that must be lodged through the Council, as opposed to through a building certifier.

| Answer | Town planning matters that are NOT accepted development Houses more than 9.5 metres high from natural ground level in most residential zones Redevelopment or removal of heritage and character buildings |  |
| --- | --- | --- |

What should be considered when developing a plan for the process of lodging approval applications?

| Answer | Compliance with building approval authority requirements Supporting documentation. Study council website for lodgement requirements Compliance with Planning Act 2016 (QLD), City Plan Stakeholder impacts and strategies to maximise their support for the development. Requirement of an impact assessment. Mitigating strategies if required Client requirements and expectations Lodgement fees |  |
| --- | --- | --- |

True or False (Tick your response): The plan in Q36. should be submitted for approval.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

The council assesses development under the application types listed below. Describe each assessment type and whether the public must be notified.

| Answer | Assessment type | Description |  |
| --- | --- | --- | --- |
|  | Code assessment | Assessed against all codes in the City Plan. Proposal must comply with all assessable benchmarks or provide acceptable information showing compliance with the purpose of the code Does NOT require public to be notified |  |
|  | Impact assessment | - Required when there is a need to consider the impacts of the development - Assessed against all relevant codes identified in the assessment table Part 5 of City Plan and any other relevant parts of the City Plan. Does require public to be notified |  |

Identify two (2) stakeholders that may be impacted by a planning application. Describe the impact on each stakeholder and strategies that could be adopted to maximise their support of the planning application.

| Answer |  | Stakeholder 1 | Stakeholder 2 |  |
| --- | --- | --- | --- | --- |
|  | Stakeholder type | Neighbours | Local council |  |
|  | Impact of planning application on stakeholder | Height (e.g. blocking sunlight, invading privacy). Drainage (e.g. stormwater being re-directed to neighbouring properties, flooding). Noise (e.g. insufficient acoustics, buildings too close) Excessive traffic | Extra pressure on stormwater/sewerage/water infrastructure. Increased traffic. On-street parking. Aesthetics. |  |
|  | Strategies adopted to maximise stakeholder support | Send letter detailing the proposed development and meet with neighbours well in advance of construction. Advise intended start date and site contact numbers. Discuss all aspects of the build and answer any questions. Be flexible with design where possible. Commit to keep neighbours informed each step of the way and advise when excessive noise, activity or deliveries may be taking place. Request letters of support for your development from neighbouring property owners. | Have a sustainability plan in place e.g. Rainwater tanks, recycling, grey water system, greenspaces, on-site sewerage treatment system, water saving devices fitted through-out building Ample on-site parking spaces with visitor parking. Safe access and egress of property with turn-circles. Funding for turning lane into property if required. Landscaping plan. Conservation of any existing vegetation where possible. Development design in keeping with the character of the locality. |  |

Identify the Council documentation that outlines whether your proposal will require an impact assessment.

| Answer | City plan - Zone map - Neighbourhood plan - Overlay map - Planning Regulation 2017 - Planning Act 2016 |  |
| --- | --- | --- |

Name three (3) projects that are always deemed ‘accepted development’.

| Answer | Building work if in the Commercial character building overlay. If building work involving only demolition internal to an existing building to facilitate internal building work or a fit-out. Reconfiguring a lot. If involving road widening or a truncation required as a condition of a development approval. Any development. If required to be carried out by a lawful order or instruction issued under any legislation. |  |
| --- | --- | --- |

True or False (Tick your response): Projects that are ‘accepted development, subject to requirements’ do not need anything in writing from the Council to commence the activity, provided that a self-assessment has been carried out to ensure compliance with the relevant code.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

Other than legislation relating to planning, name two (2) types of building legislation/requirements considered during the building approval process.

| Answer | Building Code of Australia QBCC Industry Standards |  |
| --- | --- | --- |

During the building approval process, which of the following issues are considered under various codes and regulations? Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Building is soundly/safely designed and constructed |  |
|  |  | Degree of fire safety |  |
|  |  | Energy and water efficiency standards |  |
|  |  | Protection from pests |  |
|  |  | Sewerage and drainage |  |

Identify and describe two (2) industry standards that you may refer to when preparing a building application (e.g. to address Q44.).

| Answer | Standard number | Description |  |
| --- | --- | --- | --- |
|  | AS/NZS 3500 | Plumbing and Drainage |  |
|  | AS 3786 | Smoke Alarms |  |

Identify and describe two (2) regulations that you may refer to when preparing a building application.

| Answer | Regulation name | Description |  |
| --- | --- | --- | --- |
|  | Building Regulation 2006 | Set out mandatory obligations relating to inspections, building permits, technical requirements and other administrative provisions requirements |  |
|  | Planning Regulation 2017 (QLD) | Supports the Planning Act 2016 and outlines rules and regulations regarding the carrying out of building work. |  |

Describe the importance of using document control processes when submitting an application.

| Answer | Using an efficient document control system will ensure all documents are the most current version and are all complete. It will enable you to manage the documentation and confirm that ALL relevant paperwork is included before lodging, reducing the risk of losing any important documents. It will also assist in tracking documents and monitoring the application status once lodged. Documents will be saved and stored in the correct manner so they are easily available when required. |  |
| --- | --- | --- |

How the currency of a plan determined?

| Answer | There will be a date and revision number on the plan. |  |
| --- | --- | --- |

True or False (Tick your response): To ensure continuing progress of your application, you should confirm the application status at appropriate intervals.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

Identify (and name, if applicable) whether there is an electronic portal used to check applications submitted to Council?

| Answer | The Brisbane City Council has a portal on its website called PD Online. |  |
| --- | --- | --- |

Order the following stages from 1 to 6 to identify the application assessment process used by the Council.

| Answer | Stage Number | Stage |  |
| --- | --- | --- | --- |
|  | 6 | Appeals |  |
|  | 1 | Application |  |
|  | 5 | Decision and conditions |  |
|  | 3 | Information request |  |
|  | 4 | Public notification |  |
|  | 2 | Referrals |  |

True or False (Tick your response): If a planning application is incomplete at the time of lodgement, the Council will always issue a ‘not properly made’ action notice.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

The applicant has how many business days to comply with an action notice?

| Answer | 20 business days |  |
| --- | --- | --- |

Identify three (3) common application problems.

| Answer | Unacceptable impacts the project may have on the environment i.e. vegetation, waterways and/or any nearby properties, people or public spaces e.g. excessive noise, overshadowing Submissions from the public raising concerns about the development Owners consent not gained or 0nly partially gained |  |
| --- | --- | --- |

True or False (Tick your response): When considering an appeal, the applicant should seek private legal advice.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

Minor amendments to an application should be negotiated with which of the following parties? Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Approval authority |  |
|  |  | Client |  |
|  |  | Organisation |  |