BSBOHS201A

Participate in OHS processes

ASSESSMENT SUMMARY

The assessment activities for BSBOHS201A Participate in OHS processes cover all aspects of the unit of competency required. To demonstrate competence in this unit, a participant must undertake all tasks and complete them satisfactorily and consistently. For Apprentices and Trainees, this includes employer completion of the Employer Record Book (ERB).

Participants must read all the information given before starting any assessment task. The assessment tasks are intended to be equitable, fair and flexible. If a participant has any questions or requires alternative arrangements to be made such requests should be directed to their Trainer/Assessor, who should note such arrangements in the ‘Assessor Comments’ on the affected assessment. If a participant feels they are not yet ready to be assessed, they should discuss their options with their Trainer/Assessor.

To satisfactorily complete the assessment, the participant is required to answer all questions correctly and demonstrate their skills to industry standards. If a participant does not answer some questions or perform some tasks, they will be deemed ‘Not Satisfactory’ for the task and ‘Not Yet Competent’ for the unit of competency. The Trainer/Assessor is able to provide the participant with additional information on interpreting the assessment outcomes and provide guidance on the participant’s options. Should a participant still be deemed ‘Not Satisfactory’ they will have the opportunity to undertake a supplementary assessment or appeal the result.

By signing the cover page for each assessment item, the participant acknowledges they understand the assessment instructions and requirements and consent to being assessed. The participant also verifies and assures the RTO that the work submitted is their own work.

| Participant Name | Jessica Proudley |
| --- | --- |

| Task | Assessment Results |  |
| --- | --- | --- |
| Theory Exam | Satisfactory | Not Yet Satisfactory |
| Practical Assessment | Satisfactory | Not Yet Satisfactory |
| Outcome for Unit of Competency | Competent | Not Yet Competent |

| Assessor Name |  |
| --- | --- |
| Assessor Signature |  |
| Unit Competence Determination Date | Click here to enter a date. |

BSBOHS201A

Participate in OHS processes

THEORY EXAM INSTRUCTIONS - This exam will occur under standard assessment conditions. - An oral examination may be allowed under certain circumstances, whereby the Participant’s answers will be documented by a party other than the Participant. This must be acknowledged in writing (e.g. FT001 and Assessor Comments) - Participants must provide enough detail in their responses to demonstrate their knowledge and skills. - Please fill in the details below, providing your name, signature and date of assessment. - Your signature acknowledges that this is your own work. - Assessors are to complete marking in pen of a different colour to the Participant. - If a Participant initially answers a question incorrectly but modifies their response, they must follow the instructions below: - If a Participant changes their response in writing, they must put their initials next to the written change. - If a Participant changes their response verbally, this must be noted on the exam paper by the assessor.

| Participant name | Jessica Proudley |
| --- | --- |
| Participant signature |  |
| Date | 20/12/2021 |

| Assessor name |  |
| --- | --- |
| Assessor signature |  |
| Date | Click here to enter a date. |

| Number of questions | Number of questions answered correctly | Task outcome (Tick) |  |
| --- | --- | --- | --- |
| 24 |  | Satisfactory | Not Yet Satisfactory |

ASSESSOR Comments

Match the terms and definitions by writing the appropriate letter next to each number in the centre column.

| Answer | Terms |  | Answers | Definition |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1 | PRA | 1 - F | A | Outline the requirements of the work to be undertaken. |  |
|  | 2 | JSA | 2 - E | B | Outline what to do in the event of fires and other accidents |  |
|  | 3 | Plans and specifications | 3 - A | C | Contains important information about hazardous materials or chemicals. |  |
|  | 4 | Manufacturer specifications | 4 - D | D | Outlines how tools and equipment should be used (e.g. lubricant levels, maintenance requirements). |  |
|  | 5 | SDS | 5 - C | E | Identifies hazards and potential hazards that may be encountered while working. |  |
|  | 6 | Emergency procedures | 6 - B | F | A personal assessment of risk |  |

For each of the hazards identified below, describe the injuries/damage they may cause (i.e. to fellow workers, other people and/or animals) and suggest a suitable control measure.

| Answer | Hazard | Injuries or damage | Control measure |  |
| --- | --- | --- | --- | --- |
|  | Hazardous chemical | Skin rashes, respiratory damage, chemical burns. | Use appropriate PPE - respirator, gloves, goggles and overalls. Where possible substitute with less hazardous chemicals. |  |
|  | Noise | Headache, fatigue, hearing damage. | Mechanical modification such as insulation. Use of appropriate PPE such as ear plugs. |  |
|  | Manual handling | Sprains, strains. | Ensure correct manual handling techniques are being utilised, upright neck, feet flat, shoulders level, natural back curve, good grip and knees bent. |  |

Who should you speak to if you find problems (e.g. hazards, accidents and emergencies)?

| Answer | Site or Project Manager, Supervisor and Health & Safety representative. |  |
| --- | --- | --- |

What action should be taken if plant, tools and/or equipment are found to be defective? Tick your response.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Report, Record |  |
|  |  | Stop, Report, Record |  |
|  |  | Stop, Tag, Report, Record |  |
|  |  | Stop, Fix, Report, Record |  |

Which legislation relates to safety in the workplace? Tick your response.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Occupational Health and Safety Act 2015 |  |
|  |  | Work Health and Safety Act 2011 |  |
|  |  | Safety Act 1900 |  |
|  |  | Safety and Health Act 2011 |  |

True or False (Tick your response): Employers are required by legislation to implement safety measures which are reasonably practicable.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

True or False (Tick your response): Workers are required by legislation to follow reasonable employer directives (e.g. to undertake first aid and/or fire safety training).

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

The following symbols appear under the heading ‘Required PPE’ on an SDS. What do the symbols represent?

| Answer | Symbol | Meaning |  |
| --- | --- | --- | --- |
|  |  | Overalls |  |
|  |  | Gloves |  |
|  |  | Goggles |  |
|  |  | Respirator |  |

Describe the meaning and use of the signs displayed below.

| Answer | Sign | Meaning |  |
| --- | --- | --- | --- |
|  |  | Location of first aid kit. |  |
|  |  | Warning to prevent entry of unauthorised persons |  |
|  |  | Location of fire fighting equipment/facilities |  |

Refer to the Risk Matrix below when responding to Q10. to Q12.

| Matrix | A: Death or permanent disability | B: Serious injury | C: Lost time injury | D: Casualty Treatment | E: First Aid |
| --- | --- | --- | --- | --- | --- |
| 1: Very Likely | E | E | H | H | M |
| 2: Likely | E | H | H | M | M |
| 3: Possible | H | H | M | M | L |
| 4: Unlikely | H | M | M | L | L |
| 5: Very Unlikely | M | M | L | L | L |

| E: Extreme | H: High | M: Medium | L: Low |
| --- | --- | --- | --- |

Which rating is designated for the highest risk and which rating is used for the lowest risk?

| Answer | Highest Risk: Extreme Lowest Risk: Low |  |
| --- | --- | --- |

What is the risk rating for the following circumstances: An event which is possible and would require low-level hospital treatment.

| Answer | Medium |  |
| --- | --- | --- |

What are the factors that would lead to a risk rating of E: Extreme?

| Answer | An event that is very likely (1) or likely (2) to result in death or permanent disability (A) or an event that is very likely (1) to cause serious injury (B). |  |
| --- | --- | --- |

When should safety checks be undertaken on machinery and equipment? Tick your response.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Before operation |  |
|  |  | After operation |  |
|  |  | Both before and after operation |  |

Order the elements of the hierarchy of control, from 1 to 6 (i.e. most effective to least effective).

| Answer | Label (1 – 6) | Element |  |
| --- | --- | --- | --- |
|  | 3 | Isolate the hazard |  |
|  | 6 | Provide and use Personal Protective Equipment (PPE) |  |
|  | 1 | Eliminate the hazard |  |
|  | 4 | Control the hazard by engineering means |  |
|  | 5 | Control the hazard by administrative means |  |
|  | 2 | Substitute the hazard |  |

Why is elimination the best method of hazard control and PPE considered the least effective?

| Answer | If a hazard is eliminated there is no longer any hazard or risk present. Whilst using PPE the hazard is still present and there is risk of it being used incorrectly – there is still the chance for human error making PPE the least effective. |  |
| --- | --- | --- |

If you were to replace a flammable solvent with a water-based solvent, what category of control measure would that be under the hierarchy of control?

| Answer | Substitute the hazard. |  |
| --- | --- | --- |

In the diagram below a person is lifting a tool box.
Filling in the table, indicate how each body part should be used/placed.

F. Hands

E. Knees

D. Shoulders

C. Feet

B. Back

A. Neck

| Answer | Area of body (Letter) | How it should be placed |  |
| --- | --- | --- | --- |
|  | D | Level |  |
|  | A | Upright |  |
|  | F | Good grip |  |
|  | B | Natural curve |  |
|  | E | Bent |  |
|  | C | Flat, facing load |  |

In a medical emergency, what is the first step in the DRSABCD action plan that you need to follow?

| Answer | Danger checked – checking for danger |  |
| --- | --- | --- |

In the event of a fire emergency, what should you do? 
Fill in the table below to explain the acronym R.A.C.E.

| Answer | Letter | Meaning |  |
| --- | --- | --- | --- |
|  | R | Remove people from immediate danger |  |
|  | A | Alert people of emergency |  |
|  | C | Confine fire and smoke |  |
|  | E | Evacuate to the assembly area |  |

What should you do as part of the clean up process? Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Store all plant, tools and equipment without cleaning |  |
|  |  | Maintain plant, tools and equipment |  |
|  |  | Dispose of materials that cannot be reused or recycled |  |
|  |  | Report faults in logbook and/or site records |  |
|  |  | Clean all plant, tools and equipment before storage |  |

Read the scenario below and complete the Hazard Report Form.

SCENARIO

Today at 10:45am a delivery truck reversed into a car park (located at 54 Standish Street, Salisbury) to deliver gardening materials. There was nobody in the car park to guide the driver and he backed into a number of drums of diesel located to the left of the photograph (below).  One drum was pierced and its contents erupted from the container.

The marked area on the photo represents the diesel spill, measuring 3m x 4m.

HAZARD REPORT FORM

| GENERAL |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Date | 20/12/2021 |  |  |  |  |  |
| Submitted by | Jessica Proudley |  |  |  |  |  |
| HAZARD DETAILS |  |  |  |  |  |  |
| Location | 54 Standish Street, Salisbury |  |  |  |  |  |
| Description of issue | A delivery truck reversed into a car park unguided and unassisted causing the truck to reverse into a number of diesel containing drums stored in the car park. As a result of the impact from the truck one of the drums was pierced leading to a diesel spill. |  |  |  |  |  |
| Risk class | Extreme | High | Medium |  |  | Low |
| CONTROL MEASURES |  |  |  |  |  |  |
| Corrective action required | Control and Isolate the hazard. Spill to be utilised and if available fire suppressant foam. Cordon off area to prevent entry of cars and people to reduce ignition sources. If possible and safe remove remaining drums from the vicinity. |  |  |  |  |  |
| Person to action | Store WHS officer and driver of truck. |  |  | When | Immediately Within 24 hours Within 7 days |  |

| Answer |  |  |
| --- | --- | --- |

Who would you submit a hazard report form to?

| Answer | Both the store and the delivery drivers WHS officer – if driver not employed by the store. |  |
| --- | --- | --- |

Who would approve proposed risk controls and organise the required resources for implementation of the risk control?

| Answer | Delivery driver and WHS officer/s |  |
| --- | --- | --- |

Identify one (1) way you can assess if risk controls have been effective.

| Answer | Return to inspect and check that hazardous materials are no longer being stored incorrectly. |  |
| --- | --- | --- |