# Building Report

### Jessica Manley 15/02/2021

##### Project Address: 13 Nineteenth Avenue, PALM BEACH QLD

Class of Building: The building shown in the drawings is a Class 9B building. This is a building of a public nature: a primary or secondary school. This is a Prep classroom in a primary school. This has been identified in the building assessment documents and drawing specifications.

Construction Type: This is a construction type C and has a floor area of 208m2 for the classroom and 65m2 for the deck, with a total floor area of 273m2.

Fire Resistance Level: The required FRL of the building shown in the drawing is 90/90/90 for all common and fire rated walls, and -/-/- for the two side external walls as there are no fire-source features in proximity of closer than 3m. There may be a few trees nearby (minimum proximity of 2000mm) so the back wall should have a fire rating of 60/60/60.

Distance from building to boundaries/potential fire source features:

- 3000mm from left side external wall to other existing building.

- 4000mm to boundary on back external wall. There may also be trees behind the external wall. These will be a minimum 2000mm away from the back external wall.

- No boundaries or potential fire source features in close enough proximity to the right-side external wall or front/veranda.

Bushfire Attack Level: This project has a BAL (Bushfire Attack Level) of low. This is due to the location of the project as there is minimal to low vegetation nearby, or in proximity, as specified in AS3959.

Passive Fire Protection: The passive fire protection in the construction of this building include soffits covered with 6mm compressed fibre cement. Also, the top of the external walls and the roof is sheeted in Colourbond metal cladding. These items are non-combustible.

Active Fire Protection: Complying with the NCC ‘Volume One’, active fire protection is placed throughout this Class 9b building. Items include fire extinguishers, smoke alarms and free-to-egress door hardware. Also, fire blankets and emergency luminaire lighting.

Product/Supplier: BetaBoard – 6.0mm HardieFlex (Compressed fibre cement). These sheets can be used on the bottom half of the building. HardieFlex sheets can achieve a FRL of up to 90/90/90 if a James Hardie system is used correctly. This product complies with the NCC ‘Volume One’ (C1.9 and C1.10) as it is a non-combustible product. HardieFlex sheets have a ‘Fire Propagation index’ of 0, and ‘spread of flame index’ of Class 1, as shown below.