CPCCBC4010B

Apply structural principles to residential low rise constructions

CPCCBC4011B

Apply structural principles to commercial low rise constructions

THEORY EXAM INSTRUCTIONS - This exam will occur under standard assessment conditions. - An oral examination may be allowed under certain circumstances. Answers will be documented and FT003 (Oral examination declaration form) must be completed. - Participants must provide enough detail in their responses to demonstrate their knowledge and skills. - Please fill in the details below, providing your name, signature and date of assessment. - Your signature acknowledges that this is your own work. - If a participant initially answers a question incorrectly but modifies their response, they must follow the instructions below: - If a participant changes their response in writing, they must put their initials next to the written change. - If a participant changes their response verbally, this must be noted on the exam paper by the assessor.

| Participant name | Shane palmer |
| --- | --- |
| Participant signature |  |

| Assessor name |  |
| --- | --- |
| Assessor signature |  |

| Date | Click here to enter a date. |
| --- | --- |

| Number of questions | Number of questions answered correctly | Satisfactory? |  |
| --- | --- | --- | --- |
| 93 |  | Yes | No |

Assessor Comments

What is the most important aspect in the design of buildings? 
Tick your response.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Aesthetics |  |
|  |  | Cost |  |
|  |  | Location |  |
|  |  | Stability |  |

Where can the definitive properties of materials used in the building industry be found? Tick your response.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Australian Standards |  |
|  |  | Building journals |  |
|  |  | Practical learning on the job |  |
|  |  | Textbooks on building |  |

Which industry professional would you contact for structural advice? Tick your response.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Architect |  |
|  |  | Draftsperson |  |
|  |  | Engineer |  |
|  |  | Quantity surveyor |  |

Which industry professional would you contact for the preparation of working drawings? Tick your response.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Building surveyor |  |
|  |  | Draftsperson |  |
|  |  | Quantity surveyor |  |

Briefly describe the role of an architect.

| Answer | Design buildings and in many cases supervise their construction. |  |
| --- | --- | --- |

What is the difference between a building surveyor and a land surveyor?

| Answer | Building surveyors provide professional advice on construction and property while land surveyors asses the surrounding terrain and landscape to be developed. |  |
| --- | --- | --- |

Which industry professional would you contact for matters relating to cost? Tick your response.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Architect |  |
|  |  | Building surveyor |  |
|  |  | Engineer |  |
|  |  | Quantity surveyor |  |

A land surveyor is to reinstate survey pegs. Which of the following documents would the surveyor need? Tick your response.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Title |  |
|  |  | Contour plans |  |
|  |  | Project specifications |  |

When preparing a trench a builder digs through a gas pipe which was not shown on the plans. Which report should be used to prove the mistake was not the builder’s? Tick your response.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Engineer’s drawings |  |
|  |  | Condition report |  |
|  |  | Soil report |  |

Which two (2) of the following can be determined from the contour report? Tick your responses.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Extent of cut and fill |  |
|  |  | Height of retaining walls |  |
|  |  | Location of trees |  |
|  |  | Location of windows |  |

The customer has requested a glass balcony. Which document would the builder refer to for information on structural members? 
Tick your response.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Building specifications |  |
|  |  | Condition report |  |
|  |  | Engineering report |  |

The builder is planning a project and requires information on the location of fixtures and fittings, windows and doors. Which plans show this information? Tick your response.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Cross sections |  |
|  |  | Floor plans |  |
|  |  | Footing designs |  |

After an inspection, the client believes that what was installed is different to what they had chosen in the contract. What document details the materials the builder was meant to use? Tick your response.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Building specifications |  |
|  |  | Plan of the subdivision |  |
|  |  | Site plan |  |

What key pieces of information can be found on a site plan? 
Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Location of street frontage |  |
|  |  | Location of proposed building |  |
|  |  | North point |  |

What is the purpose of a footing design?

| Answer | To provide details on footing depths and widths, slabs and stiffening beams and type and positioning of steel reinforcement for the proposed building. |  |
| --- | --- | --- |

Why would a pre-commencement site inspection be conducted? 
Tick your response.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | To ensure all contract documentation clearly reflects the actual site |  |
|  |  | It allows the supervisor to get some fresh air |  |
|  |  | To ensure that the client actually owns the property |  |
|  |  | So that levels can be taken to check the slope of the site |  |

What are the dead loads in a building? Tick your response.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | All stationary materials |  |
|  |  | Permanent materials |  |
|  |  | Roofing materials |  |
|  |  | Use for which the building is intended |  |

What is the formula for calculating a dead load?

| Answer | Dead load = volume of member x unit weight of materials (supplied) |  |
| --- | --- | --- |

What are the live loads in a building? Tick your response.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Loads arising from the function of the structure |  |
|  |  | Loads due to rain |  |
|  |  | Loads due to wind |  |
|  |  | Utility services |  |

Refer to the table shown below to respond to Q20. to Q21.  These values are used to calculate live loads using the formula: Area of floor x Live load rating.

| Live loads for floors as per building usage | Uniformly distributed load (kPa or kN/m²) |
| --- | --- |
| Houses | 1.5 |
| Flats, motel bedrooms, apartments | 2.0 |
| Offices | 3.0 |
| Workshops | 5.0 |
| Parking, vehicle >2.5t | 5.0 |
| Hospitals, school assembly areas with fixed seating | 3.0 |
| Dance halls, bars, lounges | 5.0 |

Calculate the live load of a guest house measuring 23m x 8m. 
Show your calculations.

| Answer | 184m2 x 1.5kpa = 276KN |  |
| --- | --- | --- |

Calculate the live load of a commercial office premises floor measuring 33m x 11m. Show your calculations.

| Answer | 363m2 x 3.0kpa = 1098KN |  |
| --- | --- | --- |

Please refer to the information below in responding to Q22. to Q25.

There is a building 21m long x 10m wide x 8m high with a 25⁰ pitched gable roof. The rafter line on the metal clad roof measures 5.5m on the rafter line (from the fascia/gutter line up to the ridge line).

The wind load is calculated as Area of surface x Wind load rate. The wind load rates are as follows:

- Windward wall is 21 metres long x 8 metres high with a wind load rate of +1.5KN/m²;

- Leeward wall is 21 metres long x 8 metres high with a wind load rate of -0.8kN/m²;

- Windward roof with a wind load rate of +0.8kN/m²; and

- Leeward roof with a wind load rate of -1.5kN/m²

What is the wind load of the windward wall? Tick your response.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | -252kN |  |
|  |  | 134.4kN |  |
|  |  | 252kN |  |
|  |  | 86.4kN |  |

What is the wind load for the leeward wall? Tick your response.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | -134.4kN |  |
|  |  | -94.2kN |  |
|  |  | 252kN |  |
|  |  | 134.4kN |  |

What is the wind load for the leeward roof? Tick your response.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | -173.25kN |  |
|  |  | 92.4kN |  |
|  |  | 173.25kN |  |
|  |  | -134.4kN |  |

What is the wind load for the windward roof? Tick your response.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | 252kN |  |
|  |  | -92.4kN |  |
|  |  | -252kN |  |
|  |  | 92.4kN |  |

Explain the term ‘load path’ in reference to a building.

| Answer | The direction in which each consecutive load will pass throught connected members. |  |
| --- | --- | --- |

Forces are represented as vectors and have three (3) properties. 
Name these properties.

| Answer | Magnitude, direction and position. |  |
| --- | --- | --- |

Name five (5) structural members that a building may consist of.

| Answer | Footings, columns, trusses, walls and floors. |  |
| --- | --- | --- |

What should be considered when determining the shape and type of material to use?

| Answer | Mechanical properties such as strength and toughness and section properties which is the mathematical properties of structural shapes |  |
| --- | --- | --- |

Match the mechanical properties and definitions below by writing the appropriate letter next to each number in the centre column.

| Answer | Mechanical properties |  | Answers | Definitions |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1 | Brittleness | 1 - G | A | The resistance the structural member is able to offer the load that will just cause fracture. |  |
|  | 2 | Elasticity | 2 - C | B | The resistance to a fracture point of a material by repeated bending or twisting after yield point has passed. |  |
|  | 3 | Ductility | 3 - E | C | Present if a material deforms with loading and then returns to its original shape when the loads are removed. |  |
|  | 4 | Hardness | 4 - H | D | If a material deforms when loaded and does not return to its original shape when the load is removed, causing permanent deformation. |  |
|  | 5 | Malleability | 5 - F | E | Enables a material to be drawn out in the direction of its length. |  |
|  | 6 | Plasticity | 6 - D | F | Can be hammered or otherwise worked into various shapes. |  |
|  | 7 | Strength | 7 - A | G | Causes material to break or snap off without any noticeable lengthening. |  |
|  | 8 | Toughness | 8 - B | H | The degree of resistance to indentation or abrasion. |  |

Match the beam types and descriptions by writing the appropriate letter next to each number in the centre column.

| Answer | Beam type |  | Answers | Description |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1 | Cantilevered | 1 - A | A | Overhang their supports. |  |
|  | 2 | Continuously supported | 2 - C | B | Supported at each end only. |  |
|  | 3 | Simply supported | 3 - B | C | Supported by 3+ support points. |  |

Match the column considerations and explanations by writing the appropriate letter next to each number in the centre column.

| Answer | Column consideration |  | Answers | Explanation |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1 | Axially loaded column | 1 - A | A | The load is central to the centre of gravity axis. |  |
|  | 2 | Buckling | 2 - E | B | The load is off-centre, resulting in bending and compressing stress being applied to the column. |  |
|  | 3 | End fixity and effective length | 3 - C | C | The rigid connection to other members at the top and bottom affects the way a column buckles. |  |
|  | 4 | Non-axially loaded column | 4 - B | D | Affects the maximum load a column can be allowed to support. |  |
|  | 5 | Slenderness ratio | 5 - D | E | Occurs at the weakness axis unless a lateral restraint is used to prevent it. |  |

Explain how a column is in compression.

| Answer | A column under compression is load bearing and subject to flexing, buckling and bending. |  |
| --- | --- | --- |

Explain the letters ‘CHS’ and ‘SHS’ with reference to columns.

| Answer | CHS is the most efficient shape a column can be and is a circular hollow section. SHS is the second most efficient shape and is a square hollow section. |  |
| --- | --- | --- |

Match the retaining wall types and descriptions by writing the appropriate letter next to each number in the centre column.

| Answer | Retaining wall |  | Answers | Description |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1 | Cantilevered retaining wall | 1 - B | A | Leans into the excavated soil and constructed at an angle between 45 and 80 degrees. Typically constructed from brickwork or pre-cast interlocking concrete units. |  |
|  | 2 | Gravity retaining wall | 2 - C | B | Relies upon the correct placement of steel reinforcement and varying thickness of concrete to resist the soil pressure. |  |
|  | 3 | Inclined retaining wall | 3 - A | C | Relies on its own weight to resist soil forces. Typically constructed from stone, brickwork or concrete. |  |

What is a ground level concrete slab floor?

| Answer | Slabs that are directly poured into excavated trenches in the ground that rely on existing ground support. |  |
| --- | --- | --- |

What should you consider when selecting the type of slab floor?

| Answer | the finished floor height edge rebates and always consult as 2870 to obtain required dimensions and specification for depth and widths. |  |
| --- | --- | --- |

True or False (Tick your response): Concrete slab floors are popular in the construction of structures because they take less time to construct.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

Match the following slab structural performance terms with the descriptions by writing the appropriate letter next to each number in the centre column.

| Answer | Term |  | Answers | Description |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1 | Foundation movement | 1 - C | A | Placed along the perimeter of the slab. Design is influenced by the reactivity of the soils. |  |
|  | 2 | Slab bending | 2 - B | B | Movement is classified as dishing or doming. |  |
|  | 3 | Stiffening beams | 3 - A | C | Addressed by regulating the moisture content of the soil. |  |

Which of these terms relate to a roof truss? Tick your response.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Chord, knee, web |  |
|  |  | Spider, pitch, chord |  |
|  |  | Ankle, heel, tar |  |
|  |  | Web, heel, pitch |  |

What are important considerations when undertaking demolition work? Describe relevant legislation, regulations and/or Codes of Practice relating to planning requirements, environmental standards, and safe work practices.

| Answer | Important consideration when undertaking demolition are the method of demolition, extent of temporary fencing and hoardings, protection of other properties, disconnection of services, correct order, hazardous materials to be removed, on site amenities, vermin control, traffic control, truck wash bays and environmental considerations. All these considerations are to be in accordance with Australian standards and codes of practise which can be found in work health and safety regulations 2011 (qld), demolition code of practise 2013(QLD) and AS 2601: the demolition of structures. |  |
| --- | --- | --- |

When demolition is being undertaken, which of the following items must be checked daily? Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | All boundaries are secured against unlawful entry |  |
|  |  | All hazardous materials are removed or covered as required by OHS legislation |  |
|  |  | All openings and elevated free edges are properly guarded |  |
|  |  | All potential fire and safety hazards are eliminated and disconnected services are secured |  |
|  |  | Asbestos is left on the nature strip for the garbage pickup |  |
|  |  | Demolished materials have not been left on any floor or structure if their weight exceeds the safe carrying capacity of the floor or structure |  |
|  |  | Emergency access routes are clear of debris |  |
|  |  | Footpaths and public access are clear of demolished materials |  |
|  |  | Neighbours have been notified not to hang out their washing |  |
|  |  | No structure or part of a structure is left in such a condition that it may become dangerous, such as collapse due to wind or vibration |  |
|  |  | Partly demolished elements are stabilised and loose materials are secured against bad weather |  |
|  |  | Signs and balloons are in place for the garage sale |  |
|  |  | Tell the demolisher to take a long lunch break as you don't need supervision |  |
|  |  | Temporary hoardings, bracing, shoring and propping is tight and secure and in place as required to protect both workers and public |  |

Identify two (2) requirements under the NCC with regards to construction in bushfire prone areas?

| Answer | Must be designed and constructed to reduce the risk of ignition from a bushfire , appropriate to the potential for ignition caused by embers, heat or flame and intensity of the bushfire attack on the building |  |
| --- | --- | --- |

What does AS3959 refer to?

| Answer | To bushfire risk based on the severity of the buildings potential exposure to ember attack, radiant heat and direct flame contact. |  |
| --- | --- | --- |

Which of the following can assist with providing sufficient restraint to transfer wind forces to the ground to prevent collapse, uplift or sliding off the footing systems? Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Anchorage systems |  |
|  |  | Bracing systems |  |
|  |  | Connecting structural elements together |  |

Which Australian Standard outlines the requirements of high wind areas?

| Answer | AS 1170.2 |  |
| --- | --- | --- |

The acceleration coefficient is a number which indicates the expected severity of the earthquake ground movement. Which Australian Standard should be referenced for this information?

| Answer | AS 1170.4 |  |
| --- | --- | --- |

True or False (Tick your response): Where the acceleration coefficient is greater than 0.15, there are additional NCC requirements than where the acceleration coefficient is 0.12.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

Which of the following areas require specific compliance in alpine areas? Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Access bridges |  |
|  |  | Access for emergency vehicles |  |
|  |  | External doorways |  |
|  |  | External ramps |  |

Identify the Volume and Parts of the NCC that relate to each of the following areas for Classes 1 and 10, and Classes 2 to 9.

| Answer | Topic | Classes 1 and 10 | Classes 2 to 9 |  |
| --- | --- | --- | --- | --- |
|  | Bushfire | Volume 2 part 3.10.5 | Volume 1 part G5 |  |
|  | High wind | Volume 2 part P2.1.2 | Volume 1 part b1 |  |
|  | Earthquake | Volume 2 part 3.10.2 | Volume 1 part B1 |  |
|  | Alpine area | Volume 2 part 3.10.4 | Volume 1 part G4 |  |
|  | New technologies | Volume 2 parts A2.2 and A5 | Volume 1 a2.2 and A5 |  |

How are emerging building technologies and methods assessed in the National Construction Code? Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Using verification methods |  |
|  |  | With ‘deemed to satisfy’ provisions |  |
|  |  | With documentary evidence |  |

True or False (Tick your response): Footings should be set out in accordance with the building plan.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

Name three (3) different types of footing systems for Classes 1 and 10, and Classes 2 to 9.

| Answer | Classes 1 and 10 | Classes 2 to 9 |  |
| --- | --- | --- | --- |
|  | Drilled or driven piles Waffle pod slabs Mas concrete piers | Columns or stumps Concrete slabs Piers and beams |  |

Identify the Volume and Parts of the NCC that relate to footings (including damp coursing and provision of termite barriers) for Classes 1 and 10, and Classes 2 to 9.

| Answer | Classes 1 and 10 | Classes 2 to 9 |  |
| --- | --- | --- | --- |
|  | Volume 2 part 3.2.5 |  |  |

For each of the following identify two (2) Australian Standards that relate to footings.

| Answer | Classes 1 and 10 | Classes 2 to 9 |  |
| --- | --- | --- | --- |
|  | AS 2870 AS 2159 |  |  |

Name two (2) materials that may be used for damp proof coursing.

| Answer | Corrosion resistant metal and impact resistant plastics |  |
| --- | --- | --- |

Name two (2) termite resistant materials.

| Answer | Concrete and masonry |  |
| --- | --- | --- |

Name three (3) different types of floor systems for Classes 1 and 10, and Classes 2 to 9.

| Answer | Classes 1 and 10 | Classes 2 to 9 |  |
| --- | --- | --- | --- |
|  | Concrete slab floor Panel systems of concrete and AAC Timber and steel floor construction | Slab on the ground Stiffened raft slab Beares and joists platform floor |  |

What should be considered when constructing a floor system?

| Answer | The type of flooring, laying of pipes and reinforcement, is it a concrete slab or does it require bearers and joists? |  |
| --- | --- | --- |

Which Australian Standard is referenced in Volumes 1 and 2 of the NCC regarding the requirements for slabs and footings?

| Answer | AS 2870 |  |
| --- | --- | --- |

Name three (3) different types of materials used for wall systems or structures falling under Classes 1 and 10, and Classes 2 to 9.

| Answer | Classes 1 and 10 | Classes 2 to 9 |  |
| --- | --- | --- | --- |
|  | Composite walls Earth walls Masonry walls | Cavity brick Timber Structural steel |  |

Identify the Volume and Parts of the NCC that relate to masonry wall systems for Classes 1 and 10, and Classes 2 to 9.

| Answer | Classes 1 and 10 | Classes 2 to 9 |  |
| --- | --- | --- | --- |
|  | Volume 2 part 3.2.5.6 | Volume 2 C3.11 F1.12 F5.2 |  |

Which Australian Standard is referenced in Volumes 1 and 2 of the NCC regarding masonry structures?

| Answer | As 3700 |  |
| --- | --- | --- |

Explain the purpose of bracing.

| Answer | The purpose of bracing is to provide lateral support to wall framing which enables framework to resist horizontal forces. |  |
| --- | --- | --- |

Explain the purpose of fixings and tie-downs.

| Answer | The purpose of fixings and tie downs is to interconnect various framing members to resist uplift and shear forces in the floor, wall and roof framing |  |
| --- | --- | --- |

Which Australian Standard relates to timber frames, bracing and fixings?

| Answer | AS 1684.2 |  |
| --- | --- | --- |

For the Australian Standard in Q66. which part relates to cyclonic areas?

| Answer | 1.2 companon documents 1684.3 Part 3 |  |
| --- | --- | --- |

Which of the following should be observed when checking the quality of the frame (whether, factory pre-cut and pre-nailed, factory pre-cut and assembled on site, or cut and assembled on site)? Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Fixings must be protected from corrosion |  |
|  |  | Fixings must be protected from termites |  |
|  |  | Provision has been made for plumbing services and fixtures |  |
|  |  | Tie downs and nominal nailing must both be present |  |
|  |  | Top and bottom plates have a tight fit on studs |  |
|  |  | Top plate is to be reinforced when rafters land on top of plates |  |
|  |  | Walls are tied together |  |
|  |  | Walls are plumb |  |
|  |  | Walls are square |  |

True or False (Tick your response): Sarking is a substitute for cladding and is regarded as the principal means of weatherproofing.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

What is flashing?

| Answer | A material that is used at corners and vertical joint and around openings to ensure that water is prevented from penetrating the wall frame cavity |  |
| --- | --- | --- |

Identify the Volume and Parts of the NCC that relate to wall cladding material requirements for Classes 1 and 10, and Classes 2 to 9.

| Answer | Classes 1 and 10 | Classes 2 to 9 |  |
| --- | --- | --- | --- |
|  | Volume 2 Part 3.54 | Specification j1.2 |  |

True or False (Tick your response): Timber can stand greater stresses than brickwork.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

True or False (Tick your response): Individual timber boards for cladding should be nailed together at the overlap.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

When should flashing materials be fixed to plywood cladding? 
Tick your response.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | After the cladding is fixed |  |
|  |  | Before the cladding is fixed |  |
|  |  | While the cladding is being fixed |  |
|  |  | At any convenient time |  |

What is the purpose of articulation joints? Tick your response.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | They allow for shrinkage in a building |  |
|  |  | They tie the brickwork to the frame |  |
|  |  | They allow for movement in a building |  |
|  |  | They seal the cavities |  |

When is the most appropriate time for the rough-in to occur for a brick veneer building constructed on a concrete slab Tick your response.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Before brickwork |  |
|  |  | After brickwork |  |
|  |  | After plasterboard |  |
|  |  | Any time |  |

Apart from plumbing and electrical, name four (4) other services that should be considered at rough-in.

| Answer | Dusting for heating and cooling Smoke control and containment systems Powered systems for operating doors and windows Communication systems |  |
| --- | --- | --- |

Identify the Volume and Parts of the NCC that relate to window installation for Classes 1 and 10, and Classes 2 to 9.

| Answer | Classes 1 and 10 | Classes 2 to 9 |  |
| --- | --- | --- | --- |
|  | Volume 2 part 3.6 | Volume 1 b1.4,f1.13,j3.4 |  |

Name two (2) Australian Standards that relate to the installation of windows.

| Answer | As 1288 AS 2047 |  |
| --- | --- | --- |

What is the purpose of a window schedule? Tick your response.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | To help work out the appropriate type of window |  |
|  |  | To work out the appropriate opening size of windows |  |
|  |  | To work out whether to use a timber or aluminium frame |  |
|  |  | To work out the height of a window |  |

Identify the Volume and Parts of the NCC that relate to door installation for Classes 1 and 10, and Classes 2 to 9.

| Answer | Classes 1 and 10 | Classes 2 to 9 |  |
| --- | --- | --- | --- |
|  | Volume 2 part 3.6.4.1 | Volume 1 D2.19 |  |

What Australian Standard must the design of a timber roof truss adhere to?

| Answer | AS 1720.1 |  |
| --- | --- | --- |

What is a roof system? Name three (3) roof types.

| Answer | The structure that forms the roof of a building Gable Hip Skillion |  |
| --- | --- | --- |

True or False (Tick your response): Roof trusses can be modified on site to suite different needs.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

Devise a checklist for ensuring the quality of an installed roof frame.

| Answer | Rise and pitch of roof against plan Correct propping of ridge, hips, valleys, rafters and under purlins at correct centres Adequate collar ties Line of rafter, hips and valleys Support to strutting beams Facia and barge boards All tie downs and fixings Ceiling joists and alignment with rafters Hanging beams Trimmers for location Trimming to manhole |  |
| --- | --- | --- |

Devise a checklist for ensuring the quality of an installed roof truss.

| Answer | Truss layout plan Manufactures specifications for tie-downs, bracing and lateral stability Roof truss certification Adequacy of supporting structure Dimension, overall width and length of wall for fit Support to girder trusses Plumb and spacings All tie-downs and fixings Location of specialised trusses Trusses have not been modified on site Eave overhangs Trimming to manhole |  |
| --- | --- | --- |

Name three (3) roof cladding materials.

| Answer | Slate tiles Concrete tiles Terracotta tiles |  |
| --- | --- | --- |

What pitch of roof should roof tiles be applied? Are there exceptions?

| Answer | They should be applied on roofs of 15 degrees or greater but some methods of manufacture require 25 degrees or greater pitch. |  |
| --- | --- | --- |

When installing corrugated metal roof sheeting, which is the preferred direction of laying sheets to improve side lap effectiveness for minimising leakage? Tick your response.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Away from the prevailing weather |  |
|  |  | Towards the north |  |
|  |  | Towards the prevailing weather |  |

True or False (Tick your response): Sarking is advisable under any roof.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

What Australian Standard relates to sarking?

| Answer | AS/NZS 4256 |  |
| --- | --- | --- |

Name three (3) items that may require roof penetration.

| Answer | Chimney Skylight Exhaust fans |  |
| --- | --- | --- |

Identify the sections/parts of NCC Volume 1 that relate to the installation of skylights.

| Answer | Part F.1.13 C IV B1.4 H ii D |  |
| --- | --- | --- |