CPCCBC4012B

Read and interpret plans and specifications

THEORY EXAM INSTRUCTIONS - This exam will occur under standard assessment conditions. - An oral examination may be allowed under certain circumstances. Answers will be documented and FT003 (Oral examination declaration form) must be completed. - Participants must provide enough detail in their responses to demonstrate their knowledge and skills. - Please fill in the details below, providing your name, signature and date of assessment. - Your signature acknowledges that this is your own work. - If a participant initially answers a question incorrectly but modifies their response, they must follow the instructions below: - If a participant changes their response in writing, they must put their initials next to the written change. - If a participant changes their response verbally, this must be noted on the exam paper by the assessor.

| Participant name | Numan Ur Rehman |
| --- | --- |
| Participant signature |  |

| Assessor name | Brett Stoward |
| --- | --- |
| Assessor signature |  |

| Date | 19/03/2019 |
| --- | --- |

| Number of questions | Number of questions answered correctly | Satisfactory? |  |
| --- | --- | --- | --- |
| 87 |  | Yes | No |

Assessor Comments

Plans and specifications are used to inform which of the following processes? Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Building contracts |  |
|  |  | Development applications |  |
|  |  | Estimation |  |
|  |  | Planning |  |
|  |  | Supervisory tasks |  |
|  |  | Tenders |  |

Which compass direction is always shown on plans and drawings?

| Answer | True North Point Direction |  |
| --- | --- | --- |

What information is provided on a location plan?

| Answer | Shows a site on a particular street is clear. It will show lot numbers, deposit plan number s and north point. |  |
| --- | --- | --- |

What detail does a site or block plan contain?

| Answer | Site plan provides specific detail of the block on which a development is proposed A Site or Block Plan is usually gives the following information: - Outline of site boundaries showing location of proposed building and existing dwellings - Street frontage names - Major geological and topographical features (trees and vegetation) - Datum points, contours and slopes - Driveways, stormwater drainage, easements and right of carriage - Soil and surface water drains, pipe sizes - Service runs from the house to mains - utility services location (sewer, water, gas, electricity) - Details of paving and retaining walls |  |
| --- | --- | --- |

What do floor plans identify?

| Answer | Floor plans shows detail of the proposed structure, such as rooms, positions of fixtures, type of floor coverings |  |
| --- | --- | --- |

What is a landscape plan and what information might it provide?

| Answer | Landscape plan shows soft and hard landscpaes details and also details of plants, paths, garden layouts and retaining walls |  |
| --- | --- | --- |

What are elevation views? What do they provide to the plan reader?

| Answer | Watching the house by standing infront of the wall. View of the proposed structure from its sides (i.e. from north, south, east and west). Each side is what is called an elevation view. |  |
| --- | --- | --- |

What is a section?

| Answer | It is a vertical view slice through the building. The section is a view of the house after it has been cut vertically at some point. Like the floor plan it shows the size and thickness of structural members and relationships between spaces. |  |
| --- | --- | --- |

What are detailed drawings?

| Answer | Detailed drawings is a particular method of construction. Detail drawings is part of final drawings and plans which are drawings that show overall sizes, levels and references to assembly drawings. |  |
| --- | --- | --- |

What are specifications?

| Answer | Specifications set out the minimum standards or codes required for all structural components and provide details of building materials and fittings, such as: Windows, frame material and type of glass Doors, size, material and the swing on the door Finishes/colours to walls, floors and ceilings Door types and models of locks, latches, hinges, push plates and pull handles |  |
| --- | --- | --- |

Explain the meaning of the abbreviations in the table below.

| Answer | Abbreviation | Meaning |  |
| --- | --- | --- | --- |
|  | ACRYL | Acrylic |  |
|  | AS | Australian Standard |  |
|  | ASSD | Assumed Datum |  |
|  | B | Basin |  |
|  | BN | Bull Nose |  |
|  | BTH | Bath |  |
|  | BWK | Brickwork |  |
|  | CB | Concrete Block |  |
|  | CF | Concrete Floor |  |
|  | CG | Clear Glass |  |
|  | CJ | Ceiling Joist |  |
|  | CORR | Corrugated |  |
|  | CWT | Cold water tank |  |
|  | DG | Double Glazing |  |
|  | DH | Double Hung |  |
|  | DPC | Damp Proof Course |  |
|  | DSB | Distribn. Switch Board |  |
|  | DW | Dishwasher |  |
|  | EJ | Expansion Joint |  |
|  | FPBD | Fibrous plaster board |  |
|  | FFL | Finished floor level |  |
|  | FHR | Fire Hose Reel |  |
|  | GPO | General purpose outlet |  |
|  | HBD | Hardboard |  |
|  | HTR | Heater |  |
|  | HWD | Hardwood |  |

Name each of the drawing symbol images shown in the table below.

| Answer | Name | Image |  |
| --- | --- | --- | --- |
|  | Wall - Cavity brick |  |  |
|  | Wall - Brick veneer |  |  |
|  | Wall - Timber stud |  |  |
|  | Window - In cavity wall |  |  |
|  | Window - In single wall |  |  |
|  | Door - Swing single |  |  |
|  | Door - Swing double |  |  |
|  | Door - Sliding into pocket |  |  |
|  | Door - Sliding exposed on wall |  |  |
|  | Socket outlet (power) – general |  |  |

Identify what the following window symbols represent on elevations.

| Answer | Item | Drawing |  |
| --- | --- | --- | --- |
|  | Hinged at side |  |  |
|  | Hinged at top |  |  |
|  | Sliding horizontally |  |  |
|  | Fixed |  |  |

What does a clouded area on a plan indicate?

| Answer | On plans, revisions or amendments are represented by a clouded area. It highlights the amended area on drawing. Revision or revised area |  |
| --- | --- | --- |

An amendment to specifications could relate to which of the following? Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Contact details of all construction crew members |  |
|  |  | Materials or products used |  |
|  |  | Methods of carrying out specified work |  |

Refer to the title block below to respond to Q16. to Q18.

Source: http://www.dtwd.wa.gov.au/sites/default/files/teachingproducts/BC1934_CCBY.PDF

On what date was an amendment made to the window details?

| Answer | 5/10/2011 window added to study |  |
| --- | --- | --- |

True or False (Tick your response): Details of the window in Q16. should also be reflected in the specifications and confirmed with the client.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

What amendment occurred on 08/11/2011?

| Answer | Bed 3 rob deleted |  |
| --- | --- | --- |

Refer to the site plan below to respond to Q19. to Q26.

What structure is being built on this site? Tick your response.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Residence |  |
|  |  | Road |  |
|  |  | Shed |  |
|  |  | Water meter |  |

What is the Lot number of this site?

| Answer | Lot 16 |  |
| --- | --- | --- |

What is the size of this lot?

| Answer | 3653 m2 |  |
| --- | --- | --- |

Is the shed at the north-east of the proposed structure to be removed or retained? Tick your response.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Removed |  |
|  |  | Retained |  |

True or False (Tick your response): The sewer is at the eastern boundary of the property.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

True or False (Tick your response): The driveway is located to the south of the proposed property.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

What size is the easement on the eastern boundary of the site?

| Answer | 5 meter |  |
| --- | --- | --- |

True or False (Tick your response): The stormwater drain is constructed from PVC pipe.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

Refer to the contour plan below to respond to Q27. to Q28.

What is the level at the northernmost point of the site?

| Answer | 3.94 |  |
| --- | --- | --- |

Which end of the site has the lowest level? Tick your response.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | East |  |
|  |  | West |  |

Refer to the sectional drawing below to respond to Q29. to Q49.

Give a brief description about the building from the detail available in the sectional drawing. Is it the same structure proposed in Q19. to Q26.?

| Answer | It’s a proposed residence for H&M smith going to built on lot 16 Glenvalley Road Richmond. It’s a 4 bedroom , family room and 2 verandas house refer to the drawing number WD 9733. Yes, it’s the same structure purposed in Q19. |  |
| --- | --- | --- |

Who is the client?

| Answer | H&M Smith |  |
| --- | --- | --- |

Where is this project to be built?

| Answer | Lot 16 Glenvalley Road Richmond |  |
| --- | --- | --- |

What are the contact details for the draftsman?

| Answer |  |  |
| --- | --- | --- |

What sheet number is this drawing?

| Answer | 5 of 7 |  |
| --- | --- | --- |

What timber is required for loading and non-load bearing walls?

| Answer | Pine |  |
| --- | --- | --- |

What spacing is recommended for common studs?

| Answer | 600 max spacing |  |
| --- | --- | --- |

What type of roof truss is to be installed?

| Answer | Scissor trusses 900 spaces Top chord 30 bottom chord 20 Every truss strapped to wall |  |
| --- | --- | --- |

What centres are the trusses positioned at?

| Answer | 900 mm centers |  |
| --- | --- | --- |

What size are the veranda posts?

| Answer | 90 x 90 |  |
| --- | --- | --- |

What timber species are the veranda posts to come from?

| Answer | OREGON |  |
| --- | --- | --- |

What pitch is the roof?

| Answer | 30 DGREES |  |
| --- | --- | --- |

What size are the noggings and at what spacing?

| Answer | 35 X 70 1350 MAX SPACING |  |
| --- | --- | --- |

What size are the ceiling joists?

| Answer | HJ 240 Joists |  |
| --- | --- | --- |

What timber is used for roof battens and what spacing?

| Answer | 35 X 70 FG PINE AT 900 SPACING |  |
| --- | --- | --- |

What thickness is the plasterboard sheeting?

| Answer | 10 mm plaster board sheeting + ceiling |  |
| --- | --- | --- |

What type of flooring is to be installed?

| Answer | 15 mm plywood flooring |  |
| --- | --- | --- |

What is the roof cladding material?

| Answer | Colorbond corrugated metal roofing |  |
| --- | --- | --- |

Where will you find information about the footings?

| Answer | Engineers drawing |  |
| --- | --- | --- |

What do the letters ‘DAR’ refer when timber ordering?

| Answer | Dressed all round |  |
| --- | --- | --- |

Explain what ‘240 x 63 LVL’ refers to on the sectional drawing. What do the letters ‘LVL’ refer to?

| Answer | Starting joist between family room and varendah. LVL refers to Laminated Veneer Lumber |  |
| --- | --- | --- |

Refer to the drawing below to respond to Q50. to Q65.

What is the number of the drawing?

| Answer | 2 |  |
| --- | --- | --- |

What is the Job Number?

| Answer | 00R - 112 |  |
| --- | --- | --- |

What is the scale of the drawing?

| Answer | 1:100 |  |
| --- | --- | --- |

What does the drawing show?

| Answer | Elevations – North, South, east and west side. Section of the house |  |
| --- | --- | --- |

Which view shows the front of the house?

| Answer | East Elevation |  |
| --- | --- | --- |

How many motor vehicles can be parked in the garage?

| Answer | 2 |  |
| --- | --- | --- |

What are the window and doors made from?

| Answer | Windows from aluminium And doors from timber |  |
| --- | --- | --- |

What is the size of the eaves?

| Answer | 450 mm |  |
| --- | --- | --- |

What are the eaves lined with?

| Answer | Hariflex sheeting |  |
| --- | --- | --- |

What type of roof structure is the roof built from?

| Answer | Timber framed roof truss |  |
| --- | --- | --- |

Which Clause of the NCC should be read for more detail about roof structure in Q59. ?

| Answer | Clause 3.11.6 |  |
| --- | --- | --- |

Which Australian Standard should be read for more detail about the roof structure in Q59. ?

| Answer | AS 17203. |  |
| --- | --- | --- |

What is the roof cladding materials made from?

| Answer | In Australia steel, concrete tiles and terracotta tiles are the most popular material for cladding |  |
| --- | --- | --- |

Which Clause of the NCC should be read for more detail about the roof cladding materials in Q62. ?

| Answer | NCC volume 2 - 3.5.1.0 and 3.5.1.2 |  |
| --- | --- | --- |

Which Australian Standard should be read for more detail about the roof cladding materials in Q62. ?

| Answer | AS 2049. |  |
| --- | --- | --- |

What is the roof pitch?

| Answer | 22.5 degrees |  |
| --- | --- | --- |

Identify one (1) regulatory body you may need to submit plans to for approval?

| Answer | City council |  |
| --- | --- | --- |

Briefly describe the approval process and timeframes for an approval through the regulatory body in Q66.

| Answer | Approval process is Stage 1: Pre-lodgement (Getting it right at the start) Stage 2: Lodgement and initial administration by council of your application Stage 3: Assessment (What happens to my DA now?) Stage 4: Determination (The Decision) Stage 5: After decision – get your Construction Certificate (approval to build) Stage 6: Get your Occupation Certificate (OC) and celebrate It takes from 2 weeks to 3 months. |  |
| --- | --- | --- |

What is a prime cost (PC) item? Give an example.

| Answer | A PC item is: - An amount of money included in a contract sum to purchase a specified item such as tiles, taps, doors or bathroom fittings - An agreed estimated amount is included at contract signing but the specific products are not selected until a later stage - A builder must estimate the cost of such items at or above the lowest amount these items could reasonably cost |  |
| --- | --- | --- |

What is a provisional sum (PS) item? Give an example.

| Answer | A provisional sum is (toilet, bath): - An amount of money included in contract sum to cover work and/or materials, not detailed when entering the contract - Typically builders will include a PS for site work costs - The builder is legally required to take reasonable steps to ensure an accurate site works cost estimate but unforeseen events mean that the provisional sum is exceeded which is added to the contract sum |  |
| --- | --- | --- |

Refer to the Bill of Quantities below to respond to Q70. to Q76.

Source: http://organizedbuilders.com.au/sites/default/files/BOQ-Summary.pdf

True or False (Tick your response): This Bill of Quantities represents a job for a two-storey house.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

Identify two (2) materials being used for the floors?

| Answer | May include carpet vinyle and floating floor |  |
| --- | --- | --- |

True or False (Tick your response): A landscaper has been contracted for this job.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

How much does the contracted plasterer cost?

| Answer | 12,900 |  |
| --- | --- | --- |

How much do the kitchen appliances cost?

| Answer | 2700 |  |
| --- | --- | --- |

What percentage of the total cost does the fixing stage represent of the building contract?

| Answer | 23% |  |
| --- | --- | --- |

What percentage of the total cost does the completion stage represent of the building contract?

| Answer | 17% |  |
| --- | --- | --- |

Refer to the Purchase Order below to respond to Q77. to Q82.

Source: http://organizedbuilders.com.au/sites/default/files/Databuild-Bill-of-Quantities.pdf

How many Pine F5 90 x 45 studs (2.4m long) are required?

| Answer | 100 |  |
| --- | --- | --- |

If the material in Q77. increased in cost to $2.75 each, what would be the total cost for this material?

| Answer | 2.75 X 100 = 275.00 dollars |  |
| --- | --- | --- |

How many sheet of ply are required in total (i.e. all sizes)?

| Answer | 13 sheets |  |
| --- | --- | --- |

What material has the highest total cost?

| Answer | 90 X 45 =mgp 10 (long sheets) |  |
| --- | --- | --- |

How many metres are required of the material in Q80. ?

| Answer | 322.45 meter |  |
| --- | --- | --- |

What beam is identified as being a better option than F17?

| Answer | E14 |  |
| --- | --- | --- |

Refer to the labour chart below to respond to Q83. to Q86.

Source: https://www.archsd.gov.hk/media/89170/sor2013_vol1_builders_works_2013_11_04.pdf

Based on the hourly rates given, which is the least expensive trade to contract?

| Answer | Asphalter (Road construction) |  |
| --- | --- | --- |

If the trade in Q83. were to be contracted for two (2) hours’ work, how much would this cost?

| Answer | 107 X 2 = 214.80 dollars |  |
| --- | --- | --- |

Based on the hourly rates given, which is the most expensive trade to contract?

| Answer | Bar bender and fixer |  |
| --- | --- | --- |

If the trade in Q85. were to be contracted for six (6) hours’ work, how much would this cost?

| Answer | 214.30 X 6 = 1285.60 |  |
| --- | --- | --- |

Name the building legislation (Act and Regulations) that apply within your jurisdiction and should be referenced when reading plans and building contracts.

| Answer | Building Act 1975 QLD Building Regulation 2006 |  |
| --- | --- | --- |