# Building Report

### Jessica Manley 11/02/2021

##### Project Address: LOT 8 REES COURT, PALM BEACH QLD

Class of Building: The building shown in the drawings is a Class 1A building. This is clear as it is shown that this is a detached single dwelling. This is a residence house as it shows bedrooms, bathrooms and living rooms.

Total floor area: 359.25 m2

Distance from building to boundaries/potential fire source features:

- 3700mm from the back of the house to the next boundary

- 2015 from the right side of the house to the boundary beside it

- 840mm from the ledge of the pool on the left side, to the boundary.

- No boundaries or potential fire source features in close enough proximity to this project.

Bushfire Attack Level: This project has a BAL (Bushfire Attack Level) of low. This is due to the location of the project as there is minimal to low vegetation nearby, or in proximity, as specified in AS3959.

Passive Fire Protection:

- Sarking on external walls

- 4.5mm Fibre cement sheets on eaves and fascia

- External walls to be brick veneered

- Non-combustible roofing and gutters

Product/Supplier: This Fibre Cement villa board used (James Hardie Fibre Cement 9mm Villaboard), has a fire rating of up to 60 minutes. This complies with the performance requirements outlined in BCA Vol 2 section 3.7.:
3.7.2.4 Construction of external walls
External walls (including gables) required to be fire-resisting (referred to in 3.7.2.2 or 3.7.2.5) must—
(i) Commence at the footings or ground slab, except where the external wall commences above a separating wall complying with 3.7.3.2 (see Figure 3.7.2.2b)
(ii) Extend to—
(A) The underside of a non-combustible roof covering, except that a wall may terminate not more than 200 mm from the underside of a non-combustible roof covering, where the area between the external wall and underside of the roof covering is sealed with a non-combustible fascia, gutter or flashing; or
(B) The underside of a non-combustible eaves lining (See Figure 3.7.2.3); and
(iii) Be constructed in accordance with (b).
(B) A wall required by (a) must—
(i) Have an FRL of not less than 60/60/60 when tested from the outside

Active Fire Protection: Types of active fire protection required in the construction of the building as detailed in BCA Vol 2 Section 3.7.:

- Plasterboard

- Perforated gypsum lath with a normal paper finish

- Fibrous-plaster sheet

- Pre-finished metal sheeting having a combustible surface finish not exceeding 1 mm thick and where the Spread-of-Flame Index of the product is not more than 0

- Sarking-type materials that do not exceed 1 mm in thickness and have a flammability index not greater than 5

- Bonded laminated materials where—

(i) Each lamina, including any core, is non-combustible; and
(ii) Each adhesive layer does not exceed 1 mm in thickness and the total thickness of the adhesive layers does not exceed 2 mm; and

(iii) The Spread-of-Flame Index and the Smoke-Developed Index of the bonded laminated material as a whole do not exceed 0 and 3 respectively

Recommendation: Another alternative external cladding which can be used could be James Hardie Matrix Cladding. This also have a fire rating of up to 60 minutes. This is also a better fit for the environment as it is pre-cut and pre-finished. It has better thermal properties and goes up to R3.3 and it is easier to install than standard cladding.

Scyon_Matrix_Installation_Guide_220620_web.pdf (ctfassets.net)