CPCCBC4029B

Apply construction information to the sales process

ASSESSMENT SUMMARY

The assessment activities for CPCCBC4029B Apply construction information to the sales process cover all aspects of the unit of competency required. To demonstrate competence in this unit, a participant must undertake all tasks and complete them satisfactorily and consistently. For Apprentices and Trainees, this includes employer completion of the Employer Record Book (ERB).

Participants must read all the information given before starting any assessment task. The assessment tasks are intended to be equitable, fair and flexible. If a participant has any questions or requires alternative arrangements to be made such requests should be directed to their Trainer/Assessor, who should note such arrangements in the ‘Assessor Comments’ on the affected assessment. If a participant feels they are not yet ready to be assessed, they should discuss their options with their Trainer/Assessor.

To satisfactorily complete the assessment, the participant is required to answer all questions correctly and demonstrate their skills to industry standards. If a participant does not answer some questions or perform some tasks, they will be deemed ‘Not Satisfactory’ for the task and ‘Not Yet Competent’ for the unit of competency. The Trainer/Assessor is able to provide the participant with additional information on interpreting the assessment outcomes and provide guidance on the participant’s options. Should a participant still be deemed ‘Not Satisfactory’ they will have the opportunity to undertake a supplementary assessment or appeal the result.

By signing the cover page for each assessment item, the participant acknowledges they understand the assessment instructions and requirements and consent to being assessed. The participant also verifies and assures the RTO that the work submitted is their own work.

| Participant Name | Virginia Hemsley |
| --- | --- |

| Task | Assessment Results |  |
| --- | --- | --- |
| Theory Exam | Satisfactory | Not Yet Satisfactory |
| Practical Assessment | Satisfactory | Not Yet Satisfactory |
| Outcome for Unit of Competency | Competent | Not Yet Competent |

| Assessor Name |  |
| --- | --- |
| Assessor Signature |  |
| Unit Competence Determination Date | Click here to enter a date. |

CPCCBC4029B

Apply construction information to the sales process

THEORY EXAM INSTRUCTIONS - This exam will occur under standard assessment conditions. - An oral examination may be allowed under certain circumstances, whereby the Participant’s answers will be documented by a party other than the Participant. This must be acknowledged in writing (e.g. FT001 and Assessor Comments) - Participants must provide enough detail in their responses to demonstrate their knowledge and skills. - Please fill in the details below, providing your name, signature and date of assessment. - Your signature acknowledges that this is your own work. - Assessors are to complete marking in pen of a different colour to the Participant. - If a Participant initially answers a question incorrectly but modifies their response, they must follow the instructions below: - If a Participant changes their response in writing, they must put their initials next to the written change. - If a Participant changes their response verbally, this must be noted on the exam paper by the assessor.

| Participant name | Virginia Hemsley |
| --- | --- |
| Participant signature |  |
| Date | 25/03/2021 |

| Assessor name |  |
| --- | --- |
| Assessor signature |  |
| Date | Click here to enter a date. |

| Number of questions | Number of questions answered correctly | Task outcome (Tick) |  |
| --- | --- | --- | --- |
| 18 |  | Satisfactory | Not Yet Satisfactory |

ASSESSOR Comments

Outline the circumstances in which a building licence must be obtained before carrying out building work.

| Answer | -Where the work is valued over $3,300; -Building work over $1,100 involving Hydraulic Services Design; -Building work of any value involving: Drainage, plumbing & drainage, gas fitting, termite chemical management, fire protection, completed residential building inspection, Building design – low, medium rise & open site classification |  |
| --- | --- | --- |

Describe the steps involved in organising electricity supply to site. Provide approximate timeframes for this process.

| Answer | If the site does not have available supply the electrical contractor can make an enquiry with the Distribution Network Service Provider (DNSP) for an indicative estimate of costs (client has 20 bus days to make a decision) If supply available, the electrical contractor will need to enter into Network Connection Agreement with the DNSP. If the application is for a standard connection contract, the DNSP will advise of an offer within 10 bus days After agreeing to connection offer, electrical contractor will prepare the Premises for connection and the electrical contractor will submit an Electrical Work Request (EWR) to the DNSP and a copy sent to the client’s preferred electricity retailer Once all forms received, DNSP will visit the premises to carry out the work within 5 bus days |  |
| --- | --- | --- |

True or False (Tick your response): Staged approval is one type of approval that may be sought for the building approval stage.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

True or False (Tick your response): Building provision approvals are issued by the local government (e.g. Brisbane City Council).

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

Describe the steps involved in organising water supply to site. Provide approximate timeframes for this process.

| Answer | -Submit a Water Approval application to Qld Urban Utilities -Council will require evidence there is a connection available -Standard connections for simple developments usually take up to 5 days |  |
| --- | --- | --- |

Identify the Volume and Parts of the NCC which outline energy efficiency requirements for Classes 1 and 10, and Classes 2 to 9.

| Answer | Classes 1 and 10 | Classes 2 to 9 |  |
| --- | --- | --- | --- |
|  | Volume 2 A6.1 – Class 1 A6.10 – Class 10 | Volume 1 A6.2 to A6.9 |  |

Describe the long-term benefits of sustainable building solutions. List one (1) environmental benefit and one (1) benefit for the client.

| Answer | Topics | Definition |  |
| --- | --- | --- | --- |
|  | Environmental benefit | Using building products that incorporate recycled materials reduces the need for new raw material e.g recycled timber, paper etc reduces need for growth/harvest of new trees |  |
|  | Client benefit | Long term savings in energy bills – Electricity, gas – lighting, aircon Water – recycling, rain tanks Noise reduction – double glazed windows |  |

Match the insurance types and definitions by writing the appropriate letter next to each number in the centre column.

| Answer | Insurances |  | Answers | Definition |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1 | Builder’s All Risk | 1 -B | A | Covers personal injury or property damage. |  |
|  | 2 | Indemnity | 2 -C | B | Covers accidental risks of physical loss or damage during construction, third party liabilities and advance loss of profits. |  |
|  | 3 | Public Liability | 3 -A | C | Covers costs where a contractor has caused building errors or omissions. |  |
|  | 4 | Workers’ Compensation | 4 -D | D | Covers death, injury or illness of employees. |  |

True or False (Tick your response): Insurance should be taken out by the contractor prior to starting construction.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

Select and describe five (5) trades involved in the construction process.

| Answer | Trade | Description |  |
| --- | --- | --- | --- |
|  | Bricklayer | Constructs the brick base i.e sub-base, brickwork, walls and piers Installs walls |  |
|  | Tiler | Installs wall and floor tiles |  |
|  | Painter | Paints walls, ceilings, doors |  |
|  | Carpenter | Installs wall frames, roof trusses, windows, doors, joinery, flooring, decks |  |
|  | Landscaper | External drainage, paving, gardens etc |  |

Order the following steps from 1 to 10 to identify an appropriate construction sequence.

| Answer | Step Number | Steps |  |
| --- | --- | --- | --- |
|  | 3 | Floor frame |  |
|  | 5 | Windows and external door frames |  |
|  | 7 | Wall insulation |  |
|  | 1 | Set out |  |
|  | 9 | Painting |  |
|  | 6 | Electrical wiring |  |
|  | 2 | Plumbing of waste pipes |  |
|  | 8 | Internal linings |  |
|  | 10 | Landscaping |  |
|  | 4 | Wall frames |  |

For each of the main stages of construction listed below, identify the main trades involved and an approximate duration of each stage for a standard.

| Answer | Stage | Description |  |  |
| --- | --- | --- | --- | --- |
|  | 1: Clearing of the site | Trades | Earthmoving contractor – excavator – remove any rubbish, trees etc Silt Fence Installer Soil Tester |  |
|  |  | Duration | 1 – 2 days (Varies) |  |
|  | 2: Slab or base stage | Trades | Surveyor – set out Excavator – level site, remove topsoil, footings Plumber/Drainer – pipes, drains Concretor – footings/piers, formwork/boxing, slab Bricklayer – sub-floor brickwork, dwarf walls Termite barrier supplier/installer Reinforcement contractor |  |
|  |  | Duration | 1 – 2 weeks |  |
|  | 3: Frame stage | Trades | Carpenter – floor frame, wall frame, roof trusses, windows, door frames Electrician – wiring Plumber – roof and guttering, rough in Bricklayer – walls Gasfitter – gas pipes if gas appliances are used Roofing contractor |  |
|  |  | Duration | 3 – 4 weeks |  |
|  | 4: Lockup stage | Trades | Carpenter – windows, doors, Plumber Electrician Cabinetmaker Brickalyer |  |
|  |  | Duration | 4 weeks |  |
|  | 5: Fit-out or fixing stage | Trades | Plasterer – ceilings, walls, cornices Carpenter – joinery and fix out Plumber Water proofer Insulation contractor – roof and wall insualtion Tiler – wall and floor tiles Electrician – aircon, ducted heating etc |  |
|  |  | Duration | 5 – 6 weeks |  |
|  | 6: Practical completion stage | Trades | Painter – external and internal walls, skirting boards Plumber – shower heads, taps, toilets, shower screens Electrician – power points, light fittings, phone/data points, appliances Carpenter – internal doors, handles, latches Carpet/wood floor installer Landscaper Cleaner |  |
|  |  | Duration | 7 – 8 weeks |  |

Identify five (5) factors that can affect the duration of each construction stage.

| Answer | Location Size of the project Project function Height of the building Complexity of the building |  |
| --- | --- | --- |

Describe how each of the following factors can affect construction times.

| Answer | Factor | Effect |  |
| --- | --- | --- | --- |
|  | Changed conditions | If the project is altered, timeframes will change especially if additional materials and/or labour is required |  |
|  | Labour availability | Organising trades in the correct process sequence so each trade can perform their tasks in a timely manner so as not to hold up the following trades |  |
|  | Material availability | If materials are not ordered/delivered when the subby is ready to do their work, this can hold up other trades. Rare or difficult materials should be ordered well in advance |  |
|  | Weather | Can delay trades from completing their work which has a flow on affect to finishing the build on time |  |

Describe the site establishment requirements for each of the points listed.

| Answer | Requirement | Description |  |
| --- | --- | --- | --- |
|  | Earthworks | Ensure service location has been done so as not to cause damage to any existing services when excavation is carried out |  |
|  | Electricity | Temp electricity - Install meter box on a pole Connect site to main power supply |  |
|  | Neighbours | Provide notice to adjoining properties of start date, site contact numbers. Ensure neighbours can access their property Signage |  |
|  | Waste disposal | Rubbish bins/skip bins delivered to site and set up in a designated area so as not to block site access |  |
|  | Water connection | Plumber to perform any disconnections prior to any demo works Plumber to install pipes and drainage before slab poured |  |

Identify the section of the Work Health and Safety Regulations that outlines the site establishment requirements listed.

| Answer | Regulation section | Requirement |  |
| --- | --- | --- | --- |
|  | 40 | The layout of the work area must allow for persons to enter, exit and move about without risk to health and safety |  |
|  | 41 | Adequate facilities must be provided for workers, including toilets, drinking water, washing facilities and eating facilities. |  |
|  | 308 | A principal contractor for a construction project must install signs outside the work area that shows their name and telephone contact numbers and site office address. |  |

True or False (Tick your response): The owner is able to access the site at any time.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

Which of the following parties should the client contact to access the site? Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Contractor |  |
|  |  | Local Government |  |
|  |  | Industry Regulator (e.g. Queensland Building and Construction Commission) |  |
|  |  | Subcontractors |  |

CPCCBC4029B

Apply construction information to the sales process

PRACTICAL ASSESSMENT INSTRUCTIONS - The following practical assessment will require the Participant to complete work related activities. - The Assessor will conduct observations of the Participant’s work to determine whether they display sufficient practical skills and knowledge. - Please fill in the details below – providing your name, signature and date of assessment. - The Participant’s signature acknowledges that the Assessor provided clear instructions and the Participant understood what was required to complete this assessment. - The Assessor may substitute a Practical Task with an alternate task of equal value. - Alternate tasks must be fully documented by the assessor in the Assessor Comments area.

| Participant name | Virginia Hemsley |
| --- | --- |
| Participant signature |  |
| Date | 25/03/2021 |

| Assessor name |  |
| --- | --- |
| Assessor signature |  |
| Date | Click here to enter a date. |

| Task | Task outcome (Tick) |  |
| --- | --- | --- |
|  | Satisfactory | Not Yet Satisfactory |
| Task 1 (Occasion 1) |  |  |
| Task 1 (Occasion 2) |  |  |

Assessor Comments

CPCCBC4029B

Apply construction information to the sales process

| Practical Task 1 (2X) |  |  |  |
| --- | --- | --- | --- |
| Description of task: On two (2) occasions, a suitable observer is to verify the participant’s ability to apply construction information during the sales process in order to develop the customer’s knowledge of construction processes and the effects on the contract and building timelines. The participant is to gather evidence of how they have completed each component of the Practical Task, which is to be submitted with this Practical Assessment sheet. Evidence to be submitted may include: - RFI and relevant correspondence with client - A variation, price and acceptance by the client The observer is to provide detailed comments on the tasks completed by the participant. The assessor is to verify the observer’s comments in the Assessor Comments section, and provide the Practical Assessment outcome on the front cover of this document. |  |  |  |
| Resources required: Two (2) sales contracts. |  |  |  |
| Assessment of task |  |  |  |
| Name of participant: |  |  |  |
| Name of observer: |  |  |  |
| Role of observer (Confirm suitability to sign) |  |  |  |
| Email address of observer |  |  |  |
| Signature of observer: |  |  |  |
| Date: | Click here to enter a date. |  |  |
| When completing the task, did the participant: |  | Tick if satisfactorily completed. Provide comments if left unticked. |  |
|  |  | 1st Occasion | 2nd Occasion |
| - Identify and explain to the customer the approvals required before starting construction, including: - Statutory approvals - Sustainability requirements - Insurance covers |  |  |  |
| - Identify and explain to the customer the stages and timeframes of construction, including: - Main trade components - Construction process sequence (including approximate duration) |  |  |  |
| - Identify and explain to the customer potential industry conditions that can affect construction times. |  |  |  |
| - Identify and explain to the customer site establishment requirements before starting construction. |  |  |  |
| - Establish site access arrangements with the client. |  |  |  |
| - Effectively communicate using the following methods: - Verbally using language/concepts appropriate to cultural differences - Non-verbal communication - Active listening and responding to client questions |  |  |  |
| The participant’s performance was: Satisfactory Not Satisfactory |  |  |  |

observer Comments