Jeremy Tydd

CPCCBC4012 Read and interpret plans and specifications

Any documentation or photographic evidence in support of reading and interpreting plans and specifications (e.g. any modifications that the candidate has applied to existing project plans, environmental plans, plans/drawings and specifications)

10X75mmx250mm plate has been added in on top of the posts for the beams to be bolted to make the structure stronger. This can be seen in the attached photos

All structural steel works is in accordance with AS4100, AS1418, AS/NZS1554

C1 Members are 100x 5mm SHS Columns, these are the two large posts holding the structure up on the outside.

C2 members are 76x4.5 mm CHS  and there is 4 off them the are able to be a little smaller as there is 4, as 76mm sits on the top plate of the house a lot neater.