CPCCBC4002A

Manage occupational health and safety in the building and construction workplace

ASSESSMENT SUMMARY

The assessment activities for CPCCBC4002A Manage occupational health and safety in the building and construction workplace cover all aspects of the unit of competency required. To demonstrate competence in this unit, a participant must undertake all tasks and complete them satisfactorily and consistently. For Apprentices and Trainees, this includes employer completion of the Employer Record Book (ERB).

Participants must read all the information given before starting any assessment task. The assessment tasks are intended to be equitable, fair and flexible. If a participant has any questions or requires alternative arrangements to be made such requests should be directed to their Trainer/Assessor, who should note such arrangements in the ‘Assessor Comments’ on the affected assessment. If a participant feels they are not yet ready to be assessed, they should discuss their options with their Trainer/Assessor.

To satisfactorily complete the assessment, the participant is required to answer all questions correctly and demonstrate their skills to industry standards. If a participant does not answer some questions or perform some tasks, they will be deemed ‘Not Satisfactory’ for the task and ‘Not Yet Competent’ for the unit of competency. The Trainer/Assessor is able to provide the participant with additional information on interpreting the assessment outcomes and provide guidance on the participant’s options. Should a participant still be deemed ‘Not Satisfactory’ they will have the opportunity to undertake a supplementary assessment or appeal the result.

By signing the cover page for each assessment item, the participant acknowledges they understand the assessment instructions and requirements and consent to being assessed. The participant also verifies and assures the RTO that the work submitted is their own work.

| Participant Name | Michael Carter |
| --- | --- |

| Task | Assessment Results |  |
| --- | --- | --- |
| Theory Exam | Satisfactory | Not Yet Satisfactory |
| Practical Assessment | Satisfactory | Not Yet Satisfactory |
| Outcome for Unit of Competency | Competent | Not Yet Competent |

| Assessor Name |  |
| --- | --- |
| Assessor Signature |  |
| Unit Competence Determination Date | Click here to enter a date. |

CPCCBC4002A

Manage occupational health and safety in the building and construction workplace

THEORY EXAM INSTRUCTIONS - This exam will occur under standard assessment conditions. - An oral examination may be allowed under certain circumstances, whereby the Participant’s answers will be documented by a party other than the Participant. This must be acknowledged in writing (e.g. FT001 and Assessor Comments) - Participants must provide enough detail in their responses to demonstrate their knowledge and skills. - Please fill in the details below, providing your name, signature and date of assessment. - Your signature acknowledges that this is your own work. - Assessors are to complete marking in pen of a different colour to the Participant. - If a Participant initially answers a question incorrectly but modifies their response, they must follow the instructions below: - If a Participant changes their response in writing, they must put their initials next to the written change. - If a Participant changes their response verbally, this must be noted on the exam paper by the assessor.

| Participant name | Michael Carter |
| --- | --- |
| Participant signature |  |
| Date | 28/05/2020 |

| Assessor name |  |
| --- | --- |
| Assessor signature |  |
| Date | Click here to enter a date. |

| Number of questions | Number of questions answered correctly | Task outcome (Tick) |  |
| --- | --- | --- | --- |
| 28 |  | Satisfactory | Not Yet Satisfactory |

Assessor Comments

For each of the common building and construction jobs listed below, identify one (1) hazard specific to that job.

| Answer | Job | Risks |  |
| --- | --- | --- | --- |
|  | Construction equipment use (e.g. power tools, concrete mixers) | Excessive noise |  |
|  | High risk construction equipment use (e.g. lasers, EPT) | Debris in eye |  |
|  | Crane use | Contact with powerlines. |  |
|  | Demolition | Live services |  |
|  | Formwork | Falls from Height |  |
|  | Scaffolding use | Falling Objects |  |
|  | Work at heights | Falls from heights |  |
|  | Welding | Burns |  |

Using the risk table below, determine the level of risk for each of the hazards identified in Q1. Identify how each risk level was determined, by identifying the likelihood and consequences of each hazard.

| Matrix | A: Death or permanent disability | B: Serious injury | C: Lost time injury | D: Casualty Treatment | E: First Aid |
| --- | --- | --- | --- | --- | --- |
| 1: Very Likely | E | E | H | H | M |
| 2: Likely | E | H | H | M | M |
| 3: Possible | H | H | M | M | L |
| 4: Unlikely | H | M | M | L | L |
| 5: Very Unlikely | M | M | L | L | L |

| E: Extreme | H: High | M: Medium | L: Low |
| --- | --- | --- | --- |

| Answer | Job | Level of risk |  |
| --- | --- | --- | --- |
|  | Construction equipment use (e.g. power tools, concrete mixers) | 1C High |  |
|  | High risk construction equipment use (e.g. lasers, EPT) | 1D High |  |
|  | Crane use | 3A High |  |
|  | Demolition | 3A High |  |
|  | Formwork | 3B High |  |
|  | Scaffolding use | 3B High |  |
|  | Work at heights | 3A High |  |
|  | Welding | 2B High |  |

Identify an appropriate control measure for each of the hazards identified in Q1.

| Answer | Job | Control measure |  |
| --- | --- | --- | --- |
|  | Construction equipment use (e.g. power tools, concrete mixers) | Use of hearing protection. |  |
|  | High risk construction equipment use (e.g. lasers, EPT) | Use of eye protection |  |
|  | Crane use | De-energise or insulate lines. |  |
|  | Demolition | Isolate all live services. |  |
|  | Formwork | Use of a leading edge system. Placement of joists from underneath, |  |
|  | Scaffolding use | Use of kickboards and unimesh. |  |
|  | Work at heights | Edge protection. Fall arrest system. |  |
|  | Welding | Protective clothing. |  |

True or False (Tick your response): Risk control options can come from both internal sources (e.g. policies and procedures) and external sources (e.g. codes and standards).

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

Name two (2) workplace personnel you may speak to for safety advice.

| Answer | Workplace health and safety officer. Foreman. |  |
| --- | --- | --- |

Referring to the WHS Regulations, explain how the following documents contribute towards identifying hazards and risks in the workplace.

| Answer | Document | Description |  |
| --- | --- | --- | --- |
|  | Safe Work Method Statements | Section 299 of the WHS regulations states that a SWMS must identify hazards relating to high risk construction work and risks to health and safety associated with those hazards. |  |
|  | Site Safety Plan / WHS Management Plan | Section 309 of the WHS regulation states that a WHS management plan must include arrangements for the collection, monitoring and review of SWMS which then identify hazards and risks related to the workplace. |  |

Identify two (2) advisory standards you might refer to safely undertake building and construction work and briefly summarise their requirements.

| Answer | Standard name | Requirements |  |
| --- | --- | --- | --- |
|  | WHS Regulations | The work health and safety regulations describe how to prevent or minimise a risk at your workplace. |  |
|  | How to Safely Remove asbestos code of practice. | The code of practice gives practical guidance on how to achieve standards required under the WHS act and WHS regulations and how to identify hazards and manage risks when removing asbestos. |  |

Building and construction industry contracts require building legislation and the National Construction Code to be observed. Refer to these documents when responding to Q8. to Q10.

Identify the requirements in the National Construction Code Volume 1 with regards to air pressurisation for fire-isolated exits. Identify the relevant section of the Code and the Australian Standard referenced.

| Answer | Code section(s) | Requirements and Standards |  |
| --- | --- | --- | --- |
|  | D1.7 | If more than 2 access doorways open to a required fire isolated exit in the same story the exit must be pressurised in accordance with AS 1668.1. |  |

Identify the requirements in the National Construction Code Volume 2 with regards to smoke alarms. Identify the relevant section of the Code and the Australian Standard referenced.

| Answer | Code section(s) | Requirements and Standards |  |
| --- | --- | --- | --- |
|  | Section 3.7.5.2 | Smoke alarms must be located in a class 1a and 1b building, comply with AS 3786, be powered by mains and be interconnected where there is more than one. |  |

Name the building legislation (Act and Regulations) that apply within your jurisdiction.

| Answer | Building Act 1975. Building Regulation 2006. |  |
| --- | --- | --- |

Which of the following should be considered when developing a risk control plan? Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | How and when controls will be implemented |  |
|  |  | Evaluation processes for implemented controls |  |
|  |  | Consultation with individuals and parties |  |
|  |  | Responsibilities for implementation |  |

True or False (Tick your response): If control measures do not adequately control risks, they must be revised.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

Which of the following are appropriate techniques, tools and processes for identifying hazards and risks? Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Workplace inspections |  |
|  |  | Compensation claim information |  |
|  |  | Safe Work Method Statements |  |
|  |  | Information issued by WHS regulators |  |

Which of the following can be used to assist with workplace inspections? Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Hazard and risk checklists |  |
|  |  | Job Safety Analysis (JSA) |  |
|  |  | Manifests and registers |  |
|  |  | Surveys |  |
|  |  | Workplace walk-through |  |

Which of the following are limitations of using a generic hazard/risk assessment checklist? Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Specifics of the workplace are not considered |  |
|  |  | Not all checklist points will be applicable |  |
|  |  | Such checklists/processes must be completed on a computer |  |
|  |  | You always have to purchase these checklists |  |

Identify the applicable WHS Act in your jurisdiction. Identify its requirements for PCBUs with regards to identifying hazards and risks.

| Answer | Act name and section(s) | Requirements |  |
| --- | --- | --- | --- |
|  | Work Health and safety act 2011 | Work health and safety act 2011 states that consultation is required when identifying hazards and assessing risks to health and safety. |  |

Identify the applicable WHS Regulation in your jurisdiction. Identify its requirements for PCBUs with regards to identifying hazards and risks.

| Answer | Regulation name and section(s) | Requirements |  |
| --- | --- | --- | --- |
|  | Work health and safety Regulations 2011 | Workplace health and safety regulations 2011 state that a duty holder must identify reasonably foreseeable hazards that could give rise to risks to health and safety. |  |

Identify a relevant policy in your organisation with regards to identifying hazards and risks and briefly describe its requirements.

| Answer | Policy name | Requirements |  |
| --- | --- | --- | --- |
|  | HSEQ Management plan. | The process of hazard identification, risk assessment and the formulation and implementation of appropriate control measures to mitigate risks is a dynamic one which involves all persons engaged in the project. |  |

Identify the statutory requirements in your jurisdiction with regards to the completion of inspection reports.

| Answer | Legislation name and section(s) | Requirements |  |
| --- | --- | --- | --- |
|  | WHS Act 2011 Section 103E | States that the work health and safety office must prepare a report for the assessment carried out under 103(d) and if the assessment identifies any risks to health and safety arising from the work carried out conduct of the business or undertaking- include in the report recommendations about managing the risks. |  |

Which of the following individuals and parties may require information about workplace hazards and proposed risk controls? 
Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Health and Safety Committees |  |
|  |  | PCBU |  |
|  |  | All members of the public |  |
|  |  | Workers |  |

Identify the requirements in the Code of Practice with regards to consulting parties when managing risks.

| Answer | Legislation name and section(s) | Requirements |  |
| --- | --- | --- | --- |
|  | Work health and safety consultation, co-operation and coordination Code of practice 2011 Section 2.1 | The code states that Consultation is required with workers who will be effected when identifying hazards, assessing risks and deciding on measures to control those risks. |  |

Identify the requirements in the Code of Practice with regards to consulting parties when developing procedures for communicating WHS policy and practice requirements.

| Answer | Legislation name and section(s) | Requirements |  |
| --- | --- | --- | --- |
|  | Work health and safety consultation, co-operation and coordination Code of practice 2011 Section 2.4 | The Code states that you must consult with affected workers and also that procedures should be in writing and easily accessible. |  |

Name three (3) parties who should consult to determine effective strategies for communicating WHS policy and practice.

| Answer | Direct employees, labour hire staff and subcontractors. |  |
| --- | --- | --- |

Name two (2) communication strategies that may be used in the building and construction industry.

| Answer | Toolbox talks/meetings. Memos/emails. |  |
| --- | --- | --- |

Name two (2) education programs that may be implemented in the building and construction industry.

| Answer | Induction training. Mental health awareness training. |  |
| --- | --- | --- |

Fill in the blanks for the statement below:

Consulted..

training

The ‘How to manage work health and safety risks Code of Practice 2011’ identifies that control measures are more effective when ______________________ is provided and when workers are _____________________.

| Answer |  |  |
| --- | --- | --- |

Explain why it is important to review the effectiveness of communications and educational programs.

| Answer | To ensure that the training is working and that risks are being reduced. |  |
| --- | --- | --- |

List three (3) methods for reviewing the effectiveness of communication and educational programs.

| Answer | Post Training surveys. Reviewing statistics related to the training. Reviewing training materials for relevance. |  |
| --- | --- | --- |

CPCCBC4002A

Manage occupational health and safety in the building and construction workplace

PRACTICAL ASSESSMENT INSTRUCTIONS - The following practical assessment will require the Participant to complete work related activities. - The Assessor will conduct observations of the Participant’s work to determine whether they display sufficient practical skills and knowledge. - Please fill in the details below – providing your name, signature and date of assessment. - The Participant’s signature acknowledges that the Assessor provided clear instructions and the Participant understood what was required to complete this assessment. - The Assessor may substitute a Practical Task with an alternate task of equal value. - Alternate tasks must be fully documented by the assessor in the Assessor Comments area.

| Participant name | Michael Carter |
| --- | --- |
| Participant signature |  |
| Date | 28/05/2020 |

| Assessor name |  |
| --- | --- |
| Assessor signature |  |
| Date | Click here to enter a date. |

| Task | Task outcome (Tick) |  |
| --- | --- | --- |
|  | Satisfactory | Not Yet Satisfactory |
| Task 1 (Occasion 1) |  |  |
| Task 1 (Occasion 2) |  |  |

Assessor Comments

CPCCBC4002A

Manage occupational health and safety in the building and construction workplace

| Practical Task 1 (2X) |  |  |  |
| --- | --- | --- | --- |
| Description of task: On two (2) occasions, a suitable observer is to verify the participant’s ability to conduct and report on a WHS risk analysis, including workplace inspection. The participant is to gather evidence of how they have completed each component of the Practical Task, which is to be submitted with this Practical Assessment sheet. Evidence to be submitted may include: - A copy of a Site Specific Safety Plan or Construction Safety Plan, including: - Copies of Risk Assessments completed on a project - Copies of Safety Work Method Statements used on a project - A copy a workplace safety inspection checklist that has been used on a project The observer is to provide detailed comments on the tasks completed by the participant. The assessor is to verify the observer’s comments in the Assessor Comments section, and provide the Practical Assessment outcome on the front cover of this document. |  |  |  |
| Resources required: Computer. |  |  |  |
| Assessment of task |  |  |  |
| Name of participant: |  |  |  |
| Name of observer: |  |  |  |
| Role of observer (Confirm suitability to sign) |  |  |  |
| Email address of observer |  |  |  |
| Signature of observer: |  |  |  |
| Date: | Click here to enter a date. |  |  |
| When completing the task, did the participant: |  | Tick if satisfactorily completed. Provide comments if left unticked. |  |
|  |  | 1st Occasion | 2nd Occasion |
| - Conduct a workplace inspection to evaluate construction site safety for the range of occupations in the workplace according to WHS legislation and company policies, including: - Identifying construction hazards and potential risk areas - Identifying and prioritising hazards - Identifying and prioritising risks |  |  |  |
| - Seek expert advice as appropriate, including: - Advice from workplace personnel regarding the application of building codes and standards - Consulting with relevant parties regarding compliance with statutory requirements (including SWMS and site safety plans) - Consulting with appropriate personnel to determine effective strategies for communicating WHS policy. |  |  |  |
| - Complete an inspection report in accordance with best practice and statutory obligations: including: - Recommendations from findings - Required approaches to remediation |  |  |  |
| - Implement agreed control measures in conjunction with relevant workplace personnel, implementing negotiation and conflict resolution skills, as necessary. |  |  |  |
| - Monitor and review control measures, updating records with findings |  |  |  |
| - Establish communication strategies and educational programs specific to the building and construction industry in accordance with statutory requirements and best practice. |  |  |  |
| - Review the effectiveness of communication and educational programs, updating records with findings. |  |  |  |
| - Effectively communicate using the following methods: - Verbally using language/concepts appropriate to cultural differences - Non-verbal communication |  |  |  |
| The Participant’s performance was: Satisfactory Not Satisfactory |  |  |  |

observer Comments