4009

Practical Activity 1

You are required to make an application for a low rise builders licence with the QBCC. Access www.qbcc.qld.gov.au and follow the prompts to complete an application form for a low rise builder’s licence. Once you have completed the application form DO NOT SUBMIT & PAY but rather print the form and attach as part of your practical assessment with  all necessary supporting documentation for your application (with the exception of your certificate 4 building and construction qualification/ units).

Practical Activity 2

Review the attached development application and create a report detailing the following information:

- Document submissions required prior to commencement of construction

- Meetings required prior to commencement of construction

- Inspections and approvals required during construction

Practical Activity 3

You have been asked by the client for the highest home you are building to complete a front fence. The client has asked for a pointed brick fence 43m L, 1.5m H with engaged piers every 2m.

To complete the fence you will also be required to construct a 600mm x 400mm footing with starter bars set at each engaged pier for the entire 43m length of the fence (All dimensions advised by consulting structural engineer).

You have communicated the above to the bricklaying and concreting contractors you have engaged for the construction project.

The bricklaying contractor has proved a quote to you at $4,800 + GST for the face brick with an additional $150 per engaged pier resulting in a total price of $7, 350 + GST.

Your Concretor has quoted a price of $4300 + GST for 10.3m2 of concrete +\- 5%, reinforcement steel (hurdles, stock and hook bar with 1200mm starters for all 17 piers as well as excavation of the trench for the footing and tie wire.

All prices are inclusive of labour but do not include your builder’s margin of 15%

Use the information required to issue a contract variation to your client ‘Mr & Mrs Bird’

Resources:

DA package example

‘