# Documentation of design and construction (suitability or evidence of suitability)

| # Project Name: Proposed Residential Dwelling |  |  |  |
| --- | --- | --- | --- |
| # Project address: Pallara, Queensland |  | # Site Plans attached: HO 1 and 2 |  |
| Prepared by: Abbie H |  |  |  |
| # Date: 12/08/2025 |  |  |  |
| # Description of project: # Construction of a single-storey Class 1a residential dwelling with a waffle pod slab, timber-framed walls, prefabricated roof trusses, brick veneer, and weatherboard cladding, located in Climate Zone 2. |  |  |  |
| # Site Classification/s, Class and Climate Zone Compliance (NCC bushfire, high wind, earthquake and alpine environment requirements.) # Site Classification: Class P (reactive soils with abnormal conditions). # Building Classification: Class 1a (single residential dwelling). # Climate Zone: Climate Zone 2 (warm humid summers, mild winters). # Bushfire: Not applicable (no bushfire-prone area). # High Wind: Wind classification N2, designed for non-cyclonic conditions. # Earthquake: Low seismic activity, compliant with NCC. # Alpine: Not applicable. |  |  |  |
| Documentation of design and construction (suitability or evidence of suitability) |  |  |  |
| Site preparation |  |  |  |
| Site preparation | NCC / Australian Standards requirements |  | Compliant Performance solution |
| Earthworks | AS 2870-2011 |  | Cut and fill to RL 15.43 AHD with a maximum of 300 mm fill compacted in 150 mm layers. |
| Site preparation |  |  |  |
| Site preparation | NCC / Australian Standards requirements |  | Compliant Performance solution |
| Earth retaining structures |  |  |  |
| Drainage | NCC requirements for site drainage. |  | Grading site to prevent water ponding and installing A.G. drains if required. |
| Site preparation | NCC / Australian Standards requirements |  | Compliant Performance solution |
| Termite risk management | AS 3660.1-2000. |  | Termite barriers |
| Materials |  |  |  |
| Footings and slabs |  |  |  |
| Site classification - Class P (reactive soils). |  |  |  |
| Footings and slabs | NCC / Australian Standards requirements |  | Compliant Performance solution |
| Footings and slabs |  |  | Waffle pod slab (310 mm depth, 225 mm void, 85 mm concrete topping). |
| Footings and slabs | NCC / Australian Standards requirements |  | Compliant Performance solution |
| Moisture Barrier |  |  | 0.2 mm polyethylene membrane, taped and lapped 200 mm. |
| Materials |  |  |  |
| Concrete and reinforcement |  |  |  |
| Concrete and reinforcement | NCC / Australian Standards requirements |  | Compliant Performance solution |
| Concrete and reinforcement | Reinforcement: (per AS 3600-2018). |  | Concrete: 20 MPa strength at 28 days. Reinforcement: SL82 mesh and N12 bars |
| Materials |  |  |  |
| Masonry |  |  |  |
| Masonry | NCC / Australian Standards requirements |  | Compliant Performance solution |
| Masonry | AS 4773.1-2015 |  | Brick veneer walls with 15 MPa compressive strength. |
| Moisture Barrier |  |  | Flashing installed with weep holes at 1200 mm spacing. |
| Materials |  |  |  |
| Framing/Structure |  |  |  |
| Framing/Structure | NCC / Australian Standards requirements |  | Compliant Performance solution |
| Framing | AS 1684-2010 |  | Timber framing compliant with AS 1684-2010 for residential construction. Prefabricated roof trusses designed to handle N2 wind loads. |
| Materials |  |  |  |
| Roof and Wall Cladding |  |  |  |
| Roof and wall cladding | NCC / Australian Standards requirements |  | Compliant Performance solution |
| Roof and wall cladding |  |  | Roof Cladding: Concrete roof tiles installed at 22.5° pitch. Wall Cladding: Combination of brick veneer and James Hardie Newport weatherboards. Sarking: Vapour-permeable membrane installed behind cladding. |
| Materials |  |  |  |
| Glazing |  |  |  |
| Glazing | NCC / Australian Standards requirements |  | Compliant Performance solution |
| Glazing | Windows and sliding doors installed per AS 2047 and AS 1288. |  |  |
| Materials |  |  |  |
| Fire Safety |  |  |  |
| Fire Safety | NCC / Australian Standards requirements |  | Compliant Performance solution |
| Fire Safety | Not applicable (non-bushfire-prone area). |  |  |
| Access/Stairs/Lifts/Ramps |  |  |  |
| Access/Stairs/Lifts/Ramps | NCC / Australian Standards requirements |  | Compliant Performance solution |
| Access | Access complies with NCC requirements for residential dwellings. |  |  |
| Materials |  |  |  |
| Ancillary Provisions |  |  |  |
| Ancillary Provisions | NCC / Australian Standards requirements |  | Compliant Performance solution |
| Energy Efficiency |  |  |  |
| Energy Efficiency | NCC / Australian Standards requirements |  | Compliant Performance solution |
|  | NCC energy efficiency requirements for Climate Zone 2. |  | R2.5 insulation batts in the ceiling and R1.5 batts in external walls. |
| Health and Safety |  |  |  |

| Issues to be addressed and consultation |  |  |  |  |
| --- | --- | --- | --- | --- |
| Issue | Who to consult | Improvements for consideration | Feedback following consideration | Supporting Resources |
| Structural Integrity | Structural Engineer, Builder | Reinforce trusses for improved snow loads | Feedback integrated into roof design plans | Site Plans: Attached. |
| Termite Risk Management | Pest Control Specialist | Use physical barriers for long-term protection | Approved and implemented on-site |  |