CPCCBC4001A

Apply building codes and standards to the construction process for low rise building projects

THEORY EXAM INSTRUCTIONS - This exam will occur under standard assessment conditions. - An oral examination may be allowed under certain circumstances. Answers will be documented and FT003 (Oral examination declaration form) must be completed. - Participants must provide enough detail in their responses to demonstrate their knowledge and skills. - Please fill in the details below, providing your name, signature and date of assessment. - Your signature acknowledges that this is your own work. - If a participant initially answers a question incorrectly but modifies their response, they must follow the instructions below: - If a participant changes their response in writing, they must put their initials next to the written change. - If a participant changes their response verbally, this must be noted on the exam paper by the assessor.

| Participant name | Jamie McCarthy |
| --- | --- |
| Participant signature |  |

| Assessor name |  |
| --- | --- |
| Assessor signature |  |

| Date | 17/07/2018 |
| --- | --- |

| Number of questions | Number of questions answered correctly | Satisfactory? |  |
| --- | --- | --- | --- |
| 35 |  | Yes | No |

Assessor Comments

The diagram below represents an external house wall, shows part of roof and eave and a window in the wall and the vertical boundary line fence is shown a prescribed measurement of 900mm from the house wall to allow light from the sun to reach the window.

- What type of regulation could allow a lesser distance to be used if appropriate justification was provided?

In what document/s might you find the appropriate information?

| Answer | Performance based regulation |  |
| --- | --- | --- |

In regard to the above 900mm distance from house wall to boundary fence and in using or referencing performance-based regulations in the BCA what else should you be aware of and consider?

| Answer | - Fire Separation and Fire Resistance level - Boundaries as local government may be subject to planning laws |  |
| --- | --- | --- |

Who has responsibility for producing and maintaining The Building Code of Australia (BCA) and who is it done on behalf of?

| Answer | ABCB – Standards Australia consults state governments to maintain the same standards throughout Australia |  |
| --- | --- | --- |

Fill in the missing words to complete the following two sentences.

provisions

Technical

design

construction

document

Performance - based

The BCA is a uniform set of ___________ ______________ for the __________ and ______________ of buildings and other structures throughout Australia.

Legislative

requirements

It is a ______________________ ________________ that is referenced so that your building system or material can demonstrate that it meets certain performance requirements in order to meet __________________ ____________________.

| Answer |  |  |
| --- | --- | --- |

What is the principal goal of the Building Code of Australia (BCA)?

| Answer | To achieve nation wide consistency. The minimum necessary standards of safety, health, amenity and sustainability objectives. |  |
| --- | --- | --- |

How many volumes are there in the BCA and to what do the volumes refer?

| Answer | Volume 1: - Class 2-9 - Access requirements for people with disability in class 1b & 1a - Certain 10b structures including access requirements for people with disability in class 10 Volume 2: - Class 1 & 10a - Class 10c - Requirements for residential and non habitable buildings and structures Volume 3: - Plumbing and drainage for all classes of buildings |  |
| --- | --- | --- |

Explain the BCA performance requirements for low rise projects from the diagram below.

| Answer | Objectives - To regulate and safeguard people Functional Statements - Explains how a building is to meet the community expectations Performance requirements - These outline a suitable level of performance for building materials, components, design factors and the construction methods, in order for a building to meet the relevant functional statements and in turn the relevant objectives Building solutions - Two methods that set out the means of achieving compliance with the performance requirements (Deemed to satisfy: BCA acceptable solutions & Alternative solutions: requirements that still achieve the outcome but a different way). |  |
| --- | --- | --- |

What is the intent of the ‘Relevant Performance Requirements’ in the BCA?

| Answer | A method of determining the appropriate performance requirements to be used when an Alternate Solution is used, to provide that a building solution complies with the BCA. |  |
| --- | --- | --- |

Building solutions set out the means of achieving compliance with the performance requirements. The BCA provides for two methods that can be followed to develop a building solution. Explain these two methods.

| Answer | Deemed to Satisfy: Examples of materials, components, design factors and construction methods. If used it will result in compliance with the performance requirements of the BCA. Alternative solutions: There is no obligation to adopt any particular material, component, design factor or construction method. An approval authority may still issue an approval if a building proposal differs in whole or in part from the deemed to satisfy provisions, and if it can be demonstrated that the design complies with relevant performance requirements. |  |
| --- | --- | --- |

The deemed-to-satisfy building solution establishes a clear and concise process for compliance for a builder, building surveyor or the like (building certifier) who does not want to develop a new means of achieving performance requirements; what does ‘deemed-to-satisfy’ mean?

| Answer | It essentially means the building solution complies in an equivalent or superior way to a deemed to satisfy provision. |  |
| --- | --- | --- |

Write an example of a ‘deemed-to-satisfy’ provision for a class 1 building including deemed-to-satisfy requirements for kitchen, bathroom, laundry and sanitary facilities.

Nominate where you found the information in the BCA.

| Answer | Kitchen 3.8.3.2 DTS: A Class 1 building must be provided with: - A kitchen sink and facilities for the preparation and cooking of food; and - A bath or shower; and - Clothes washing facilities, comprising at least one washtub and space in the same room for a washing machine Requirement 2.4.3: A food preparation facility must be provided which includes: - A means for food rinsing, utensil washing and the sanitary disposal of associated waste water; and - A means for cooking; and - A space for food preparation Laundry & Sanitary 3.8.3.3 DTS: The door to a fully enclosed sanitary compartment must: - Open outwards; or - Slide; or - Be readily removable from the outside of the compartment, unless there is a clear space of at least 1.2m measured in accordance, between the closet pan within the sanitary compartment and the doorway. Requirement 2.4.3 - Suitable sanitary facilities for personal hygiene must be provided in a convenient location within or associated with a building appropriate to its function or use - Laundering facilities or space for laundering facilities and the means for sanitary disposal of waste water must be provided in a convenient location within, or associated with a building, appropriate to its function or use. Bathroom 3.8.1.1 DTS: Building elements in wet areas within a building must: - Be waterproof or water resistant in accordance with table 3.8.1.1; and - Comply with AS3740 Requirement 2.4.1 To protect the structure of the building and to maintain the amenity of the occupants, water must be prevented from penetrating: - Behind fittings and linings; or - Into concealed spaces Of sanitary facilities, bathrooms, laundries and the like. |  |
| --- | --- | --- |

Who and what is Standards Australia?

| Answer | Australia’s peak non-government standards body. Recognised through memo of understanding with the Australian government. Standards Australia is an independent non government not for profit organisation. It developed internationally aligned Australian Standards and related publications to help assure the safety, reliability and performance of a range of products, services and systems. |  |
| --- | --- | --- |

What are Australian Standards?

| Answer | Standards are published documents setting out specifications and procedures designed to ensure products, services and systems are safe, reliable and consistently perform the way they were intended to. They establish a common language which defines quality and safety criteria. |  |
| --- | --- | --- |

The classification of a building or part of a building is determined by the purpose for which it is designed, constructed or adapted to be used. Please summarise the 10 building classifications below.

| Answer | Class | Summary |  |
| --- | --- | --- | --- |
|  | 1 | One or more building which constitute a class 1a – single dwelling, class 1b – boarding house, hostel |  |
|  | 2 | Contains two or more sole occupancy units, each being a separate dwelling |  |
|  | 3 | A residential building other than class 1 or 2. Which is a common place of long term or transient living for a number of unrelated persons. Eg. Boarding house, school |  |
|  | 4 | A dwelling in a building that is class 5,6,7,8 or 9. It is the dwelling in the building. |  |
|  | 5 | An office building used for professional or commercial purposes, excluding classes 6,7,8 or 9. |  |
|  | 6 | A shop or building used for selling of goods by retails or supply of services to the direct public. |  |
|  | 7 | A building which is class 7a – car park or 7b – storage or display of goods or products for sale |  |
|  | 8 | A laboratory or building for the production, assembly, altering, repairing, packing or cleaning of goods for sale |  |
|  | 9 | A building of public nature. 9a: health care building eg. Day-care surgeries 9b: an assembly building eg. Theatre, nightclub 9c: an aged care building |  |
|  | 10 | A non-habitable building or structure. 10a: non-habitable building eg. Private garage 10b: fence, mast, pool 10c: shelter eg. Private bushfire shelter |  |

Other than the 10 BCA building classifications (A3.2), there are also multiple classifications (A3.3) where each part of a building is classified separately. 

Where would you find a full list of Past A3 classification of Buildings and Structures?

| Answer | Standard Building By-Laws 1975 |  |
| --- | --- | --- |

True or False (Tick your response): If a building or part of a building has more than one classification applying to the whole building or part in accordance with, that building or part must comply with all the relevant provisions of the BCA for each classification.

| Answer | ☒ | True |  |  |
| --- | --- | --- | --- | --- |
|  | ☐ | False |  |  |

Familiarity with plans and specifications for a project when interpreting a building solution and therefore trying to interpret clauses and other referenced documents in the BCA is important.

What do plans and specifications include?

| Answer | Floor level plans Elevation plans Sectional Drawings All Specifications |  |
| --- | --- | --- |

You must have a clear picture of the building to be constructed and be able to identify the criteria that you will need to compare to the Deemed-To-Satisfy (DTS). What is it you must also consider when constructing complex buildings?

| Answer | Some requirements may have impacts on other requirements under the BCA. |  |
| --- | --- | --- |

Complete the missing words from the sentence below and explain the difference between these points.

mandatory

recipes

BCA Performance Requirements are _________________ and Deemed-to-Satisfy provisions are ______________.

| Answer | Performance requirement: is the level of performance which a building solution must meet. Deemed to satisfy: Establishes a clear and concise process for compliance. |  |
| --- | --- | --- |

What is a Building Solution?

| Answer | Sets out means for achieving compliance with performance requirements. Two methods being Deemed to Satisfy provisions and Alternative Solutions. |  |
| --- | --- | --- |

Referring to the BCA, what are the objectives of Section C Fire Resistance?

| Answer | Safeguard people from illness or injury due to a fire in a building Safeguard occupants from illness or injury while evacuating a building during a fire. Facilitate the activities of emergency services personnel Avoid the spread of fire between buildings Protect other property from physical damage caused by structural failure of a building as a result of a fire. |  |
| --- | --- | --- |

Why is it necessary to identify and document performance-based solutions?

| Answer | Comply with organisational quality procedures and processes Apply and interpret relevant documentation and codes Accurately apply national and state and territory OHS requirements relating to construction workplace Identify faults and problems impacting on OHS and proposing appropriate action to rectify. |  |
| --- | --- | --- |

In the context of the Building Code of Australia what are Assessment Methods used to determine?

| Answer | They are used to determine if a building solution complied with relevant BCA performance requirements. |  |
| --- | --- | --- |

Explain the importance of maintaining detailed documentation for a building project.

| Answer | To ensure the building is compliant and can pass assessment. The ability to provide a reference tool required for inspections of key building elements, ensures they are not inadvertently overlooked. |  |
| --- | --- | --- |

With regard to detailed documentation what are some of the documents that must be provided and maintained by contractors?

| Answer | In plans all calculations of building areas, site areas, site coverage, floor area, building height, habitable rooms, windows and secluded private open space on adjoining allotments in order to enable the authority to determine compliance with regulations Plans and Drawings: maintain all necessary notes, specifications and analysis to enable the authority to determine compliance with Acts, Regulations and BCA. May include construction of sanitary compartments, smoke alarms or glazing calculation notes. Contain full details of any alternative solutions proposed – disclose full procedures and documentation for further details. Contain all submissions necessary in support of the exercise of discretionary powers under either national, state or territory regulations. |  |
| --- | --- | --- |

Other than the Builder maintaining detailed documentation, who will also have responsibility to resolve and correct errors for any ambiguity identified in the plans?

| Answer | Engineers Surveyor Certifier Home Owners Architects |  |
| --- | --- | --- |

The threat of an accidental fire in a building and to prevent it are important to understanding fire and taking all steps to prevent or confine fire and to prevent fire spreading within the structure as well as to adjoining or adjacent buildings. 

In this regard Fire Protection Systems are paramount and are subject to?

| Answer | Building Regulations Fire Regulation Insurance company requirements Local government requirements other regulatory authorities |  |
| --- | --- | --- |

Briefly explain the following fire definitions:

| Answer | Term | Explanation |  |
| --- | --- | --- | --- |
|  | Fire | Rapid oxidation of a gas liquid or solid substance accompanied by release of heat and light |  |
|  | Passive Fire Protection | A component of three components of structural fire protection and fire safety in a building.A component of three components of structural fire protection and fire safety in a building |  |
|  | Active Fire Protection | Characterised by items and systems which require a certain amount of motion and response in order to work. (fire sprinkler)Characterised by items and systems which require a certain amount of motion and response in order to work (fire sprinkler) |  |

The purpose of fire protection is to:

- 1. Prevent/reduce the possibility of fire and 
2. Reduce the spread of fire within and around a building

Name the two categories or systems that provide measurement and arrangements for fire protection.

| Answer | Active Systems Passive Systems |  |
| --- | --- | --- |

With respect to the general level of fire protection explain separately the terms ‘Fire Load’ and ‘Fire Load Density’?

| Answer | Fire Load -amount of potential heat that can be liberated during the fire - expressed as a numerical value of kj/kg Fire density -fire load of a given enclosure or building divided by floor area - expressed as fire load per unit area of floor space. (bigger the fire load the more severe the fire) |  |
| --- | --- | --- |

Using Volume 2 of the BCA respond to the following:

- Where in the BCA would you look to find requirements for building in bushfire prone areas for Class 1 and 10 buildings?

- Where in the BCA would you look to find requirements for building in bushfire prone areas for Class 2-9 buildings?

- What Australian Standard must be complied with to satisfy the Deemed to Satisfy Provisions for construction of Class 2 and 3 buildings in bushfire prone areas?

| Answer | a) | Part 3.7.4 Bushfire areas |  |
| --- | --- | --- | --- |
|  | b) | Part 3.7.4 & AS3959 |  |
|  | c) | AS3959 |  |

What is a Fire Safety Schedule and its relationship to the approval of development of a class 2 – 9 building?

| Answer | It lists the essential fire safety measures that should be installed in the building and performance standard which each of these measures are most capable of operating. |  |
| --- | --- | --- |

What are building certifiers responsible for?

| Answer | They are responsible for ensuring specific compliance is achieved. Mostly conducted by visual inspection. |  |
| --- | --- | --- |

What are building certifiers not responsible for?

| Answer | They are not responsible for the quality of workmanship within the building process. |  |
| --- | --- | --- |

Where in the BCA would you determine the level of fire resistance required for the construction of buildings?

| Answer | Volume 1 C1.1 |  |
| --- | --- | --- |