CPCCBC4007A

Plan building or construction work

ASSESSMENT SUMMARY

The assessment activities for CPCCBC4007A Plan building or construction work cover all aspects of the unit of competency required. To demonstrate competence in this unit, a participant must undertake all tasks and complete them satisfactorily and consistently. For Apprentices and Trainees, this includes employer completion of the Employer Record Book (ERB).

Participants must read all the information given before starting any assessment task. The assessment tasks are intended to be equitable, fair and flexible. If a participant has any questions or requires alternative arrangements to be made such requests should be directed to their Trainer/Assessor, who should note such arrangements in the ‘Assessor Comments’ on the affected assessment. If a participant feels they are not yet ready to be assessed, they should discuss their options with their Trainer/Assessor.

To satisfactorily complete the assessment, the participant is required to answer all questions correctly and demonstrate their skills to industry standards. If a participant does not answer some questions or perform some tasks, they will be deemed ‘Not Satisfactory’ for the task and ‘Not Yet Competent’ for the unit of competency. The Trainer/Assessor is able to provide the participant with additional information on interpreting the assessment outcomes and provide guidance on the participant’s options. Should a participant still be deemed ‘Not Satisfactory’ they will have the opportunity to undertake a supplementary assessment or appeal the result.

By signing the cover page for each assessment item, the participant acknowledges they understand the assessment instructions and requirements and consent to being assessed. The participant also verifies and assures the RTO that the work submitted is their own work.

| Participant Name | Christine Muter |
| --- | --- |

| Task | Assessment Results |  |
| --- | --- | --- |
| Theory Exam | Satisfactory | Not Yet Satisfactory |
| Practical Assessment | Satisfactory | Not Yet Satisfactory |
| Outcome for Unit of Competency | Competent | Not Yet Competent |

| Assessor Name |  |
| --- | --- |
| Assessor Signature |  |
| Unit Competence Determination Date | Click here to enter a date. |

CPCCBC4007A

Plan building or construction work

THEORY EXAM INSTRUCTIONS - This exam will occur under standard assessment conditions. - An oral examination may be allowed under certain circumstances, whereby the Participant’s answers will be documented by a party other than the Participant. This must be acknowledged in writing (e.g. FT001 and Assessor Comments) - Participants must provide enough detail in their responses to demonstrate their knowledge and skills. - Please fill in the details below, providing your name, signature and date of assessment. - Your signature acknowledges that this is your own work. - Assessors are to complete marking in pen of a different colour to the Participant. - If a Participant initially answers a question incorrectly but modifies their response, they must follow the instructions below: - If a Participant changes their response in writing, they must put their initials next to the written change. - If a Participant changes their response verbally, this must be noted on the exam paper by the assessor.

| Participant name | Christine Muter |
| --- | --- |
| Participant signature |  |
| Date | Click here to enter a date. |

| Assessor name |  |
| --- | --- |
| Assessor signature |  |
| Date | Click here to enter a date. |

| Number of questions | Number of questions answered correctly | Task outcome (Tick) |  |
| --- | --- | --- | --- |
| 45 |  | Satisfactory | Not Yet Satisfactory |

ASSESSOR Comments

- Name one (1) document you might read alongside the contract to identify the following aspects:

- Material requirements

- Plant requirements

- On-site labour requirements

- Any unusual aspects of construction

| Answer | Project Plan Specifications |  |
| --- | --- | --- |

- Name the Act and identify the section that outlines the maximum deposit amounts that may be accepted by a builder before services are provided. Identify the maximum deposit amounts and the penalties for breaching such requirements.

| Answer | Section of Act | Requirements and penalties |  |
| --- | --- | --- | --- |
|  | Part 4, Division 2, SS 33 | (1) The building contractor under a regulated contract must not, before starting to provide the contracted services at the building site, demand or receive a deposit under the contract of more than— (a) for a level 1 regulated contract (other than a contract mentioned in paragraph (c))—10% of the contract price; or (b) for a level 2 regulated contract (other than a contract mentioned in paragraph (c))—5% of the contract price; or (c) for a level 1 or 2 regulated contract under which the value of the off-site work is more than 50% of the contract price—20% of the contract price. Maximum penalty—100 penalty units. |  |

- Identify three (3) subcontractors you may engage for a job and the type of work they may undertake.

| Answer | Subcontractor | Work |  |
| --- | --- | --- | --- |
|  | Gas Fitter | Installs gas pipes to relevant appliances eg, Hot water systems and gas cook tops |  |
|  | Plasterer | Installs the internal wall linings eg, walls and ceilings |  |
|  | Plumbers | Provides water supply, roof guttering, internal and external pipes and drains, taps, toilets hot water systems, and ensures the sewer is connected correctly |  |

- Which of the following methods may be used to find subcontractors for a project? Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Advertising |  |
|  |  | Networking |  |
|  |  | Online searches |  |
|  |  | Tenders |  |
|  |  | Word of mouth |  |

- Explain the importance of determining subcontractor or plant availability for a job.

| Answer | Once the required materials and plant hire is identified, it is important to ensure the trades and required machinery is available to carry out the work in the time frame that is required for the project |  |
| --- | --- | --- |

- Identify two (2) methods you might use to find a supplier for materials.

| Answer | Liaise with known suppliers that are reliable and have a wide rage of resources Phone around to see who may have the supplies required for the project. |  |
| --- | --- | --- |

- Name two (2) factors that may impact on the availability of materials.

| Answer | Delays caused by imported or interstate materials An unreliable supplier |  |
| --- | --- | --- |

- Australian Standards must be considered when selecting materials. Identify the Australian Standard (referenced in the National Construction Code) which relates to the requirements for interior lighting.

| Answer | AS/NZS 1680.0 |  |
| --- | --- | --- |

- Identify two (2) types of plant that may be used for a building project and identify the type of work they may be used for.

| Answer | Plant | Work |  |
| --- | --- | --- | --- |
|  | Excavator | Prepare the land for the project site |  |
|  | Fork truck | Moving heavy pallets of materials around site, eg bricks |  |

- Identify two (2) types of equipment that may be used for a building project and identify the type of work they may be used for.

| Answer | Equipment | Work |  |
| --- | --- | --- | --- |
|  | String line | Identify the boundary of the site |  |
|  | Wheelbarrow | Transporting lighter loads of materials or mixing smaller amounts or wet materials such as mortar for bricks |  |

- Identify two (2) of the records you should keep when hiring plant and/or equipment.

| Answer | Written confirmation that the equipments meets the relevant standards in relation to construction and are compliant. Inspect the condition of the equipment when it arrives recording its condition |  |
| --- | --- | --- |

- Name five (5) temporary services that need to be organised for a building site.

| Answer | Electricity supply Water supply Temporary fencing to prevent public access Waste bins Portable toilet |  |
| --- | --- | --- |

- True or False (Tick your response): Site accommodation should be provided where construction work occurs in a remote area.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

- What is required of a builder before any person is allowed on a building site? Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | To check the person has a Construction Induction Card |  |
|  |  | To check the person has the approval of the local council |  |
|  |  | To check the person is wearing appropriate signage |  |
|  |  | To keep a record of all inductions and all personnel on the site |  |

- Who issues a construction induction card? Tick your response.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Australian Parliament upon fee payment |  |
|  |  | Each site at the conclusion of a site induction |  |
|  |  | Registered Training Organisations at the conclusion of training |  |

- Identify one (1) action you might take to facilitate site access for machinery.

| Answer | If a crane is required on site, traffic control may be required |  |
| --- | --- | --- |

- Identify the Volume and Sections of the NCC that relate to access to buildings for Classes 1 and 10, and Classes 2 to 9.

| Answer | Classes 1 and 10 | Classes 2 to 9 |  |
| --- | --- | --- | --- |
|  | Volume 1 Section D | Volume 2 Part 2.5 |  |

- You should contact your local council to be approved for out of hours construction. Identify the required documentation for an application and the timeframe for approval.

| Answer | - Along with the application from the Gold Coast City Council, you must identify: - - Times and dates for the intended work - - Number of staff undertaking the work - - Contact details of the person in charge of the work - - Plant/equipment/machinery and noise output (dBs) of the equipment, etc. - - Pay the relevant application fee. |  |
| --- | --- | --- |

- In addition to the permit from Q18. identify two (2) other permits you may need to obtain to undertake a building project. 

Briefly describe the documents required for approval and who these should be submitted to for approval.

| Answer | Permit | Documentation and approval body |  |
| --- | --- | --- | --- |
|  | Building approval l | DA Form 2 – Building work details to make a development application involving building work.- Local Council |  |
|  | Design Certificates | Form 15 - Compliance certificate for building design or specification.- Local Council |  |

- True or False (Tick your response): Project commencement date is always at the date the contract is signed.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

- What document should be developed when purchasing supplies or construction materials?

| Answer | A Delivery sheet |  |
| --- | --- | --- |

- True or False (Tick your response): Every delivery should be physically checked against the delivery sheet.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

- Who should be contacted if items are missing from a delivery?

| Answer | The supplier |  |
| --- | --- | --- |

- Name the Act and identify the section that outlines the requirement to implement hazard and risk management on site. Briefly describe the requirements.

| Answer | Section of Act | Requirements |  |
| --- | --- | --- | --- |
|  | The Work Health and Safety Act 2011 (Qld). Sections 17-18 | This section of the act outlines that risks need to be identified and eliminated as is as reasonably practical or minimised as so far as reasonably practical . |  |

- Identify one (1) regulation that an organisation’s WHS policies and procedures must comply with.

| Answer | Work Health and Safety Regulation 2011 (QLD) |  |
| --- | --- | --- |

- What document should be followed when disposing of hazardous materials?

| Answer | The Safety Data Sheet |  |
| --- | --- | --- |

- An asbestos removal control plan is required when more than ____m2 of asbestos containing material is to be removed.

| Answer | 10 m2 |  |
| --- | --- | --- |

- True or False (Tick your response): A site assessment should be undertaken if contamination is known or likely to be present from the removal of existing services.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

- True or False (Tick your response): Multiple projects can be planned using a single Gantt Chart.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

- What are the advantages of hiring subcontractors to work across multiple nearby projects? Explain with an example.

| Answer | By having trades work across multiple projects, it not only saves time and therefore money, it provides the assurance the trades will remain with you and not leave to attend another site/project, if this occurs it may be difficult to ensure they return in time for the next phase of the project. |  |
| --- | --- | --- |

- Provide one (1) example to explain why construction operations must be sequenced.

| Answer | Without planning the sequence of the construction works cannot be completed in accordance to the required elements of the buildings construction. For example, the waterproofing must be completed before the tiles in the wet areas are laid and this cannot be done until the internal walls are in place. |  |
| --- | --- | --- |

- Which of the following should be considered as part of task sequencing? Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Holidays |  |
|  |  | Labour availability |  |
|  |  | Plant availability |  |

- Order the following construction operations from 1 to 6 to form the most efficient sequence.

| Answer | Step Number | Steps |  |
| --- | --- | --- | --- |
|  | 2 | Excavation of footings |  |
|  | 3 | Floor frame |  |
|  | 5 | Installing brick base |  |
|  | 6 | Roof trusses |  |
|  | 1 | Set out |  |
|  | 4 | Wall frames |  |

- Refer to the table below to complete the chart shown. Label the blank boxes with the appropriate Task ID letters.

| Task ID | Task | Preceding event | Nodes |
| --- | --- | --- | --- |
| A | Make enquiries and order all materials required | Start | 1 |
| B | Wait for bricks, sand and cement to arrive | A | 2 |
| C | Clear vegetation from the site | A | 2 |
| D | Order concrete to be delivered | A | 2 |
| E | Dig the trench | C | 4 |
| F | Pour the footings | E | 5 |
| G | Footings curing time | F | 6 |
| H | Load the bricks on the site | B | 3 |
| I | Mix the mortar and lay the bricks | H | 7 |
| J | Clean rubbish from the area | I | 8 |
|  | Finish | J | 9 |

H

G

A

E

I

J

D

C

B

A

| Answer |  |  |
| --- | --- | --- |

- Name one (1) computer-based software package that may be used to prepare a project schedule.

| Answer | PERT |  |
| --- | --- | --- |

Refer to the chart below to respond to Q36. to Q40. This chart shows the task sequence for building a brick fence.

- What is the total time for the process if the top path is taken? 
Tick your response.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | 8 days |  |
|  |  | 9 days |  |
|  |  | 11 days |  |

- What is the total time for the process if the path through nodes 2, 4 and 5 is taken? Tick your response.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | 8 days |  |
|  |  | 9 days |  |
|  |  | 11 days |  |

- What is the total time for the process if the path through nodes 2, 5 and 6 is taken? Tick your response.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | 8 days |  |
|  |  | 9 days |  |
|  |  | 11 days |  |

- Describe the critical path.

| Answer | The critical path identifies the longest time period that it will take to compete the project. This determines the earliest possible completion date for the project |  |
| --- | --- | --- |

- True or False (Tick your response): Even if the brick delivery in the top path was late by two (2) days, it is possible that the job will still run on schedule.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

- Identify three (3) factors that may impact on the critical path of a project and cause it to need revision.

| Answer | Delays on deliveries Inclement weather Unforseen issues at the time of site preparation |  |
| --- | --- | --- |

- Which major point(s) should you note when carrying out an inspection of neighbouring properties? Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | If the neighbours drive the right kind of car for that neighbourhood |  |
|  |  | Structure and condition of any boundary fences |  |
|  |  | Type and condition of all structures on the boundary |  |
|  |  | Whether the houses are of Victorian heritage |  |

- Why should you take photographs of site boundaries with neighbouring properties?

| Answer | Images support the condition report of the current condition of any neighbouring property that may be affected by building works. |  |
| --- | --- | --- |

- When should a condition report be given to owners of adjacent buildings? Tick your response.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Before construction commences |  |
|  |  | During construction |  |
|  |  | After construction completes |  |

- When might a neighbouring property owner be required to refer to a condition report?

| Answer | The report clearly identifies if any damages were caused by the construction work or if it was there before the construction commenced. They would refer to this if they have a dispute with the builder if they believe damages were caused by the construction |  |
| --- | --- | --- |

CPCCBC4007A

Plan building or construction work

PRACTICAL ASSESSMENT INSTRUCTIONS - The following practical assessment will require the Participant to complete work related activities. - The Assessor will conduct observations of the Participant’s work to determine whether they display sufficient practical skills and knowledge. - Please fill in the details below – providing your name, signature and date of assessment. - The Participant’s signature acknowledges that the Assessor provided clear instructions and the Participant understood what was required to complete this assessment. - The Assessor may substitute a Practical Task with an alternate task of equal value. - Alternate tasks must be fully documented by the assessor in the Assessor Comments area.

| Participant name | Christine Muter |
| --- | --- |
| Participant signature |  |
| Date | 20/01/2022 |

| Assessor name |  |
| --- | --- |
| Assessor signature |  |
| Date | Click here to enter a date. |

| Task | Task outcome (Tick) |  |
| --- | --- | --- |
|  | Satisfactory | Not Yet Satisfactory |
| Task 1 (Occasion 1) |  |  |
| Task 1 (Occasion 2) |  |  |

Assessor Comments

CPCCBC4007A

Plan building or construction work

| Practical Task 1 (2X) |  |  |  |
| --- | --- | --- | --- |
| Description of task: On two (2) occasions, a suitable observer is to verify the participant’s ability to plan on-site activities, including the employment of physical and human resources and the development of documentation and advice for relevant authorities: - Occasion 1: Residential projects - Occasion 2: Commercial projects The participant is to gather evidence of how they have completed each component of the Practical Task, which is to be submitted with this Practical Assessment sheet. Evidence to be submitted may include: - A construction program/schedule for a project showing: - The sequencing of the trades, etc. - Working and non-working days, etc. - Local authority construction commencement notices - Pre-construction site inspection reports or dilapidation reports - WHS policy The observer is to provide detailed comments on the tasks completed by the participant. The assessor is to verify the observer’s comments in the Assessor Comments section, and provide the Practical Assessment outcome on the front cover of this document. |  |  |  |
| Resources required: Computer (ideally with project scheduling software), Building Contract, Organisational policies, Telephone, Facsimile. |  |  |  |
| Assessment of task |  |  |  |
| Name of participant: | Christine Muter |  |  |
| Name of observer: | John Gammon |  |  |
| Role of observer (Confirm suitability to sign) | Construction Manager |  |  |
| Email address of observer | John.gammon@tbs.com.au |  |  |
| Signature of observer: |  |  |  |
| Date: | 20/01/2022 |  |  |
| When completing the task, did the participant: |  | Tick if satisfactorily completed. Provide comments if left unticked. |  |
|  |  | 1st Occasion | 2nd Occasion |
| 1 Appraise contract documentation |  |  |  |
| - Review contract documentation, codes, standards and regulations to identify any unusual aspects of construction, use of materials or penalties. |  |  | Commercial projects are not part of TBS business model |
| - Determine availability of selected subcontractors to suit the job requirements. |  | This part of the project planning is carried out by the job supervisor | Commercial projects are not part of TBS business model |
| - Assess availability of materials and confirm with suppliers. |  | This part of the project planning is carried out by the job supervisor | Commercial projects are not part of TBS business model |
| - Identify site access requirements and limitations, taking action to facilitate entry. |  | This part of the project planning is carried out by the job supervisor | Commercial projects are not part of TBS business model |
| - Prepare documentation for authorities controlling construction work and determine project commencement date. |  |  | Commercial projects are not part of TBS business model |
| - Implement procedures for controlling and recording site deliveries. |  | This part of the project planning is carried out by the job supervisor | Commercial projects are not part of TBS business model |
| 2 Construction operation strategies |  |  |  |
| - Identify organisational strategies for implementing construction. |  |  | Commercial projects are not part of TBS business model |
| - Implement procedures for recording the hire of plant and equipment. |  | This part of the project planning is carried out by the job supervisor | Commercial projects are not part of TBS business model |
| - Implement organisational WHS policies and procedures, including hazard and risk management. |  |  | Commercial projects are not part of TBS business model |
| - Implement procedures for the removal of existing services and hazardous materials, in accordance with Environmental Protection Agency requirements. |  | This part of the project planning is carried out by the job supervisor | Commercial projects are not part of TBS business model |
| - Follow procedures for the control of multiple projects. |  |  | Commercial projects are not part of TBS business model |
| 3 Project schedule |  |  |  |
| - Sequence construction operations. |  |  | Commercial projects are not part of TBS business model |
| - Enter operation details into a manually prepared project schedule or computer-based software package. |  |  | Commercial projects are not part of TBS business model |
| - Define critical path of the project, revising as required. |  |  | Commercial projects are not part of TBS business model |
| - Adjust project timeframes to account for anticipated delays. |  |  | Commercial projects are not part of TBS business model |
| 4 Required resources |  |  |  |
| - Determine and document temporary services and site accommodation requirements. |  |  | Commercial projects are not part of TBS business model |
| - Determine and document plant requirements and availability, with reference to contract documentation. |  |  | Commercial projects are not part of TBS business model |
| - Determine and document on-site labour requirements, with reference to contract documentation. |  | This part of the project planning is carried out by the job supervisor | Commercial projects are not part of TBS business model |
| 5 Condition reports |  |  |  |
| - Complete reports on the condition of existing buildings and structures on adjacent site boundaries. |  | This part of the project planning is carried out by the job supervisor | Commercial projects are not part of TBS business model |
| - Forward copies of condition reports to the owners of adjacent buildings prior to commencing construction. |  | if required | Commercial projects are not part of TBS business model |
| - Effectively communicate using the following methods: - Telephone, facsimile, email and in writing - Language/concepts appropriate to cultural differences - Non-verbal communication |  |  | Commercial projects are not part of TBS business model |
| The Participant’s performance was: Satisfactory Not Satisfactory |  |  |  |

observer Comments

Christine has shown a strong aptitude in the areas relevant in the above criteria that fall within her job description. One part of Christine’s duties is that she is responsible for ensuring the relevant building approval is obtained before works are due to commence on site, this includes obtaining any required BAL requirements pertaining to re roofing and then to follow through and obtain the required certification from the relevant builder once works are completed in order to obtain the Form 21. Christine’s responsibilities are to also ensure that each of the site supervisors are operating in accordance with the planning chart that is set out to ensure works are completed by the contracted Finish date, this requires weekly and sometimes daily meetings with each supervisor to confirm works are on track and if any unforeseen delays do occur then the relevant delay notice is forwarded to the client for review. Generally the site supervisor will arrange the trades at TBS and they are subcontracted to carry out the work, therefore are responsible for ordering the required amount of materials in accordance with the work order/ purchase order provided ( once the quote is reviewed and approved) and to ensure the arrive to site on the required date noted on the purchase order. On occasion Christine does arrange to send out purchase orders to trades with a breakdown of what is required to complete the work. Christine monitors multiple supervisors who each have several projects running at any given time.