CPCCBC4002A

Manage occupational health and safety in the building and construction workplace

ASSESSMENT SUMMARY

The assessment activities for CPCCBC4002A Manage occupational health and safety in the building and construction workplace cover all aspects of the unit of competency required. To demonstrate competence in this unit, a participant must undertake all tasks and complete them satisfactorily and consistently. For Apprentices and Trainees, this includes employer completion of the Employer Record Book (ERB).

Participants must read all the information given before starting any assessment task. The assessment tasks are intended to be equitable, fair and flexible. If a participant has any questions or requires alternative arrangements to be made such requests should be directed to their Trainer/Assessor, who should note such arrangements in the ‘Assessor Comments’ on the affected assessment. If a participant feels they are not yet ready to be assessed, they should discuss their options with their Trainer/Assessor.

To satisfactorily complete the assessment, the participant is required to answer all questions correctly and demonstrate their skills to industry standards. If a participant does not answer some questions or perform some tasks, they will be deemed ‘Not Satisfactory’ for the task and ‘Not Yet Competent’ for the unit of competency. The Trainer/Assessor is able to provide the participant with additional information on interpreting the assessment outcomes and provide guidance on the participant’s options. Should a participant still be deemed ‘Not Satisfactory’ they will have the opportunity to undertake a supplementary assessment or appeal the result.

By signing the cover page for each assessment item, the participant acknowledges they understand the assessment instructions and requirements and consent to being assessed. The participant also verifies and assures the RTO that the work submitted is their own work.

| Participant Name | Matthew Cumiskey |
| --- | --- |

| Task | Assessment Results |  |
| --- | --- | --- |
| Theory Exam | Satisfactory | Not Yet Satisfactory |
| Practical Assessment | Satisfactory | Not Yet Satisfactory |
| Outcome for Unit of Competency | Competent | Not Yet Competent |

| Assessor Name |  |
| --- | --- |
| Assessor Signature |  |
| Unit Competence Determination Date | Click here to enter a date. |

CPCCBC4002A

Manage occupational health and safety in the building and construction workplace

THEORY EXAM INSTRUCTIONS - This exam will occur under standard assessment conditions. - An oral examination may be allowed under certain circumstances, whereby the Participant’s answers will be documented by a party other than the Participant. This must be acknowledged in writing (e.g. FT001 and Assessor Comments) - Participants must provide enough detail in their responses to demonstrate their knowledge and skills. - Please fill in the details below, providing your name, signature and date of assessment. - Your signature acknowledges that this is your own work. - Assessors are to complete marking in pen of a different colour to the Participant. - If a Participant initially answers a question incorrectly but modifies their response, they must follow the instructions below: - If a Participant changes their response in writing, they must put their initials next to the written change. - If a Participant changes their response verbally, this must be noted on the exam paper by the assessor.

| Participant name | Matthew Cumiskey |
| --- | --- |
| Participant signature |  |
| Date | 2/12/2019 |

| Assessor name |  |
| --- | --- |
| Assessor signature |  |
| Date | Click here to enter a date. |

| Number of questions | Number of questions answered correctly | Task outcome (Tick) |  |
| --- | --- | --- | --- |
| 28 |  | Satisfactory | Not Yet Satisfactory |

Assessor Comments

For each of the common building and construction jobs listed below, identify one (1) hazard specific to that job.

| Answer | Job | Risks |  |
| --- | --- | --- | --- |
|  | Construction equipment use (e.g. power tools, concrete mixers) | A risk with using a power tool such as a circular saw is cutting yourself with the blade |  |
|  | High risk construction equipment use (e.g. lasers, EPT) | When using an EPT there is a risk of eye damage if you are not wearing eye protection. |  |
|  | Crane use | There is a risk of the load that the crane is carrying falling. |  |
|  | Demolition | There is a risk of what you are demolishing falling on you |  |
|  | Formwork | When carrying formwork materials there is a risk of back injury if you do not use proper lifting techniques |  |
|  | Scaffolding use | There is a risk of the scaffold falling over if it has not been braced/tied to the building |  |
|  | Work at heights | There is a risk of falling when working at heights |  |
|  | Welding | There is a risk of damaging your eyes if not wearing the appropriate protective gear |  |

Using the risk table below, determine the level of risk for each of the hazards identified in Q1. Identify how each risk level was determined, by identifying the likelihood and consequences of each hazard.

| Matrix | A: Death or permanent disability | B: Serious injury | C: Lost time injury | D: Casualty Treatment | E: First Aid |
| --- | --- | --- | --- | --- | --- |
| 1: Very Likely | E | E | H | H | M |
| 2: Likely | E | H | H | M | M |
| 3: Possible | H | H | M | M | L |
| 4: Unlikely | H | M | M | L | L |
| 5: Very Unlikely | M | M | L | L | L |

| E: Extreme | H: High | M: Medium | L: Low |
| --- | --- | --- | --- |

| Answer | Job | Level of risk |  |
| --- | --- | --- | --- |
|  | Construction equipment use (e.g. power tools, concrete mixers) | High risk as cutting yourself with a circular saw could lead to a large cut or possible amputation |  |
|  | High risk construction equipment use (e.g. lasers, EPT) | There is high risk when using EPT such as ramset guns as the tool has the force to cause serious injuries if not used properly with correct PPE. |  |
|  | Crane use | Crane use is high risk as they lift such heavy loads and if there is a fault a serious injury could occur. |  |
|  | Demolition | Demolition has medium risks as although precautions are taken injuries can still occur |  |
|  | Formwork | Formwork has medium risks as there are protocols to complete the work safely injuries can still occur |  |
|  | Scaffolding use | Scaffolding has medium risks similar to formwork, protocols are in place to carry out work safely but injuries and accidents can still occur |  |
|  | Work at heights | Medium risk for working at heights as all people doing it have the proper qualifications to do so |  |
|  | Welding | Low risk for welding as anyone welding on site will have the proper PPE on to ensure their safety |  |

Identify an appropriate control measure for each of the hazards identified in Q1.

| Answer | Job | Control measure |  |
| --- | --- | --- | --- |
|  | Construction equipment use (e.g. power tools, concrete mixers) | A control measure for using a power tool such as a circular saw would be to ensure you are always using a sharp blade to avoid any kickback from the saw which could result in injury |  |
|  | High risk construction equipment use (e.g. lasers, EPT) | A control measure for using an EPT such as a ramset gun would be to make sure the tool is serviced and cleaned regularly to avoid any misfiring or problems with the tool |  |
|  | Crane use | A control measure would be to make sure that no workers ever stand underneath a load being lifted and in turn ensure that the crane crew know not to lift over anybody |  |
|  | Demolition | A control measure would be to set up all appropriate exclusion zones to keep people out of the area you are demolishing |  |
|  | Formwork | A control measure for formwork is for barriers and signage to be used to prevent people from walking into an area where the formwork is being built |  |
|  | Scaffolding use | A control measure for scaffolding would be if you are doing the exterior of the building to have an exclusion zone and a spotter down on the ground to protect any workers or members of the public |  |
|  | Work at heights | A control measure for working at heights is to make sure there is edge protection and a catch deck where possible to try and eliminate the risk of falling from height |  |
|  | Welding | A control measure for welding is to make sure all appropriate signage is up to make people aware that welding is happening in the area so workers can try to avoid it or be careful where they look |  |

True or False (Tick your response): Risk control options can come from both internal sources (e.g. policies and procedures) and external sources (e.g. codes and standards).

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

Name two (2) workplace personnel you may speak to for safety advice.

| Answer | - Site safety officer - Your company representative in the safety committee |  |
| --- | --- | --- |

Referring to the WHS Regulations, explain how the following documents contribute towards identifying hazards and risks in the workplace.

| Answer | Document | Description |  |
| --- | --- | --- | --- |
|  | Safe Work Method Statements | As per Division 2 of Part 6.3 in the WHS Regulations, a safe work method statement must: - Identify the work that is high risk construction work - State hazards relating to the high-risk construction work and the risks to health and safety associated with these hazards - Describe the measures to be implemented to control the risks - Describe how the control measures are to be implemented, monitored and reviewed |  |
|  | Site Safety Plan / WHS Management Plan | As per Division 2 of Part 6.4 in the WHS Regulations, a WHS Management Plan must include: - The names, positions and health and safety responsibilities of all persons at the workplace whose positions or roles involve specific health and safety responsibilities in connection with the project - The arrangements in place, between any persons conducting a business or undertaking at the workplace where the construction project is being undertaken, for consultation, cooperation and the coordination of activities in relation to compliance with their duties under the Act and this regulation - The arrangements in place for managing any work health and safety incidents that occur - Any site-specific health and safety rules, and the arrangements for ensuring that all persons at the workplace are informed of these rules - The arrangements for the collection and any assessment, monitoring and review of safe work method statements at the workplace. |  |

Identify two (2) advisory standards you might refer to safely undertake building and construction work and briefly summarise their requirements.

| Answer | Standard name | Requirements |  |
| --- | --- | --- | --- |
|  | Formwork code of practice | Workers and other people at the workplace must take reasonable care for their own health and safety, co-operate with reasonable policies, procedures and instructions and not adversely affect other people’s health and safety. |  |
|  | Scaffolding code of practice | Workers have a duty to take reasonable care for their own health and safety and that they do not adversely affect the health and safety of other persons. Workers must comply with any reasonable instruction and cooperate with any reasonable policy or procedure relating to health and safety at the workplace. |  |

Building and construction industry contracts require building legislation and the National Construction Code to be observed. Refer to these documents when responding to Q8. to Q10.

Identify the requirements in the National Construction Code Volume 1 with regards to air pressurisation for fire-isolated exits. Identify the relevant section of the Code and the Australian Standard referenced.

| Answer | Code section(s) | Requirements and Standards |  |
| --- | --- | --- | --- |
|  | NCC Volume 1 Section D1.7 | (d) If more than 2 access doorways, not from a sanitary compartment or the like, open to a required fire-isolated exit in the same storey— (i) a smoke lobby in accordance with D2.6 must be provided; or (ii) the exit must be pressurised in accordance with AS 1668.1. |  |

Identify the requirements in the National Construction Code Volume 2 with regards to smoke alarms. Identify the relevant section of the Code and the Australian Standard referenced.

| Answer | Code section(s) | Requirements and Standards |  |
| --- | --- | --- | --- |
|  | NCC Volume 2 Section Part 3.7.5.2 | Smoke alarms must— (a) be located in— (i) Class 1a buildings in accordance with 3.7.5.3 and 3.7.5.5; and (ii) Class 1b buildings in accordance with 3.7.5.4 and 3.7.5.5. (b) comply with AS 3786, except that in a Class 10a private garage where the use of the area is likely to result in smoke alarms causing spurious signals, any other alarm deemed suitable in accordance with AS 1670.1 may be installed provided that smoke alarms complying with AS 3786 are installed elsewhere in the Class 1 building; and (c) be powered from the consumer mains source where a consumer mains source is supplied to the building; and (d) be interconnected where there is more than one alarm |  |

Name the building legislation (Act and Regulations) that apply within your jurisdiction.

| Answer | #### QBCC Act This legislation addresses licensing, the licensee register, statutory insurance and building products. - Queensland Building and Construction Commission Act 1991 - Queensland Building and Construction Commission Regulation 2018 - Queensland Building and Construction Commission (Minimum Financial Requirements) Regulation 2018 #### Building Act The QBCC administers provisions of the Building Act 1975 including certification, pool safety and non-conforming building products. - Building Act 1975 - Building Regulation 2006 |  |
| --- | --- | --- |

Which of the following should be considered when developing a risk control plan? Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | How and when controls will be implemented |  |
|  |  | Evaluation processes for implemented controls |  |
|  |  | Consultation with individuals and parties |  |
|  |  | Responsibilities for implementation |  |

True or False (Tick your response): If control measures do not adequately control risks, they must be revised.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

Which of the following are appropriate techniques, tools and processes for identifying hazards and risks? Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Workplace inspections |  |
|  |  | Compensation claim information |  |
|  |  | Safe Work Method Statements |  |
|  |  | Information issued by WHS regulators |  |

Which of the following can be used to assist with workplace inspections? Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Hazard and risk checklists |  |
|  |  | Job Safety Analysis (JSA) |  |
|  |  | Manifests and registers |  |
|  |  | Surveys |  |
|  |  | Workplace walk-through |  |

Which of the following are limitations of using a generic hazard/risk assessment checklist? Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Specifics of the workplace are not considered |  |
|  |  | Not all checklist points will be applicable |  |
|  |  | Such checklists/processes must be completed on a computer |  |
|  |  | You always have to purchase these checklists |  |

Identify the applicable WHS Act in your jurisdiction. Identify its requirements for PCBUs with regards to identifying hazards and risks.

| Answer | Act name and section(s) | Requirements |  |
| --- | --- | --- | --- |
|  | Workplace Health and Safety Act 2011 Subdivision 6 | 70 General obligations of person conducting business or undertaking ( (1) The person conducting a business or undertaking must— (c) allow any health and safety representative for the work group to have access to information that the person has relating to— (i) hazards (including associated risks) at the workplace affecting workers in the work group; and (ii) the health and safety of the workers in the work group; |  |

Identify the applicable WHS Regulation in your jurisdiction. Identify its requirements for PCBUs with regards to identifying hazards and risks.

| Answer | Regulation name and section(s) | Requirements |  |
| --- | --- | --- | --- |
|  | Work Health and Safety Regulation 2011 – part 3.1 S34 | A duty holder, in managing risks to health and safety, must identify reasonably foreseeable hazards that could give rise to risks to health and safety. |  |

Identify a relevant policy in your organisation with regards to identifying hazards and risks and briefly describe its requirements.

| Answer | Policy name | Requirements |  |
| --- | --- | --- | --- |
|  | Work Health and Safety Policy | The policy outlines that: “It is the responsibility of all people working at Action Formwork to: …. (c) Report any actual or potential safety hazards or unsafe work practices through the reporting WHS issues process” |  |

Identify the statutory requirements in your jurisdiction with regards to the completion of inspection reports.

| Answer | Legislation name and section(s) | Requirements |  |
| --- | --- | --- | --- |
|  | # Work Health and Safety Act 2011 - 103E - Assessment reports | The work health and safety officer must – A – Prepare a report (an assessment report) for the assessment carried out under section 103D(1); and B – if the assessment identifies any risks to health and safety arising from the work carried out as part of the conduct of the business or undertaking – include in the report recommendations about managing the risks The work health and safety officer must, within 30 days after the assessment has been carried out, give a copy of the assessment report to— A – if the health and safety officer is not the person conducting the business or undertaking – the person conducting the business or undertaking; and B – if there is a health and safety committee at the business or undertaking – the committee |  |

Which of the following individuals and parties may require information about workplace hazards and proposed risk controls? 
Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Health and Safety Committees |  |
|  |  | PCBU |  |
|  |  | All members of the public |  |
|  |  | Workers |  |

Identify the requirements in the Code of Practice with regards to consulting parties when managing risks.

| Answer | Legislation name and section(s) | Requirements |  |
| --- | --- | --- | --- |
|  | How to manage work health and safety risks Code of Practice 2011 (QLD) | WHS Act section 46: The WHS Act requires that you consult, co-operate and co-ordinate activities with all other persons who have a work health or safety duty in relation to the same matter, so far as is reasonably practicable WHS Act section 47: The WHS Act requires that you consult, so far as is reasonably practicable, with workers who carry out work for you who are (or are likely to be) directly affected by a work health and safety matter. WHS Act section 48: If the workers are represented by a health and safety representative, the consultation must involve that representative |  |

Identify the requirements in the Code of Practice with regards to consulting parties when developing procedures for communicating WHS policy and practice requirements.

| Answer | Legislation name and section(s) | Requirements |  |
| --- | --- | --- | --- |
|  | Work health and safety consultation, co-operation and co-ordination Code of Practice 2011 Section 2.4 | Developing Procedures – A procedure sets out the steps to be followed for work activities. You must consult with affected workers when developing procedures for: resolving work health and safety issues consulting with workers on work health and safety monitoring workers’ health and workplace conditions providing information and training. |  |

Name three (3) parties who should consult to determine effective strategies for communicating WHS policy and practice.

| Answer | 1. Management 2. Health and safety representatives 3. Health and safety committees |  |
| --- | --- | --- |

Name two (2) communication strategies that may be used in the building and construction industry.

| Answer | 1. Issued site specific instructions and signage 2. Written communications, including memos and emails |  |
| --- | --- | --- |

Name two (2) education programs that may be implemented in the building and construction industry.

| Answer | 1. General and site-specific induction training 2. Other forms of specialist and targeted training |  |
| --- | --- | --- |

Fill in the blanks for the statement below:

consulted

training

The ‘How to manage work health and safety risks Code of Practice 2011’ identifies that control measures are more effective when ______________________ is provided and when workers are _____________________.

| Answer |  |  |
| --- | --- | --- |

Explain why it is important to review the effectiveness of communications and educational programs.

| Answer | After you have established appropriate education and training programs for staff, it is important to review them. Should communications and/or training have taken place, yet WHS hazards/risks are unaffected (i.e. not reduce in likelihood of occurrence), then they must be reviewed. |  |
| --- | --- | --- |

List three (3) methods for reviewing the effectiveness of communication and educational programs.

| Answer | 1. Reviewing training materials for accuracy of content, currency and suitability 2. Conducting post-training surveys 3. Ensuring employees have an adequate understanding |  |
| --- | --- | --- |

CPCCBC4002A

Manage occupational health and safety in the building and construction workplace

PRACTICAL ASSESSMENT INSTRUCTIONS - The following practical assessment will require the Participant to complete work related activities. - The Assessor will conduct observations of the Participant’s work to determine whether they display sufficient practical skills and knowledge. - Please fill in the details below – providing your name, signature and date of assessment. - The Participant’s signature acknowledges that the Assessor provided clear instructions and the Participant understood what was required to complete this assessment. - The Assessor may substitute a Practical Task with an alternate task of equal value. - Alternate tasks must be fully documented by the assessor in the Assessor Comments area.

| Participant name | Matt Cumiskey |
| --- | --- |
| Participant signature |  |
| Date | 12/01/2020 |

| Assessor name |  |
| --- | --- |
| Assessor signature |  |
| Date | Click here to enter a date. |

| Task | Task outcome (Tick) |  |
| --- | --- | --- |
|  | Satisfactory | Not Yet Satisfactory |
| Task 1 (Occasion 1) |  |  |
| Task 1 (Occasion 2) |  |  |

Assessor Comments

CPCCBC4002A

Manage occupational health and safety in the building and construction workplace

| Practical Task 1 (2X) |  |  |  |
| --- | --- | --- | --- |
| Description of task: On two (2) occasions, a suitable observer is to verify the participant’s ability to conduct and report on a WHS risk analysis, including workplace inspection. The participant is to gather evidence of how they have completed each component of the Practical Task, which is to be submitted with this Practical Assessment sheet. Evidence to be submitted may include: - A copy of a Site Specific Safety Plan or Construction Safety Plan, including: - Copies of Risk Assessments completed on a project - Copies of Safety Work Method Statements used on a project - A copy a workplace safety inspection checklist that has been used on a project The observer is to provide detailed comments on the tasks completed by the participant. The assessor is to verify the observer’s comments in the Assessor Comments section, and provide the Practical Assessment outcome on the front cover of this document. |  |  |  |
| Resources required: Computer. |  |  |  |
| Assessment of task |  |  |  |
| Name of participant: |  |  |  |
| Name of observer: |  |  |  |
| Role of observer (Confirm suitability to sign) |  |  |  |
| Email address of observer |  |  |  |
| Signature of observer: |  |  |  |
| Date: | Click here to enter a date. |  |  |
| When completing the task, did the participant: |  | Tick if satisfactorily completed. Provide comments if left unticked. |  |
|  |  | 1st Occasion | 2nd Occasion |
| - Conduct a workplace inspection to evaluate construction site safety for the range of occupations in the workplace according to WHS legislation and company policies, including: - Identifying construction hazards and potential risk areas - Identifying and prioritising hazards - Identifying and prioritising risks |  |  |  |
| - Seek expert advice as appropriate, including: - Advice from workplace personnel regarding the application of building codes and standards - Consulting with relevant parties regarding compliance with statutory requirements (including SWMS and site safety plans) - Consulting with appropriate personnel to determine effective strategies for communicating WHS policy. |  |  |  |
| - Complete an inspection report in accordance with best practice and statutory obligations: including: - Recommendations from findings - Required approaches to remediation |  |  |  |
| - Implement agreed control measures in conjunction with relevant workplace personnel, implementing negotiation and conflict resolution skills, as necessary. |  |  |  |
| - Monitor and review control measures, updating records with findings |  |  |  |
| - Establish communication strategies and educational programs specific to the building and construction industry in accordance with statutory requirements and best practice. |  |  |  |
| - Review the effectiveness of communication and educational programs, updating records with findings. |  |  |  |
| - Effectively communicate using the following methods: - Verbally using language/concepts appropriate to cultural differences - Non-verbal communication |  |  |  |
| The Participant’s performance was: Satisfactory Not Satisfactory |  |  |  |

observer Comments