RIICWM402E

Supervise civil works contractors

ASSESSMENT SUMMARY

The assessment activities for RIICWM402E Supervise civil works contractors cover all aspects of the unit of competency. For Apprentices/Trainees, this includes supervisor completion of the Apprentice/Trainee Record (ATR).

Participants must read all the information given before starting any assessment task. The assessment tasks are intended to be fair, flexible, valid and reliable. If a Participant has any questions or requires alternative arrangements to be made such requests should be directed to their Trainer/Assessor, who should note such arrangements in the ‘Assessor Comments’ on the affected assessment. If a Participant feels they are not yet ready to be assessed, they should discuss their options with their Trainer/Assessor.

To satisfactorily complete the assessment, the Participant is required to answer all questions correctly and demonstrate their skills to industry standards. If a Participant does not answer all questions or perform all tasks, they will be deemed ‘Not Satisfactory’ for the task and ‘Not Yet Competent’ for the unit of competency.

The Trainer/Assessor can provide the Participant with additional information on interpreting the assessment outcomes and guide the Participant’s options. If a Participant is still deemed ‘Not Satisfactory’ they will have the opportunity to undertake a supplementary assessment or appeal the result as per the process in the Student Handbook.

By signing below, the Participant acknowledges:

- They understand the assessment instructions and requirements, and consent to being assessed.

- The work submitted is to be their own work – with the work of/with others needing to be clearly acknowledged and documented.

- They consent to any photo/video recording of their work that occurs during assessment.

| Participant name | Benjamin Wixon |
| --- | --- |
| Participant signature |  |

Pre-requisite unit(s) to be achieved and authenticated prior to undertaking assessment

 Not applicable

| Task | Date completed | Assessment Results |  |
| --- | --- | --- | --- |
| Theory Exam | Select Date. |  Satisfactory |  |
| Practical Assessment | Select Date. |  Satisfactory |  |
| Outcome for RIICWM402E | Select Date. |  Competent |  |

Evidence requirements checklist (ticked by assessor and evidence supplied to RTO)

 Theory Exam  Practical Assessment

| Assessor Name |  |
| --- | --- |
| Assessor Signature |  |

RIICWM402E

Supervise civil works contractors

THEORY EXAM

| Participant name | Benjamin Wixon |
| --- | --- |

FOR THE PARTICIPANT

Instructions

- You will complete written questions as outlined on the following pages. You must attempt all questions and provide enough detail in your responses to demonstrate your knowledge. - It is anticipated that you should complete this assessment within two (2) hours, but additional time may be allowed upon negotiation with the Assessor. - You must complete all questions unassisted by the Assessor or other personnel but may refer to reference material as needed. All questions must be answered correctly for a ‘Satisfactory’ assessment outcome to be awarded. - An oral examination may be allowed under certain circumstances, whereby your answers will be documented by another party. - Any written questions that are incomplete, or not answered satisfactorily will need to be completed again after further training support. - If you answer a question incorrectly but modify your response, put your initials next to the written change (or the Assessor must initial a verbal change).

Resources required

Pen

FOR THE ASSESSOR

Instructions

- Where oral examination has occurred, ensure that this is acknowledged in writing (e.g. FT001 and Assessor Comments) - All questions must be satisfactorily demonstrated by the Participant for a Satisfactory determination (indicated by an Assessor tick to the right of the question). - Assessors are to complete marking in pen of a different colour to the Participant. - If the Participant verbally modifies their response, write in the modified response verbatim and put your initials next to the written change.

ASSESSOR COMMENTS

As necessary, outline: - Details on reattempts, alternative arrangements or reasonable adjustments made. - Where a Participant is deemed ‘Not Satisfactory’, provide recommendations for future training.

What preparation is required to determine a job plan for a contractor?

| Answer | - Figure out what the whole scope of the works actually is. Get all the paperwork as soon as possible. Check contractor costings against our own costing forecasts. Same with schedules and labour needs. Break up the job into manageable pieces (earthworks, drainage, pavements) then break those down into smaller tasks if needed. Know the timelines — Start dates and finish dates, key milestones, interfaces with other works that will be affected. Check the resources. I.e for drainage who is supplying the culverts? Are the subbies doing a supply and install or just install? Who needs to order and confirm concrete? Who orders the backfill? Think about what can go wrong — Weather, material delays, labour shortages — have a backup plan or other works the crew can continue on with in case of delays Check against the contract. Make sure the subbies tasks line up with our safety and environmental requirements. |  |
| --- | --- | --- |

How will you share emergency response information relevant to the site and various tasks with sub-contractors?

| Answer | Site relevant information will be shared with all parties through the initial site induction, workpacks with relevant SWMS attached, toolbox talks and pre-start meetings |  |
| --- | --- | --- |

Which on-going risk management procedures do you follow and access when supervising civil work contractors and how do these link to legislative requirements?

| Answer | (WHS) Act 2011 (Qld) and WHS Regulation 2011 (Qld) legally require you to identify risks, manage them, and protect the health and safety of workers and others on site. To help identify any risks on site, contractors can do take 5's/JHA/JSEA to identify the day to day risks. Workpacks can help forecast works and therefore forecast future risks as well. Once a risk is identified, often the hierarchy of controls is the most suitable and easiest way to address the risk. |  |
| --- | --- | --- |

What are (1) the objectives of the Environmental Management Plan (EMP) and (2) why should you read this EMP as part of planning the job?

| Answer 1. 2. | The Environmental Management Plan (EMP) sets out how to meet environmental, cultural, and heritage responsibilities on site, in line with legal requirements. It covers erosion control, dust, noise, waste, waterways, and procedures for protecting cultural heritage. the EMP ensures that all environmental, cultural, and heritage responsibilities are met, aiding in effective and compliant job planning. |  |
| --- | --- | --- |

In the general principles of surveying, what is the importance of ‘working from the whole to the part’ (1) when surveying and (2) how this is achieved?

| Answer 1. 2. | it ensures accuracy. By creating a strong framework of control points first, you reduce errors and prevent them from spreading across the survey. Achieving this involves setting up these broad control points before zooming into smaller, detailed measurements within the framework |  |
| --- | --- | --- |

Why is it important to coordinate tasks with other work supervisors and schedule contractor activities?

| Answer | Coordinating tasks with other supervisors and scheduling contractor activities is crucial to ensure smooth workflow and avoid conflicts or delays. It helps align resources, manage overlapping projects, and confirm that teams have what they need at the right time. |  |
| --- | --- | --- |

What processes must you follow to acquire and confirm the acquisition of, required resources at your workplace?

| Answer | To get the right resources at work, I start by figuring out exactly what’s needed, then check in with engineers or supervisors to make sure it all meets our workplace requirements. I’ll put together a clear list, including helpful links or pictures from websites or magazines to avoid any confusion. Before ordering, I’ll double-check that the items are good quality, available from the supplier, and that delivery times work for our schedule. Once everything’s confirmed, I’ll get approval from a manager so the purchase order can be issued. |  |
| --- | --- | --- |

How do you communicate site safety requirements to ensure contractors wear PPE suited to the site and task?

| Answer | All general PPE requirements should be included in the site inductions. Further, site signage, toolbox talks and JSEAs are used. For task relevant PPE, SDS's SWMS etc can be utilised. |  |
| --- | --- | --- |

What is the ‘rule of thumb’ basis for matching a machine to a job?

| Answer | It involves considering operational distances and the balance between power and speed. Short distances is usually suited for higher power machines, Dozers etc Long distances is usually suitable for higher speed machines, Moxies, scrapers |  |
| --- | --- | --- |

Supervisors must have knowledge of factors affecting plant performance. What are four (4) factors that can affect plant performance and output and why must supervisors monitor each of these?

| Answer 1. 2. 3. 4. | Operator Experience: Competent operators ensure efficient and safe operation of machinery Task Suitability: Conditions like material type, site environment, and weather impact plant effectiveness. Pre and Post Operational Checks: Regular checks maintain machinery in optimal condition. Supervisors must ensure these are conducted to avoid breakdowns and delays Safety Guidelines: Adherence to manufacturer and site-specific safety protocols prevents accidents and ensures smooth operation. |  |
| --- | --- | --- |

How do you confirm that the contractor has a job plan available that meets task requirements?

| Answer | You need to review the plan against the project's scope and specifications. Ensure it aligns with the work breakdown structure, includes necessary resources, and adheres to safety and compliance standards. |  |
| --- | --- | --- |

What methods can you apply to confirm contractors have communicated instructions to team members and others involved?

| Answer | Supervisors can use methods like observing toolbox talks, reviewing written communication such as emails or meeting minutes, and conducting follow-up checks with team members. Regular performance monitoring and feedback sessions also help ensure instructions are understood and implemented correctly. |  |
| --- | --- | --- |

What is required when setting out for work tasks?

| Answer | It is essential to define the boundaries, lines, and levels of the job using control pegs to ensure the work aligns with plans and specifications. This process typically involves scopes like site clearing, earthworks, subgrade and drainage, and pavement sett out. Surveyors usually establish these control points, which are then used by supervisors or operators to guide the precise execution of the task. |  |
| --- | --- | --- |

What is the importance of the survey pegs during the road set out?

| Answer | Survey pegs are crucial during road set out as they define the boundaries, lines, and levels of the road, ensuring the construction aligns with the plans and specifications and Australian Standards |  |
| --- | --- | --- |

What methods do you apply when monitoring civil works to ensure the required outcomes or met on a daily basis?

| Answer | supervisors can use methods like routine toolbox and pre-start meetings to align teams on tasks and expectations for the shift. Regular performance checks against job plans, observing task execution, and maintaining site diaries help track progress, delays and improvements/ |  |
| --- | --- | --- |

Name and explain the purpose for three (3) of the different test results the supervisor must be able to read and understand at different hold or witness points of a project.

| Answer 1. 2. 3. | soil tests moisture, plasticity, permeability etc sub-grade and sub-base tests: compaction, sand replacement compaction tests: used to find out the low workability of concrete |  |
| --- | --- | --- |

What is the purpose of road surface drainage and what are four (4) elements used to achieve this effectively?

| Answer Purpose Element 1. 2. 3. 4. | Road surface drainage deals with the drainage of stormwater runoff from the road surface and the surfaces adjacent to the road formation Crossfall Table Drains Culverts Catch drains |  |
| --- | --- | --- |

What are the effects of inappropriate road drainage infrastructure?

| Answer | Inappropriate drainage can lead to an array of problems in the future, from erosion of land to vehicles hydroplaning and constant pavement damage |  |
| --- | --- | --- |

What are the three (3) main parts of roadway design and the importance of these features?

| Answer 1. 2. 3. | Subgrade: Subgrade provides the foundation for everything above it. If it isnt placed correctly, the road can shift, crack and fail. Pavement: the pavement layers add strength and stabiiility by using layers of highly tested gravels Wearing surface: the wearing surface provides a weather resistant, safe and smooth driving surface. It also protects the previous layers from moisture entering. |  |
| --- | --- | --- |

What are four (4) events that might occur on a normal working day that may significantly affect progress of civil works tasks and force the use of contingency plans.

| Answer 1. 2. 3. 4. | hold-ups with supplies weather unavailability of materials sickness |  |
| --- | --- | --- |

What are four (4) contingency plans that may be in place to support a civil construction operational plan?

| Answer 1. 2. 3. 4. | Weet weather plans: can we use this opportunity to potentially train and upskill some of the workforce? Supply delay plan: alternate suppliers, rescheduling works and delivveries Plant breakdown plans: which fitters are available, contacts for subcontract equipment Labour shortage plans: subcontact labour hire, reassigning crews to deal with critical path works. |  |
| --- | --- | --- |

What methods do you apply when monitoring worker and contractor performance for safety and compliance?

| Answer | Supervisors often have KPIs which address safety and compliance. Weekly task observations, Site inspections, SWMS revisions etc. |  |
| --- | --- | --- |

What is the relationship between waste management and environmental protection?

| Answer | In civil construction, managing waste is a key part of protecting the environment. This means making sure materials are disposed of correctly, following environmental rules, and preventing pollution or damage to the surrounding area. By sorting and handling waste the right way, construction teams help reduce their impact on nature and make sure they’re meeting legal and environmental standards. |  |
| --- | --- | --- |

Describe why supervisors access each of the following types of data.

| Answer | Geological Data: Supervisors use geological data to understand what’s happening below the ground—like soil type, rock conditions, and groundwater. This helps them plan civil works properly and make informed decisions, especially when interpreting test results like soil compaction or concrete strength. Hydrological Data: Hydrological data gives supervisors information about water on and around the site, such as surface water, groundwater, and water quality. They use this data to manage water flow, protect the environment, and make sure the site stays safe and compliant with regulations. Meteorological Data: Supervisors check meteorological data to plan work around weather conditions. It helps them decide whether tasks can go ahead safely and avoid risks caused by things like rain, wind, or extreme heat, especially when those conditions might impact safety or the environment. |  |
| --- | --- | --- |

RIICWM402E

Supervise civil works contractors

PRACTICAL ASSESSMENT

| Participant name | Benjamin Wixon |
| --- | --- |

FOR THE PARTICIPANT

Instructions

- The tasks outlined on the following pages will require performance of normal workplace activities, as a result of a structured activity set by an Observer in the workplace or a simulated activity set by your Assessor/Observer. - The timeframe for these tasks will be established by your Assessor/Observer, in consideration of location conditions and industry requirements. - You must complete all tasks unassisted by the Assessor or other personnel unless the steps note this assistance. - You must attempt all tasks (and itemised steps) for a ‘Satisfactory’ assessment outcome to be awarded. - For any observation conducted that is incomplete, or without satisfactory performance, the observation will need to be completed again after further training support.

FOR THE ASSESSOR/OBSERVER

Instructions

- The following observation record should be used to judge and record your observations of the Participant as they complete the task(s) set. You must record your observations of the Participant’s performance. - For each occasion, indicate the task performed and whether it was a simulation or workplace-based task. - Any simulated assessment must ensure the Participant still performs under workplace conditions which reflect industry standards. - In the comments, provide clear detail regarding the task that the Participant was required to do (e.g. PPE used, materials handled, attachments connected, plant types). - All steps must be satisfactorily demonstrated by the Participant for a ‘Satisfactory’ determination (indicated by an Assessor tick to the right of each itemised step, or an Observer tick to the right of each itemised step, and confirmation by an Assessor in the Assessor Comments). - Where you have determined that one or more task steps have not been satisfactorily demonstrated, provide the Participant with additional support and outline details of such assistance in the Assessor/Observer Comments. Allow the Participant two (2) further opportunities to demonstrate the task again to allow an opportunity for a ‘Satisfactory’ assessment outcome to be achieved. - If after three (3) attempts the Assessor determines that the Participant cannot demonstrate the required steps, a ‘Not Satisfactory’ outcome is to be determined.

| TASK OUTCOME |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Task | Date |  | Task Type (Tick) | Location | Task outcome (Tick) |
|  |  |  |  |  | Satisfactory |
| Task 1 | Select Date. |  |  Workplace  Simulation |  |  |
| Task 2 (Occasion 1) | Select Date. |  |  Workplace  Simulation |  |  |
| Task 2 (Occasion 2) | Select Date. |  |  Workplace  Simulation |  |  |

ASSESSOR COMMENTS

As necessary, outline: - Details on reattempts, alternative arrangements or reasonable adjustments made. - Where a Participant is deemed ‘Not Satisfactory’, provide recommendations for future training.

| ASSESSOR COMMENTS: PRACTICAL TASK 2: OCCASION 1 |  |
| --- | --- |
|  Individual student (as named on cover page)  Multiple students [ All from Attendance Report,  Specific students involved – identify students] |  |
| Task | Provide detail |
| Error: Reference source not found | Specify task to be completed: |
| Error: Reference source not found | Completed Task 1 meets these requirements: |
| Error: Reference source not found | Specify tool and method applied to address potential risks, hazards and environmental issues and implement control measures. |
| Error: Reference source not found | PPE worn/applied (tick or specify):  Hi-viz  Helmet  Gloves  Safety boots  Eye protection  Ear protection  Other (please specify): |
| Error: Reference source not found | Specify method used to consult with contractors: |
| Error: Reference source not found | Specify method applied to confirm resources are available: |
| Error: Reference source not found | Specify new potential hazards recognised and controlled: |
| Error: Reference source not found | Specify what has been set out: |
| Error: Reference source not found | Specify checks/measurements made: |
| Error: Reference source not found | Communication applied (tick or specify):  Face to face  Marking on pegs  Stringlines  UHF  Hand signals  Whistles  Other (please specify): |
| Error: Reference source not found | Specify adjustments made: |
| Error: Reference source not found | Outline recommendations made: |

| ASSESSOR COMMENTS: PRACTICAL TASK 2: OCCASION 2 |  |
| --- | --- |
|  Individual student (as named on cover page)  Multiple students [ All from Attendance Report,  Specific students involved – identify students] |  |
| Task | Provide detail |
| Error: Reference source not found | Specify task to be completed: |
| Error: Reference source not found | Specify tool and method applied to address potential risks, hazards and environmental issues and implement control measures: |
| Error: Reference source not found | PPE worn/applied (tick or specify):  Hi-viz  Helmet  Gloves  Safety boots  Eye protection  Ear protection  Other (please specify): |
| Error: Reference source not found | Specify method used to consult with contractors: |
| Error: Reference source not found | Specify method applied to confirm resources are available: |
| Error: Reference source not found | Specify new potential hazards recognised and controlled: |
| Error: Reference source not found | Specify what has been set out: |
| Error: Reference source not found | Specify checks/measurements made: |
| Error: Reference source not found | Communication applied (tick or specify):  Face to face  Marking on pegs  Stringlines  UHF  Hand signals  Whistles  Other (please specify): |
| Error: Reference source not found | Specify adjustments made: |
| Error: Reference source not found | Outline recommendations made: |

RIICWM402E

Supervise civil works contractors

Practical Task 1

Description of task: The participant must demonstrate undertaking the role of a civil construction supervisor and prepare a job plan which is to be carried out as part of Practical Task 2. The participant must include documentation demonstrating collating of information. If they do not have adequate existing documentation, they will be required to develop a new template for use or one from another job which illustrates the type of information they require. As part of this task, the participant is required to demonstrate the skills and knowledge to: - access, read and correctly interpret work documentation to ensure all work to be completed is compliant - identify and address potential risks, hazards and environmental issues, and implement control measures including selecting and wearing site appropriate PPE - consult with team members to confirm the task requirements - organise for the acquisition and availability of required resources - implement a job plan that meets task requirements Tasks are at the Assessor’s Discretion, but detail of tasks undertaken must be recorded in the Assessor’s Comments (on the back of the Practical Assessment cover page).

Resources required: Work team, work site, organisational policies, standard operating procedures and plans, safety processes, hazard and risk assessment documents relevant to the area of work, Project plans and specifications, EMP, TMP, Engineering survey, Testing schedule requirements, relevant SDS for any chemicals used as part of the work task being assessed.

| When completing the task, did the Participant: | Tick if satisfactorily completed. Provide comments if left unticked. |
| --- | --- |
| 1 Human resource requirements |  |
| - A clear and accurate overall number of workers required to undertake the construction task. |  |
| - Identify appropriate types of workers required for the task (e.g. skilled or unskilled labour, concreters, carpenters, road crews). |  |
| 2 Plant and machinery requirements |  |
| - Identify appropriate types of plant and machinery required to complete the task, and justification their selection. |  |
| - Identify the plant and machinery requiring a trained or licensed operator and any legislative document or note providing this information. |  |
| - Identify how plant and equipment will be procured for the job and what procedures will be followed to undertake this (e.g. hire company, plant hired after requisition form submitted to the site engineer and accounts officer). |  |
| 3 Construction material requirements |  |
| - Interpret site plans and specifications to identify the types of construction materials to be used. |  |
| - Apply accurate calculations of material quantities to be used during the task showing areas, volume or tonnage and wastage factors from plan requirements and engineer requirements. |  |
| - Identify the total value of materials to be used. |  |
| - Document contingency planning in place regarding ordering materials from alternate suppliers. |  |
| 4 Sub-contractor support requirements |  |
| - Describe the requirement for all sub-contractor to undergo induction training prior to entering site. |  |
| - Identify key personnel that sub-contractors will refer to. |  |
| - Provide documentation from site including schedules or charts identifying: - the use of sub-contractors over a given period of time - plans of drawing of work to be completed by sub-contractors - sub-contractor induction records - organisational chart indicating who sub-contractors will report to. |  |
| 5 Waste disposal requirements |  |
| - Identify relevant state authority acts, regulations, codes of practice and standards relating to waste removal (e.g. Environmental Protection Act). |  |
| - Identify and explain how materials will be removed from site (e.g. Will they be taken to landfill, decontaminated). |  |
| - Describe any specific organisational requirements for waste disposal.(as applicable). |  |
| 6 Coordination requirements |  |
| - Identify and describe tasks that occur simultaneously on the job plan. |  |
| - Accurately identify the time required to undertake the main construction steps. |  |
| - Identify the persons required to monitor to different aspects of the project. |  |
| 7 Activity scheduling |  |
| - Clearly identify the overall length of time assigned to the project and significant milestones from production schedules, construction sequence drawings, Gannt charts or similar. |  |
| - Sequence steps effectively with an efficient use of time. |  |
| 8. Materials delivery scheduling |  |
| - Clearly identify the key delivery dates for materials to be used. |  |
| - Identify the locations for site deliveries. |  |
| - Clearly explain the types of trucks, plant and machinery used during the materials delivery process. |  |
| 9. Risk assessment and management requirements |  |
| - Describe methods for prioritising risks onsite (e.g. using risks calculators, incident reports). |  |
| - Describe the processes involved with respect to deciding upon control measures. |  |
| 10. Work health and safety requirements |  |
| - Identify appropriate safety personnel for the site and their roles in the safety system. |  |
| - Provide examples of how hazards may be identified on site through inspections, conducting on going risk assessments, checking reports. |  |
| - Explain the reporting requirements for hazards and incidents on site. |  |
| - Identify the requirements for all workers on site to undergo site specific inductions. |  |
| - Explain procedures for identifying non-hazardous and hazardous materials. |  |
| - Explain procedures for handling substances/ materials onsite (e.g. SDS). |  |
| - Explain the site evacuation procedure including meeting points using the site evacuation plans. |  |
| - Identify and explain the location of firefighting and first aid equipment. |  |
| - Identify and explain the roles of emergency personnel on site (e.g. Fire warden, Chief fire warden). |  |
| 11. Quality management requirements, including testing scheduling requirements |  |
| Identify the tolerances applicable to the civil construction task from plans and specifications. |  |
| Determine from specifications and schedule testing milestones, the types of testing required. |  |
| Identify the personnel required to monitor and confirm testing. |  |
| 12. Traffic management requirements |  |
| - Identify exclusion zones, signage and barriers required for the project from the traffic management plan. |  |
| - Document the personnel required to implement and monitor the traffic management plan. |  |
| 13. Environmental requirements |  |
| - Identify and document how noise, erosion, water quality, waste disposal, flora and fauna and cultural heritage artefacts/ sites were intended to be controlled on site |  |
| 14. Task monitoring requirements |  |
| - Identify the persons required to monitor to different aspects of the project. |  |
| - Provide production charts, toolbox meeting schedule, briefing note, emails, charts diagrams or work instruction requirements for addressing issues with safety/performance. |  |
| 15. Task performance monitoring requirements |  |
| - Provide example documents including production charts, briefing note, emails, charts diagrams or work instruction indicating milestones or timeframes for the construction, organisation charts/diagrams or notes. |  |
| 16. Communication requirements |  |
| - Identify how regularly toolbox meetings occur on site. |  |
| - Explain the procedures for recording attendance and topics during toolbox meeting (agenda, sign on sheets). |  |
| - Identify communication technology which may be used onsite. |  |
| 17. Reporting requirements |  |
| - Document milestones and reporting requirements including reporting to who. |  |

RIICWM402E

Supervise civil works contractors

Practical Task 2 (2X)

Description of task: The participant must demonstrate undertaking the role of a civil construction supervisor, supervising contractors on two (2) occasions. Occasion 1 of this task will relate to the plan prepared in Practical Task 1. During this task, the participant the participant must display the skills and knowledge to effectively, safely and efficiently: - prepare to supervise contractors of civil works - confirm task initiation - confirm contractor has communicated instructions to team members and others involved during pre-start meeting - set out tasks as required for the effective completion of the civil works - while overseeing the execution of tasks: - monitor task performance for safety effectiveness and efficiency - implement adjustments to civil works practice and job plan as required - oversee plant equipment and tools maintenance and confirm maintenance requirements are carried out and recorded according to manufacturer and site requirements - report on the execution of tasks - complete and submit written and verbal reports according to site requirements - recommend improvements to civil works practice. Tasks are at the Assessor’s Discretion, but detail of tasks undertaken must be recorded in the Assessor’s Comments (on the back of the Practical Assessment cover page).

Resources required: Work team, work site, organisational policies, standard operating procedures and plans, safety processes, hazard and risk assessment documents relevant to the area of work, Project plans and specifications, EMP, TMP, Engineering survey, Testing schedule requirements, relevant SDS for any chemicals used as part of the work task being assessed, all materials, tools, plant or equipment required to complete the task, required PPE and relevant safety equipment.

| When completing the task, did the Participant: | Tick if satisfactorily completed. Provide comments if left unticked. |
| --- | --- |
|  | 1st Occasion |
| 1 Plan and prepare to supervise contractors of civil works |  |
| - Identify the location of the site using appropriate project plans. |  |
| - Determine the scope of the work task from plans, drawings and specifications. |  |
| - Complete a job plan demonstrating ability to access, read and apply workplace documentation to confirm legislative, statutory and workplace compliance. |  |
| - Confirm contractor has a job plan available that meets task and compliance requirements |  |
| - Determine and address potential risks, hazards and environmental issues and implement control measures. |  |
| - Select and wear PPE suited to site and tasks. |  |
| - Consult contractors and team members regarding resource requirements for task. |  |
| - Communicate with other supervisors/ team leaders to resolve potential coordination requirements. |  |
| - Identify specific council, state or federal requirements for the site including those relating to finishes, quality, reporting and safety. |  |
| - Confirm required resources including human resources are acquired and made available for task. |  |
| - Explain any specific organisational, client and site requirements from site plans, specifications, geological, geotechnical data. |  |
| - Clearly explain the meteorological data available for the site (rainfall, humidity, temperature and wind), and identify any issues in production (e.g. If a possibility of rain was evident), quality or safe work practices arising from the forecast. |  |
| - Interpret site plans or environmental plans to clearly indicate the location of site wash down facilities. |  |
| 2 Confirm task initiation |  |
| - Facilitate a pre-start meeting with contractor and team. |  |
| - Monitor contractor participation in pre-start meeting to confirm contractor has communicated instructions to team members and others involved to workplace requirements. |  |
| - Discuss any possible construction problems (or lack of), regarding the type of rock found onsite. |  |
| - Discuss any possible construction problems (or lack of), regarding the type of soil types found onsite. |  |
| - Discuss weather conditions and recommend actions. |  |
| - Discuss site hydrological data and associated drainage requirements. |  |
| - Explain the log book, or scheduled servicing procedure for plant and machinery used on the site. |  |
| - Concisely explain how faulty or defective equipment will be reported and recorded onsite using example hazard identification forms. |  |
| - Explain any environmental hazards posed from the use of plant and machinery including control measures to be used to control these environmental hazards. |  |
| - Listen to and respond effectively to questions and queries. |  |
| - Ensure all workers understand the SWMS for the task and have signed their understanding. |  |
| - Walk the work area with contractors and the team, identifying new potential risks, hazards and environmental issues and suitable controls. |  |
| - Ensure signs and barriers have been installed to protect work area as required. |  |
| - Set out task correctly from engineers’ survey pegs using plans, drawings and specifications. |  |
| 3 Oversee execution of tasks |  |
| - Watch workers undertake tasks, checking that tolerances are met as required. |  |
| - Monitor workers for safety concerns, making sure manual handling procedures are followed and work activities are conducted safely. |  |
| - Watch plant and equipment used to confirm machinery is used safely and within machine capabilities and limitations. |  |
| - Clearly communicate with workers regarding task execution using approved and understood communication methods. |  |
| - Write notes in diary if safety or compliance issues arise. |  |
| - Write notes of measurements made, material quality, deliveries and docket checks. |  |
| - Organise and make adjustments to civil works practice and job plan as required and confirm required outcomes are achieved. |  |
| - Confirm personnel allocated has checked that log books have been filled out and scheduled servicing has been undertaken on plant and equipment. |  |
| - Confirm that faulty or defective equipment is reported and recorded according to site requirements. |  |
| 4 Report on execution of tasks |  |
| - Make verbal reports to relevant personnel regarding completion of stages, reaching milestones, testing required. |  |
| - Complete all diary entries clearly and legibly. |  |
| - Ensure all test results and checks are documented and stored according to site requirements. |  |
| - Make verbal or written recommendations regarding changes to task execution. |  |