CONSTRUCTION WORK SITE CHECKLIST

| Site Name: The Drapery |  |  | DATE: 07/01/2020 |
| --- | --- | --- | --- |
| Project/Job: |  | Job No. or Contract No.: |  |
| Omega Apartments |  | 001 357 984 |  |
| Person carrying out site inspection (Name and Position (Print)): | Jim Jones, Safety Officer |  |  |
| Contractor (if applicable): Bigtime Builders |  |  |  |
| Contractor’s Site Supervisor: Matt Cumiskey |  |  | Telephone:0418626049 |

All non-conformances must be rectified immediately or date set for completion, and actions documented.

# (References in brackets relate to clauses in WHS Regulations 2011 or Codes of Practice)

|  | YES | NO | N/A | COMMENTS & ACTIONS |
| --- | --- | --- | --- | --- |
| ## DOCUMENTS |  |  |  |  |
| Is there a Safety Management Plan, covering this work, on site? (if required) | X |  |  |  |
| Are Safe Work Method Statements (SWMS) available on site for all site activities currently being undertaken assessed as having a high risk? (CL 291, 299) | X |  |  | All sub-contractor SWMS are kept in the safety officer’s office |
| Does the SWMS identify site specific hazards and controls related to the work being carried out? | X |  |  |  |
| Are workers on site working in accordance with their SWMS? | X |  |  |  |
| At the time of completion of this checklist all sub-contractors working on The Drapery are doing so in accordance with their SWMS. |  |  |  |  |
| ## INDUCTION |  |  |  |  |
| Are there Industry WHS Induction records on site? | X |  |  | White Card/certificate sighted |
| Have all workers undergone site specific induction training prior to starting work on site? | X |  |  | Inductions are filed and stickers are given to workers to stick on their hard hats after completion of induction |
| # CONSULTATION |  |  |  |  |
| Are Toolbox Talks held prior to site works commencing, and then weekly, or more frequently as required? | X |  |  | ## Pre starts are held every morning before work and sub-contractors provide me with a copy of minutes for their weekly toolbox talk |
| SITE AREA |  |  |  |  |
| a) Security Is the site securely fenced / barricaded / site occupants isolated from any risk? (CL 298) | X |  |  |  |
| Are all materials stored within the secured work site / site shed / vehicles? | X |  |  |  |
| b) Signage Are site safety rules available/displayed on site? (Principal Contractor site safety rules displayed when required) | X |  |  |  |
| When fencing is erected, is there a clearly visible sign with the name & phone number, (including an after hours emergency telephone number) of the Principal Contractor (CL 308) and appropriate Construction & PPE signage displayed? | X |  |  |  |
| c) Emergency /Incidents Where is the nearest phone in case of an emergency? | X |  |  | Location: Site office for a landline but at a minimum all supervisors have mobile phones |
| Is an emergency/incident procedure displayed on site and are workers aware of the procedure? (CL 43) | X |  |  |  |
| Are there suitable fire extinguishers on site (guidance: eg. hot work minimum 4.5kg) & have fire extinguishers been serviced in last six months & tag stamped? (AS 1851.1 Maintenance of Fire Protection Equipment) | X |  |  |  |
| Is there an adequately stocked first aid kit on site? (equivalent to Type B kit plus items identified as required from carrying out a risk assessment) (Code of Practice: First Aid in the Workplace– page 9) | X |  |  |  |
| Is there a qualified first aid person on site? (one first aid person for up to 25 workers for high risk work) | X |  |  | Name/s: Joel Ryan |
| Is there an accident/injury book/register on site? | X |  |  |  |
| d) Amenities – (1 toilet per 20 workers) Are there washing facilities, adequate fresh cool drinking water, clean toilet, lunch and change rooms and bins available? | X |  |  |  |
| e) Waste Disposal Is there a rubbish / waste container on site? | X |  |  |  |
| Is the site area clear of rubbish/scattered materials? | X |  |  |  |
| Is documentation available for any waste/hazardous material disposed of offsite? (eg. tip docket) | X |  |  |  |
| WORKING AT HEIGHTS |  |  |  |  |
| a) Prevention of falls if work area is above 2m and within 2m of the edge - are guard rails or Scaffolding or Harness being used? (harness must be attached) (CL79) | X |  |  |  |
| Is area below (fall zone) fenced/barricaded with warning signage, or spotter below? | X |  |  |  |
| Are tools secured? | X |  |  |  |
| If team has installed Roof Safety Mesh, has a Roof Safety Mesh Handover Certificate been completed & copy left with site owner? (CoP Safe Work on Roofs Part 1) | X |  |  |  |
| b) Ladders Are ladders in good condition, industrial rated with a visible rating sticker, secured top or bottom, extended at least 1 metre above top platform & are timber or fibreglass when working near electrical equipment? (AS1892.5 Portable ladders - Selection, safe use and care) | X |  |  |  |
| c) Scaffolding (CL 225) If the scaffold is under erection, has a safety barrier been set up near the scaffolding base? | X |  |  |  |
| Is proper ladder access provided to scaffold working decks (fixed top/bottom)? | X |  |  |  |
| Is scaffolding secured against tipping? | X |  |  |  |
| Are the planks secure/platforms fully decked? | X |  |  |  |
| Has Scaffolding of 4m or more have a relevant scaff tag or handover certificate? | X |  |  | Monthly certification on site |
| Are handrails, midrails and toe boards in place along the full length of working deck of the scaffold? | X |  |  |  |
| d) Mobile Scaffold Do mobile scaffolds have lockable wheels & are the wheels locked? |  |  | X | Mobile scaffolds will not be used on this site, scissor lifts will be used by people with the appropriate tickets required to operate them. |
| Is safe internal ladder access available above 2m? |  |  | X |  |
| Is the height less than 3 times the width of the base? |  |  | X |  |
| Do outriggers have to be used? |  |  | X |  |
| Have overhead electrical hazards been considered? |  |  | X |  |
| # HAZARDOUS CHEMICALS / SUBSTANCES |  |  |  |  |
| a) Is there a Hazardous Chemicals/Substances Register on site? (CL 346) | X |  |  | Site Office |
| b) Are copies of Safety Data sheets (SDS) available on site for all substances used/stored on site? (CL 344) | X |  |  | Site Office |
| c) Labelling Are all containers of chemicals adequately labelled? (Includes decanted materials that haven’t been decanted for immediate use) (CL 342) | X |  |  |  |
| d) Training Have site workers read SDS and controls listed in SWMS? | X |  |  |  |
| e) Ventilation Is adequate ventilation provided when chemicals/gas bottles are stored in vehicles or in use? | X |  |  |  |
| f) Are appropriate control measures in place for removal of lead based paint? | X |  |  |  |
| g) Are appropriate control measures in place for asbestos removal, & work carried out by licensed person? (CL 458, CoP How to Safely Remove Asbestos) | X |  |  |  |
| # TOOLS / ELECTRICAL |  |  |  |  |
| a) Guards Do the machines/tools on site have guards/handles fitted as per manufacturer’s recommendations? (eg. grinder, circular saw) | X |  |  |  |
| b) Electrical Are electrical leads, plugs & power tools in good condition & are they tested & tagged three(3) monthly? | X |  |  |  |
| Are extension leads off the ground? | X |  |  |  |
| Are all power supply outlets protected by earth leakage devices/RCD’s? | X |  |  |  |
| Sheds/builder’s board tested on installation & then 6 monthly? | X |  |  |  |
| c) When explosive powered tools are being used, are warning signs displayed? | X |  |  |  |
| # PLANT & EQUIPMENT |  |  |  |  |
| a) Documentation Do plant operators have appropriate licences for licensed work? (Schedule 3) | X |  |  |  |
| Does all machinery have registration/permits? (Schedule 4) | X |  |  |  |
| Are Safety Check log books for machinery being completed? | X |  |  |  |
| b) Specific Plant Is traffic control in place if required? | X |  |  |  |
| Do backhoes/excavators/bobcats/cranes/boom pumps/ EWP’s have “Beware of Electrical Hazards” signs? | X |  |  |  |
| Have existing services been verified & are workers aware of safe practices/regulations if working close to power lines &/or underground/hidden services? (eg. dial before you dig, service locator) | X |  |  |  |
| Is plant in good order (visual inspection - no oil leaks, hydraulic hoses OK etc)? | X |  |  |  |
| Is the Safe Working Load indicated on the boom of all cranes / EWP / lifts? (Division 1) | X |  |  |  |
| Are licensed persons involved with mobile crane movements & are slings & cranes in good order, & is there evidence of inspections of these? | X |  |  |  |
| Do vehicles have reverse alarms and rotating orange lights where necessary? | X |  |  |  |
| Are workers wearing high visibility clothing in the vicinity of traffic/mobile plant? | X |  |  |  |
| Are operators of EWP that can exceed 11m, licenced? (Division 1) | X |  |  |  |
| Is plant not in use switched off & key removed? | X |  |  |  |
| EXCAVATION AND TRENCHING WORK (CL 304-306, Code of Practice: Excavation) |  |  |  |  |
| Are barricades/fences/signage along/around trenches? | X |  |  |  |
| Are excavations >1.5m benched, battered, shored or certified by a Geotechnical Engineer to prevent collapse & is safe ladder access provided? | X |  |  |  |
| Is spoil deposited on downhill side of trench/excavation? | X |  |  |  |
| DEMOLITION WORK |  |  |  |  |
| Have gas, electricity and other services been located and isolated/disconnected? | X |  |  |  |
| PERSONAL PROTECTIVE EQUIPMENT (PPE) |  |  |  |  |
| Are Safety Helmets & boots being worn by all persons on work site? | X |  |  |  |
| Are site workers wearing personal protective equipment whilst doing tasks that require it to be worn (eg. dust masks, gloves, eye &/or ear protection)? | X |  |  |  |
| Are supervisors and site workers wearing UV protective equipment / clothing / cream? | X |  |  |  |
| ENVIRONMENTAL MANAGEMENT |  |  |  |  |
| # a) Are workers aware that they need to contact their manager / supervisor immediately if there is any incident including one that may affect the environment? | X |  |  | Eg. Effluent flowing into stormwater drain |
| # Is excess water from cleaning tools & equipment & waste contained – not washed into drains/ waterways? | X |  |  |  |
| # Are excess materials recycled where possible? | X |  |  |  |
| # Are trees protected during site works? | X |  |  |  |
| # Are heritage items identified & protected during works? | X |  |  |  |
| # Are site vehicles parked where they will not unduly impact local use of a street or grounds? | X |  |  |  |
| # Is vehicle access restricted to one stable entry & exit point? | X |  |  | (to reduce damage to ground / grassed areas) |
| # Is noise minimised where possible? | X |  |  |  |
| # Is dust & other particles contained? | X |  |  |  |
| # Is waste disposed of appropriately & in accordance with regulations/local authority requirements? | X |  |  |  |
| # b) Use of hazardous substances – Painting / Cleaning Are washings contained – not disposed of down drain or near drains/waterways? | X |  |  |  |
| # Are paints & chemicals stored in a lockable area that has an impervious floor & ventilation? | X |  |  |  |
| # Are waste solvents, cleaners & paints sealed & disposed of in accordance with guidelines (refer local council)? | X |  |  |  |
| # c) Excavation / Trenching Are silt fences & other sediment control measures in place & functional, if required? | X |  |  |  |
| # Are trenches filled in as soon as work is completed to minimise erosion? | X |  |  |  |
| # Is any surcharge on ground removed? | X |  |  |  |
| # d) Demolition works Are stockpiled materials covered to prevent erosion by wind & rain, or dampened with water to reduce wind blown dust? |  |  | X | There is no demolition work being carried out on site |
| # If dust is a problem, does the fence installed around the site have a cloth barrier to act as a windbreak? |  |  |  |  |

| Item No. | Non-Conformance Issue(s) Items that can be rectified on the day of the inspection are to be listed on this form. For all other non-conformances, a Site Instruction is to be issued. |  |  | Target Action Date | Sign Off when completed |
| --- | --- | --- | --- | --- | --- |
| 001 | General house keeping in certain areas could improve, a plan of areas requiring attention have been emailed to all sub-contractor supervisors |  |  | 08/01/2020 |  |
| 002 | Use of radios on site is prohibited as stated in the site induction yet people have been caught using them, an email has been sent to all sub-contractor supervisors that anyone caught using a radio will be sent home for the remainder of the day and re-inducted the following morning. |  |  | 07/01/2020 |  |
| 003 | General PPE is good but we need to continue to ensure job specific PPE (eg double eye protection for grinding/cutting steel) is used at all times |  |  | 07/01/2020 |  |
| Inspecting Person (Print Name): |  | Jim Jones | Signature: |  |  |