| SITE NAME: 30 Cliveden Av | WORK ACTIVITY: Framing |
| --- | --- |
| OBSERVERS NAME: Paul Newton | SWMS NAME/ NO: 30 Cliveden 002Frame |
| RESPONSIBLE SUBCONTRACTOR: Mitch Henderson | OBSERVATION DATE: 1/12/2020 |

| SWMS Controls required for work being observed |  | Implemented and effective? | Required Action? | Close out date |
| --- | --- | --- | --- | --- |
| 1 | Adequate Scaffolding for second level frame -Fall hazard not to exceed 2.4m | Yes | Additional temporary handrails around stairwells | 25/10/2020 |
| 2 | Edge protection to be installed prior to truss delivery and installation | Yes | Temporary platform to be installed to cover stairwell during roof truss and framing installation. | 25/10/2020 |
| 3 | “Lifting Exclusion zone” to be implemented for truss lift | Yes | Full Marked Lifting area to be designated prior to crane arriving. | 25/10/2020 |
| Safe Behavior Observed Description |  | Communicated to subcontractor | Comments | Close out date |
| Good safety gear worn at all necessary times |  | Mitch Henderson | Safety glasses ,gloves and ear protection worn when required | 22/11/2020 |
| Good practices observed when Truss delivery arrived , good communication with crane truck driver and foreman. |  | Mitch Henderson | Good idea An exclusion zone was implemented during Truss lifting on to top frame. | 22/11/2020 |
| Good understanding and practice of Working at heights requirements and scaffolding in general |  | Mitch Henderson | Care taken when working on second level framework. | 22/11/2020 |
| Unsafe Behavior Observed Description |  |  |  |  |
| Hard hats were not worn during truss lifting /crane operation |  | Mitch Henderson | Hard hats will be implemented in the future and be mandatory when lifting is in progress. | 1/12/2020 |
| Certain scaffolding ties were removed in order to install roof trusses. |  | Mitch Henderson | In Future the scaffolding company must be contacted to do any modifications to the scaffolding. Must not be altered by anyone on site as it could compromise the integrity of the scaffolding. | 1/12/2020 |

confirm that I attended the aforementioned site WHS Consultation meeting.

| OBSERVER NAME: | SIGNATURE: | ROLE: |
| --- | --- | --- |
| Paul Newton | Paul Newton | Builder, Site foreman |
| SUBCONTRACTOR/ WORKER NAME: | SIGNATURE: | ROLE: |
| Mitch Henderson | Mitch Henderson | Subcontractor Carpenter |