CPCCBC4029B

Apply construction information to the sales process

THEORY EXAM INSTRUCTIONS - This exam will occur under standard assessment conditions. - An oral examination may be allowed under certain circumstances. Answers will be documented and FT003 (Oral examination declaration form) must be completed. - Participants must provide enough detail in their responses to demonstrate their knowledge and skills. - Please fill in the details below, providing your name, signature and date of assessment. - Your signature acknowledges that this is your own work. - If a participant initially answers a question incorrectly but modifies their response, they must follow the instructions below: - If a participant changes their response in writing, they must put their initials next to the written change. - If a participant changes their response verbally, this must be noted on the exam paper by the assessor.

| Participant name | Emma Carew |
| --- | --- |
| Participant signature |  |

| Assessor name |  |
| --- | --- |
| Assessor signature |  |

| Date | 14/05/2018 |
| --- | --- |

| Number of questions | Number of questions answered correctly | Satisfactory? |  |
| --- | --- | --- | --- |
| 18 |  | Yes | No |

Assessor Comments

Outline the circumstances in which a building licence must be obtained before carrying out building work in Queensland.

| Answer | In Queensland individuals and companies must hold a QBCC licence to carry out: - Any Building work valued over $3,300.00 - Building work valued over $1,100 where it involves Hydraulic Services Design - Building work of any value where it involves: drainage, plumbing & drainage, gas fitting, termite management – chemical, fire protection, completed residential building inspection, building design – low rise, medium rise and open, site classification. ‘Carrying out building work’ includes: - Personally carry out the work - Directly or indirectly causing the building work to be carried out - Providing administrative, advisory, management or supervising the building work |  |
| --- | --- | --- |

Describe the steps involved in organising electricity supply to site. Provide approximate timeframes, assuming the site is in South East Queensland.

| Answer | The electrical contractor will advise if supply is available to the site Electrical Supply is not available - The electrical contractor can complete an enquiry on behalf of the client - The DNSP (e.g. Energex) conducts network investigations - The DNSP provides an estimate of potential costs and timeframes for the client who can then make an informed decision within 20 business days. Electrical Supply is available - The electrical contractor enters into a Network Connection Agreement with the Distribution Network Service Provider. - If the application is not actioned the Distribution Network will advise of a Network Connection Offer within 10 business days. - After agreeing to connection offer, the electrical contractor prepares the premises for connection - The electrical contractor will submit an Electrical Work Request (EWR) to the DNSP once the work is complete - A copy of the EWR is sent to the clients preferred electricity retailer - After all forms received the DNSP will visit premises to perform requested work within 5 business days for new connections Timeframes for electrical supply are dependent on: - Network availability - Access requirements - Traffic control requirements - Other impacts outside the control of the Distribution Network Service Provider (DNSP). |  |
| --- | --- | --- |

Name two (2) types of approvals that may be sought for the building approval stage.

| Answer | - Full Approval - Staged Approval |  |
| --- | --- | --- |

True or False (Tick your response): Building provision approvals are issued by the local government (e.g. Brisbane City Council).

| Answer | ☒ | True |  |  |
| --- | --- | --- | --- | --- |
|  | ☐ | False |  |  |

Describe the steps involved in organising water supply to site. Provide approximate timeframes, assuming the site is in South East Queensland.

| Answer | The Water Approval process works in conjunction with the Council development assessment process but the applications do not need to be submitted at the same time. Council will require evidence from the developer that a connection is available. Steps involved in organising water supply to site: - Submit a Water Approval application to Queensland Urban Utilities - Standard connections for simple residential developments are usually completed within 5 business days. |  |
| --- | --- | --- |

Which section of the Building Code of Australia outlines energy efficiency performance standards?

| Answer | Section J |  |
| --- | --- | --- |

Describe the long-term benefits of sustainable building solutions. List one (1) environmental benefit and one (1) benefit for the client.

| Answer | Topics | Definition |  |
| --- | --- | --- | --- |
|  | Environmental benefit | - Reduces carbon footprint by producing less waste and reducing harmful gases - Reduces energy and water consumption - Lowering greenhouse gases |  |
|  | Client benefit | - Energy efficient = reduction in water and electricity bills - lowering operational costs - Increase in property value - Lower maintenance costs due to reduced reliance on artificial light or climate contorl - Healthier and more comfortable living environment due to the design of the building and the materials used. |  |

Match the insurance types and definitions by writing the appropriate letter next to each number in the centre column.

| Answer | Insurances |  | Answers | Definition |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1 | Builder’s All Risk | 1 -B | A | Covers personal injury or property damage. |  |
|  | 2 | Indemnity | 2 -C | B | Covers accidental risks of physical loss or damage during construction, third party liabilities and advance loss of profits. |  |
|  | 3 | Public Liability | 3 -A | C | Covers costs where a contractor has caused building errors or omissions. |  |
|  | 4 | Workers’ Compensation | 4 -D | D | Covers death, injury or illness of employees. |  |

True or False (Tick your response): Insurance should be taken out by the contractor prior to starting construction.

| Answer | ☒ | True |  |  |
| --- | --- | --- | --- | --- |
|  | ☐ | False |  |  |

Select and describe five (5) trades involved in the construction process.

| Answer | Trade | Description |  |
| --- | --- | --- | --- |
|  | Excavation | Prepares the site for building by removing all vegetation and excavating the footings |  |
|  | Electrician | Organises electricity to be connected to site, places all wiring for lighting and power cables, fixes switches, installs power points, wires electrical systems |  |
|  | Plumbing | Organises water supply, installs pipes and drains, installs roof and guttering, fits taps, hot water systems and toilets, ensures that all downpipes are fitted and sewer connected |  |
|  | Gas Fitting | Places gas pipes to appliances |  |
|  | Concreting | Installs formwork and boxing, pours concrete for slabs, footings. |  |

Order the following steps from 1 to 10 to identify an appropriate construction sequence.

| Answer | Step Number | Steps |  |
| --- | --- | --- | --- |
|  | 3 | Floor frame |  |
|  | 5 | Windows and external door frames |  |
|  | 7 | Wall insulation |  |
|  | 1 | Set out |  |
|  | 9 | Painting |  |
|  | 6 | Electrical wiring |  |
|  | 2 | Plumbing of waste pipes |  |
|  | 8 | Internal linings |  |
|  | 10 | Landscaping |  |
|  | 4 | Wall frames |  |

For each of the main stages of construction listed below, identify the main trades involved and an approximate duration of each stage for a standard.

| Answer | Stage | Description |  |  |
| --- | --- | --- | --- | --- |
|  | 1: Clearing of the site | Trades | Geotechnical Engineer – Soil Testing Excavating Contractor – Clearing site |  |
|  |  | Duration | Varies |  |
|  | 2: Slab or base stage | Trades | Surveyor – Measuring out design on the site Plumber – Installing under slab drainage Termite Barrier – Install mesh for termite protection Concreter – Install moisture barrier, reinforcing, pour the footings and slab |  |
|  |  | Duration | 1-2 weeks |  |
|  | 3: Frame stage | Trades | Carpenter – Installs internal and external support structure – walls, frames, roof frame Electrician – Installs conduit Plumber – Installs waste pipes Roof Plumber – Roof sheeting, gutters and insulation |  |
|  |  | Duration | 3-4 weeks |  |
|  | 4: Lockup stage | Trades | Carpenter – Installation of all windows, doors. Plasterer – Installation of all internal linings |  |
|  |  | Duration | 4 weeks |  |
|  | 5: Fit-out or fixing stage | Trades | Waterproofing – All wet areas Tiler – Tiling all areas required Plumber – Fitting off all plumbing fixtures, wastes to basins, sink, showers, taps, hot water, toilet suites, dishwasher Carpenter / Cabinet Maker – Installation of all cabinetry units / joinery / shelving for project Plasterer – Installation of all cornices, reveals & architraves Carpenter – Installation of all robes, mirrors Electrician – Fitting off all switches, power points, wiring of fans, stoves, hot water systems, range hood, waste disposal unit and room heaters if required. |  |
|  |  | Duration | 5-6 weeks |  |
|  | 6: Practical completion stage | Trades | Painter – All painting required Completing all installations necessary and finalising any defects – ready to move in. |  |
|  |  | Duration | 7-8 weeks |  |

Identify five (5) factors that can affect the duration of each construction stage.

| Answer | - Availability of Materials - Availability of Labour - Inclement Weather - Changed Conditions - Coordination of Trades on site |  |
| --- | --- | --- |

Describe how each of the following factors can affect construction times.

| Answer | Factor | Effect |  |
| --- | --- | --- | --- |
|  | Changed conditions | If the requirements for the job are altered, this can affect the projected timeframes, in particular where additional labour and materials need to be organised |  |
|  | Labour availability | The construction process sequence is very important when organising labour required. e.g. plaster should be organised after all other trades have performed tasks correctly as electricians and plumbers are unable to access behind walls once they have been sheeted. |  |
|  | Material availability | Materials must be delivered when subcontractors are ready to complete their work so there are no delays. Materials that are hard to find / difficult to source should be ordered well in advance. |  |
|  | Weather | Weather will affect the ability of trades to complete their jobs on time. |  |

Describe the site establishment requirements for each of the points listed.

| Answer | Requirement | Description |  |
| --- | --- | --- | --- |
|  | Earthworks | Contract Dial Before You Dig before undertaking any excavation to ensure that services will not be damaged during construction. |  |
|  | Electricity | Electricity may need to be disconnected from work zones prior to construction and temporary utilities installed for the duration of the construction process |  |
|  | Neighbours | Provide advance notice to neighbours of intended start dates and site contract numbers. |  |
|  | Waste disposal | Set up a designated area for rubbish bins and waste removal. Check if permits are required to place bins on the street. |  |
|  | Water connection | Book in a plumber to perform any required disconnections prior to any demolition taking place. |  |

Identify the section of the Work Health and Safety Regulations 2011 (Qld) that outlines the site establishment requirements listed.

| Answer | Regulation section | Requirement |  |
| --- | --- | --- | --- |
|  | Section 40 | The layout of the work area must allow for persons to enter, exit and move about without risk to health and safety |  |
|  | Section 41 | Adequate facilities must be provided for workers, including toilets, drinking water, washing facilities and eating facilities. |  |
|  | Section 308 | A principal contractor for a construction project must install signs outside the work area that shows their name and telephone contact numbers and site office address. |  |

True or False (Tick your response): The owner is able to access the site at any time.

| Answer | ☐ | True |  |  |
| --- | --- | --- | --- | --- |
|  | ☒ | False |  |  |

Which of the following parties should the client contact to access the site? Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  | ☒ | Contractor |  |
|  | ☐ | Local Government |  |
|  | ☐ | Queensland Building and Construction Commission |  |
|  | ☐ | Subcontractors |  |