Quality Assurance required throughout the project build

Pre-pour checklists

- Engineer inspected including any adjustments

- Plumber signed off

- Electrician signed off

- Mechanical signed off (if required for tundishes etc.)

- Current drawings issued

- Formwork signed off

- Formwork/falsework checked

- RPEQ formwork inspection

- Penetrations formed and meshed

- Edge protection in place (if required for suspended slabs)

- Deck cleaned down (if required for suspended slabs)

- REO signed off by contractor

- Check segregation of services (if required in risers etc.)

- Check construction joints

- Test ordered (compaction cylinders)

- Access for concrete and labour

- Confirm slab RL

- Curing complete using approved method

- Surface inspection at completion of pour

- Instructions issued for non-compliance (if required)

Pre-sheet

- Trades roughed in

  - Electrical

  - Mechanical

  - Fire

  - Plumber

- Plumbing inspection complete

- Variations captured

- Noggins captured

  - Tv

  - Dryer

  - Towel rails

  - Toilet roll holder

  - Mirror

  - Shower screen

- Acoustic zones complete (if required)

- Fire party walls correct (if installed)

- Check access hatches

  - Correct location

  - Correct hatch for sound/fire rating

- Check any ductwork and ventilation installed correctly

- All tundish and AC wastes lines installed correctly with appropriate fall

- Visual check of frames

  - Button head screws behind gyprock

  - Straight and plumb

  - Screws through services

- Ceiling correct height for downlights

- Previous trades cleaned up

- Windows fitted and sealed

- Fire collars and penetrations sticker certified (if required)

Engineers Inspections

- Slabs

  - Compaction tests checked and correct compaction achieved after 28 days

  - Slab thickness correct as per drawings

  - Garage and wet area set downs achieved as per the NCC

- Frame & tie down

  - Frames installed as per structural drawings

  - Tie downs achieved as per NCC

Occupation certificate of classification inspection by certifier

- Final check and signoff by certifier to ensure all items installed to NCC standards