CPCCBC4004A

Identify and produce estimated costs for building and construction projects

THEORY EXAM INSTRUCTIONS - This exam will occur under standard assessment conditions. - An oral examination may be allowed under certain circumstances. Answers will be documented and FT003 (Oral examination declaration form) must be completed. - Participants must provide enough detail in their responses to demonstrate their knowledge and skills. - Please fill in the details below, providing your name, signature and date of assessment. - Your signature acknowledges that this is your own work. - If a participant initially answers a question incorrectly but modifies their response, they must follow the instructions below: - If a participant changes their response in writing, they must put their initials next to the written change. - If a participant changes their response verbally, this must be noted on the exam paper by the assessor.

| Participant name | Brett Young |
| --- | --- |
| Participant signature |  |

| Assessor name |  |
| --- | --- |
| Assessor signature |  |

| Date | Click here to enter a date. |
| --- | --- |

| Number of questions | Number of questions answered correctly | Satisfactory? |  |
| --- | --- | --- | --- |
| 24 |  | Yes | No |

Assessor Comments

What is the critical aspect of identifying and interpreting plans, drawings and specifications for a building project?

| Answer | It is critical to showing all parties involved in a building project the proposed structure and what it will look like when completed. |  |
| --- | --- | --- |

What is the purpose for date stamping of plans, drawings and specifications?

| Answer | Plans, drawings & specifications should be stamped once they are received and to clearly mark superseded and amended plans. |  |
| --- | --- | --- |

Nominate at least six (6) people and/or entities that would use plans drawings and specifications on a building project.

| Answer | - the client and the lending body. - local council (they must approve the application before the project can proceed) - various supply authorities, for example, water, sewerage and drainage - electrical and gas and cable supply companies - builders and sub-contractors wanting to quote on the project - the client and the lending body - engineers - carpenters - bricklayers - concreters - painters (dry wall & wet wall plasterers) - real estate agents - neighbours - kitchen manufacturers - landscaper |  |
| --- | --- | --- |

Plans and drawings usually hold many and varied features. 

Name some of the types of drawings and details you would expect to see on plans and drawings for a building project.

| Answer | Location plan: A location plan may be required if it is a big development so that the relative location of the site in a particular street is clear. It will show lot numbers, deposit plan numbers and a north point, using various scales but usually 1:5,000. Site or Block plan: A Site or Block Plan is usually drawn at 1:500 and gives the following information Floor plans: These are horizontal sections through the building viewed from above. They can be drawn at various scales, but usually 1:100 Elevations: An elevation is a view you see when standing in front of that wall. It may be multi-storey or single-storey, usually drawn to a scale of 1:100. This scale can vary. Sections: It is a vertical view (slice) through the building. The direction of section lines or arrows is usually shown on the plan to help interpret the sectional view. |  |
| --- | --- | --- |

It is necessary to acquire all building plans and specifications at the beginning of the negotiation period so we know all building project details prior to commencing work (including dimensions, construction materials, door and window schedules, masonry control joint locations and smoke alarm locations). 

What else would be expected to be provided?

| Answer | - Architectural plans and elevations including dimensions, construction materials, door and window details, masonry control joint locations and smoke alarm locations. - Building specification (materials and workmanship eg; standard HIA, MBA, BDA spec) and Schedule (if applicable). - Footing Construction Report and Foundation Soil Assessment. - Framing plans including wind speed assessment, bracing/tie down provisions, roof and wall framing members. - Roof truss computations (where applicable) and supporting documentation in accordance with “Guide to Safer Roof Framing” (available at www.sa.gov.au) including a layout indicating the truss types (numbered) all signed by trained software operator. - Energy Efficiency Report with construction details/supporting documentation (6 star Rating from NatHERS, First Rate or other accredited software - NOTE: 5 star only for Transportable Dwellings). - Other details as required such as if the building is in a Bushfire Zone or, if you are adding a swimming pool (we can advise you on what is needed) |  |
| --- | --- | --- |

Drawings are usually arranged in the approximate order of construction. What might a set of drawings consist of?

| Answer | Civil (C‐1, C‐2, etc): include plot or site plans, utilities, easements, grading and landscaping Structural (S‐1, S‐2, etc): include foundation, structural steel, building support system, roof framing Architectural (A‐1, A‐2, etc): include floor plans, elevations, building sections, door and window schedules and room finishes Electrical (E‐1, E‐2, etc): include electrical wiring, lighting plan, reflected ceiling plan, panel schedules Mechanical (M‐1, M‐2, etc): include heating, ventilating, sprinkler systems and air conditioning plans Plumbing (P‐1, P‐2, etc): include hot and cold water system layout and sewage disposal systems |  |
| --- | --- | --- |

Explain how to access the Building Code of Australia (BCA) and its relevance to estimating costs for building and construction projects.

| Answer | The BCA (or NCC) can be accessed from the Australian Building Codes Board website once logged in. [https://www.abcb.gov.au/Resources/Publications/NCC/NCC-2016-Complete-Series] The BCA can be used to ensure to help estimate building costs as it sets out requirements for how buildings are to be constructed. |  |
| --- | --- | --- |

Explain how to access the Plumbing Code of Australia (BCA) and its relevance to estimating costs for building and construction projects.

| Answer | The BCA (or NCC) can be accessed from the Australian Building Codes Board website once logged in. [https://www.abcb.gov.au/Resources/Publications/NCC/NCC-2016-Complete-Series] The BCA can be used to ensure to help estimate plumbing costs as it sets out requirements for how plumbing is to be installed. |  |
| --- | --- | --- |

Explain how to access Australian Standards and their relevance to estimating costs for building and construction projects.

| Answer | The BCA (or NCC) can be accessed from the Australian Building Codes Board website once logged in. [https://www.abcb.gov.au/Resources/Publications/NCC/NCC-2016-Complete-Series] Australian Standards can be used to ensure to help estimate construction costs as it sets out the minimum requirements for how a particular part or a installation is to be conducted. |  |
| --- | --- | --- |

Identify two (2) standards that would be relevant to the industry sector.

| Answer | AS/NZS 3000:2007 Electrical Installations (The wiring rules) AS1428 Balustrading |  |
| --- | --- | --- |

Explain the impact of WorkCover requirements on costs to the project.

| Answer | Business in Queensland who employ workers are required to insure them against workplace accidents with WorkCover Queensland. The rate paid to work cover is subject to industry rate and the amount of wages declared. The average premium rate is $1.20 per $100 of wages. This cost will need to be factored into the cost of the project, typically in the hourly rate of the employee. |  |
| --- | --- | --- |

Explain the impact of Environmental Protection Agency requirements on costs to the project.

Answer

A clear understanding of the environmental protection agency requirements is necessary when costing a project. Not including the cost of complying with environmental requirements up front can lead to cost overruns or profit loss in the future.

Calculate and cost the lineal timber order for 93 x 19 DAR Meranti and allow 5% wastage. The cost per lineal metre is $6.40. 

The requirements are: 22 @ 2400; 30 @ 3600; 10 @ 4800; 12 @ 1880 
Show your calculations.

| Answer | 22 x 2.4 (52.8) + 30 x 3.6 (108) + 10 x 4.8 (48) + 12 x 1.88 (22.56) = 261.36 261.36 + (261.36 * 0.05) = 261.36 + 13.068 = 274.428 274.428 * $6.40 = $1756.3392 |  |
| --- | --- | --- |

A building floor plan measures 24000 x 14000 and the specification states the sheet flooring is 3.6 x 0.9 x 19mm Yellow TERMIFLOR T&G H2 at $58.38 per sheet. 

Calculate the area of floor sheeting required when an allowance of 2% is made for waste and cutting. Determine the number of sheets to order to the nearest whole sheet and calculate the cost for the flooring. 

Show your calculations.

| Answer | Area of the room: 24 x 14 = 336m2 + Area * 2% = 342.72m2 Area of the flooring sheet: 3.6 x 0.9 = 3.24m2 Number of sheets: 342.72m2 / 3.24m2 = 105.78 Sheets Round up: 106 sheets Cost: 106 * $58.38 = $6,188.28 |  |
| --- | --- | --- |

If a solid cavity brick wall is to be built 23000mm long and 2400mm high (no windows and no doors), how many bricks would be required if the number of bricks per square metre is 50? Show your calculations.

| Answer | Area of the wall: 24 x 2.4 = 57.6m2 Number of bricks: 57.6m2 x 50 = 2,880 bricks |  |
| --- | --- | --- |

You are a QBCC licensed plasterer and a client (Owner Builder) Davey Jones-Locker of 100 Reef Avenue, Brisbane, Queensland wants you to quote for plastering an internal wall of his house build and has described to you the following information over the phone:

The Client tells you the Council approved plans and drawings state the finished ceiling height from the top of slab floor is 2400mm. There is one external solid timber door (D4) is 2340 x 870 x 35 in this wall and no other openings. The length of this house end wall is 11 metres.

The house is of brick veneer construction and includes a 50mm cavity and the load bearing external framing is 70 x 45 MGP12 (a Boral product “Pineguard Blue”). The bricks are standard bricks 230 x 76 x 100mm. (L x H x W).

The Client tells you that the door is positioned 2100mm from an internal corner (RHS in aerial view) and as seen from inside the house looking out. The specification and drawings state 10mm regular internal plasterboard. You know from experience regular plasterboard is supplied in the following sizes lengths 6M, 4.8M, 4.2M, 3.6M, 3M, 2.7M and 2.4M with recessed edge and widths of 1200mm and 1350mm.

The Client wants a quote to fix and finish this internal wall with plasterboard.

- Your labour and on-costs are charged at $51.50 an hour per tradesman plus materials including sheets, fixing, finishing and sanding. You estimate ancillaries including stud glue, screws, base coat, top and finish coats at $315.00 and that there is two (2) days’ work. 

Your business works a 38 hour week you allow workers a 15 minute morning tea break and half hour for lunch. You will need two (2) tradesmen first day and one (1) tradesman second day including materials and equipment supply, fixing, finishing and sanding. 

You are to research or acquire plasterboard sheet prices and attach this price list to your email. You are not required to supply or fix cornice.

Sketch the wall in Plan View (not to scale), dimension it with relevant measurements, and position the door on the right side of your plan view sketch and then calculate the plasterboard required and approximate waste expressed as a percentage (%) for internally lining this wall.

Estimate the cost of the job, including supplying plasterboard materials fixing setting, finishing and sanding plus labour. Then write a covering email to be sent to the client. You must show your methods for calculating cost of sheets, waste, plus fixing, setting, finishing and sanding. 

Attach your quote, sketch, calculations and pricing list and email this to your Trainer/Assessor (details provided by them).

DRAFT AREA

Area of the wall: 2.4 x 11 = 26.4m2

Area of the exterior door: 2.34 x 0.87 = 1.9656m2

11000mm

2400mm

840mm		2100mm

DRAFT AREA

| Answer |  |  |
| --- | --- | --- |

There are many different ways of estimating project costs. You can use your past experience, or ‘square metre’ rates but if you are unfamiliar with estimating and pricing then you should resort to the materials and labour method of costing using an item-by-item approach which is considered as the best industry approach to estimating. 

What are at least three of the nominated methods for estimating?

| Answer | - Costing by area. - Costing by material and labour. - Costing by a costing guide or estimating manual. |  |
| --- | --- | --- |

Name some of the circumstances to consider in relation to labour adjustment factors when estimating a project for construction.

| Answer | - Add 10% to 15% when working temperatures are below 4°c or above 35°c. - Add 15% to 25% for work on a ladder or a scaffold, in a crawl space, in a congested area or remote from the material storage point. - Deduct 10% when the work is in a large open area with excellent access and good light. - Include all productive labour normally. - Add 1% for each 10 feet that materials must be lifted above ground level. - Add 5% to 50% for tradesmen with below average skills. - Deduct 5% to 25% for highly motivated, highly skilled tradesmen. - Deduct 10% to 20% when an identical task is repeated many times for several days at the same site. - Add 30% to 50% on small jobs where fitting and matching of materials is required, adjacent surfaces have to be protected and the job site is occupied during construction. - Add 25% to 50% for work done following a major flood, fire, earthquake, hurricane or tornado while skilled tradesmen are not readily available. Material costs may also be higher after a major disaster. - Add 10% to 35% for demanding specs, rigid inspections, unreliable suppliers, a difficult owner or an inexperienced architect. |  |
| --- | --- | --- |

If a bricklayer lays 600 bricks in an eight (8) hour day, what is the average number of bricks laid per hour by the bricklayer? 
Tick your response.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  | ☐ | 65 |  |
|  | ☐ | 70 |  |
|  | ☐ | 55 |  |
|  | ☐ | 90 |  |
|  | ☒ | 75 |  |
|  | ☐ | 105 |  |
|  | ☐ | 85 |  |

What and where are some of the potential sources you could go to in choosing your suppliers for goods and services?

| Answer | Online is the best way to locate suppliers such as manufacturers and wholesalers. They may also advertise in the Yellow Pages. Attending industry events and exhibitions is another good way to find suppliers and examine their products and services. You can also use your networks to find suppliers. |  |
| --- | --- | --- |

What are some of the primary (key) factors for choosing your suppliers?

| Answer | Price - a key consideration for choosing suppliers is affordability. If you are focused on managing your finances, competitively priced suppliers are an attractive option. Reliability - Reliable suppliers deliver the right goods or services on time, as described. Stability - Stability is important, especially if you are entering into a long-term contract with a supplier or they are the only supplier of an item you need for your business. Location - Dealing with distant suppliers might mean longer delivery times and extra freight costs. If you need something quickly, a local supplier might be a better option. Supplier Groups - If your supplier goes out of business or cannot deliver, your business will suffer. It might be a better idea to use a carefully selected group of suppliers as insurance against any problems occurring. |  |
| --- | --- | --- |

What are some of the ways you could communicate with your suppliers for gaining quotes?

| Answer | To obtain quotes the following options are available: - written request - email request - faxed request - telephone request - verbal request |  |
| --- | --- | --- |

Explain the concept of a ‘Labour Only’ quote?

| Answer | Labour can be quoted as part of the cost of construction. In this type of quote, labour can be quoted in three different ways: - Per unit: installation of doors is usually quoted per unit - Per lineal metre: the cost of building walls to a specified height such as 2400 mm may be quoted per lineal metre of wall length - Per square metre: the cost of building wall may be quoted on the floor area in square metres |  |
| --- | --- | --- |

- You will be provided an allocated period by your instructor to complete this task. 
You may use appropriate costing software programs in completing this task.

Referring to the supplied set of house plans, you must do the following scheduling and costing:

- Identify and nominate a supplier/s and schedule the materials and cost the internal and external timber wall framing.

- Identify and nominate a supplier/s and schedule the materials and cost the foundations and lower floor concrete slab including the schedule and costing for reinforcement.

- Identify and nominate a supplier/s and schedule the materials and cost for the sub floor framing and the flooring for the upper floor plan.

- Calculate the architrave and skirting required for the fit out of the upper floor only in lineal metres.

- Calculate the wet area tiles for the upper floor only which are client chosen for both floors and walls with the wall tiles 250 x 100 and floor tiles 300 x 300.

- Estimate the labour only cost to tile all the wet areas only in the home. The tiler charges labour only $44.00 per square metre.

- Determine the roofing material and number of square metres of it that are required.

- Complete the aluminium windows and doors schedule and other external doors schedule for the lower floor only.

- Complete a full internal doors schedule for the lower floor only. 
N.B. Assume all doors are flush panel 2040mm in height and 35mm thick.

- Estimate the labour only cost for hanging all doors on the lower floor – if a tradesman takes 2 hours to jamb an opening, hang the door, fit the door furniture, door stops, architrave and fit a door-open-restraint. The hourly rate is $27.80 plus 20% on costs.

| Answer | - Identify and nominate a supplier/s and schedule the materials and cost the internal and external timber wall framing. Plantation grown seasoned timber to comply with AS1684.1999 Light timber framing code and AS1170 Loading code. - Identify and nominate a supplier/s and schedule the materials and cost the foundations and lower floor concrete slab including the schedule and costing for reinforcement. - Identify and nominate a supplier/s and schedule the materials and cost for the sub floor framing and the flooring for the upper floor plan. - Calculate the architrave and skirting required for the fit out of the upper floor only in lineal metres. - Calculate the wet area tiles for the upper floor only which are client chosen for both floors and walls with the wall tiles 250 x 100 and floor tiles 300 x 300. - Estimate the labour only cost to tile all the wet areas only in the home. The tiler charges labour only $44.00 per square metre. - Determine the roofing material and number of square metres of it that are required. - Complete the aluminium windows and doors schedule and other external doors schedule for the lower floor only. - Complete a full internal doors schedule for the lower floor only. N.B. Assume all doors are flush panel 2040mm in height and 35mm thick. - Estimate the labour only cost for hanging all doors on the lower floor – if a tradesman takes 2 hours to jamb an opening, hang the door, fit the door furniture, door stops, architrave and fit a door-open-restraint. The hourly rate is $27.80 plus 20% on costs. |  |
| --- | --- | --- |

| Answer |  |  |
| --- | --- | --- |