# Additional Project Details Identification Form

## Instructions to the Assessor

The candidate will review the construction drawings and specifications from Task 1. They will then use this Additional Project Details Identification Form to document additional details and requirements of the construction of the building.

## Instructions to the Candidate

Use this Additional Project Details Identification Form to document additional details and requirements of the construction of the building.

Ensure you have completed all fields in this form before submitting it to your assessor.

| Additional Project Details Identification Form |  |
| --- | --- |
| Candidate name | Noreen Abu-Obeid |
| Workplace/organisation | Knights Construction |
| Date of inspection | 19/12/2022 |

Details of the Building and Construction Project

| Workplace/organisation | Knights Construction |
| --- | --- |
| Name of building and construction project | Sandgate Road, Nundah Project |
| Type of project | X Residential building Commercial building |
| State/territory | QLD |

Section 1 – Building Details

| Type of Building | Residential – class 1a – single dwelling that is a detached house |
| --- | --- |
| Method of Construction | Residential |

Building Site Features (Add rows as necessary)

Boundary of the site as outlined and site area 538sqm and will have retaining wall at the right side of the property, about half of the back boundary and around the pool.

Site is surrounded by neighbour houses of which two are existing highest houses and two are existing lowest houses

House adjacent to main road and subject to Transport Noise

P Power pole on left side of property next to main road

Contours ranging from 29.75 to mostly 30 along the driveway and 30.50 from front left side of the proposed site gradually sloping towards the back lefty side to 29.75, therefore being a slight sloping land.

E Entrance to the house is West facing

Building Materials to be Used (Add rows as necessary)

Timber

Bricks

Steel beams

Timber posts – 90 x 90

St Steel posts 75 x 75 x 3.2

Truss

Tiles - floor

Tiles - wall

Tiles - splashback

Colorbond roof sheeting

Aluminium balustrade

Cladding

Plasterboard

Support posts

Floor sheet

Ply brace sheet 600mm Direction X kN3.8

Ply brace sheets 700mm Direction X kN 4.5

Ply brace sheets 900mm Direction X kN 27

Ply brace sheets 2700mm Direction X kN 48.6

Ply brace sheet 600mm Direction Y kN 3.8

Ply brace sheets 900mm Direction Y kN 27

Ply brace sheets 1800mm Direction Y kN 43.2

Ply brace sheets 900mm Direction X kN 32.4

Ply brace sheets 700mm Direction Y kN 4.5

Ply brace sheets 900mm Direction Y kN 21.6

Ply brace sheets 1800mm Direction X kN 10.8

Filler block

PFC board 250

SHS Post 75 x 75

Floor beam

Downpipes

Gutters

Wall ties

Foam backing Rod 10mm

Stud frame

Architrave

Masonry expansion tie

Articulation joints

Edge beam

Internal beam

Common beam

Patio beam

Widened rib

Strip footing

Fill Piers 450 dia

Tree Piers 450 dia

Service Piers 450 dia

SL92 Mesh 20 top cover 300 pods

SL92 Mesh 40 top cover 600 pods

2-N12 ‘Z’ bars

Underslab bedding

Trench mesh

N12 corner bars – 600 legs

Steel roof battens

Top plates 2/35 x 70 MGP12

Colourbond roof sheeting

Cyclone strap

Trusses

Gyprock ceiling

Gyprock wall lining

Sarking

Metal fascia and gutter

Cornice

FC sheets

Timber beading 42 x 11

Cladding

Breather foil

DPC

Brick ties

R1.5 Batts

Skirting

Particle board flooring

Floor joists

Studs (load bearing) 70 x 35 MGP12

Studs (Non loadbearing) 70 x 35 MGP10

Bottom plate on slab 35 x 70 MGP12

Bottom plate on floor joists 2/35 x 70 MGP12

Noggings

Lintel

1/10 55mm Roof Zip Batten screw

2/10 55mm Roof Zip batten screws

1/10mm powers screw bold

900mm plywood sheet

Villaboard TB3A

Steel angle brace

4.5mm F11 or 7mm F8 structural ply

Downpipes

ConcreteSL72 welded mesh

Turf

Swivel joints

PVC joist jackets

6 mm backing rod

Fibre reinforced waterproof membrane

10 x 20 aluminium angle

Nails 30 x 3.14 diameter flat head

Nailing plates

30mm x 2.8mm dia nails

30mm x 0.8mm tensioned galvanized metal strap

Galvanized flat head nails

Screw bolts

Backfill

Termite barrier

Garage door

M12 bolts

Vanity

Mirror

Bathtub 1500

Bathtub 1700

Undermount kitchen sink

900 canopy rangehood

Upright cooker

Hanging rail

Glass for balustrade

AC duct shaft

Window

Doors

flyscreen

Section 2 – Site Requirements

Site Facility Requirements (Add rows as necessary)

Portable Toilets provided

Drinking water provided

Safe eating area specified on site

Job site access and security

Temporary power

Communication Requirements (Add rows as necessary)

Staff and workers training and communication

Communication tools eg) mobile phones used within the team(s)

Communication to specific and use of suitably qualified tradespeople

Communication with stakeholders – clients, architects, engineers, project personnel on hazards and risks associated with the building work and its solutions

Prepare SWMS and Safety report and share with the team

Temporary Boundary Fencing Requirements (Add rows as necessary)

All boundary clearance and cut out dimensions to be verified prior to commencement of work

Site sign to be installed on the site boundary

Fencing the site off with temporary fencing and allow access to only authorized personnel

Do not place or stack materials close to fence lines or fences

Temporary fencing around excavation areas to prevent unauthorized personnel from entering

Pool safety fencing

Workplace Safety Requirements (Add rows as necessary)

Safe material handling and lifting

Handling hazardous materi8al

Compliance of materials to match specs

Reducing trip, slip and fall hazards

Safe use of equipment to comply with procedures

Safe and suitable equipment

Formwork erection and use

Scaffolding erection and its application

Temporary bracing installation procedures

Excavations – site prep, clearing, levelling before construction; soil testing ground design before construction

Excavations outside property boundaries

Excavation near services

Ensure construction loads not exceeded

Pool safety barrier

Section 3 – Waste Removal Requirements

Waste Removal Requirements (Add rows as necessary)

Rubbish removal – loading trucks, waste bins and rubbish skips with demolition waste

| Waste Removal Procedures (Must include two types of waste and steps on how to remove each type of waste from the building site) |  |
| --- | --- |
| Type of Waste (Add rows as necessary) | Excavation waste |
| Ensure you identify waste areas for collection, movement, storage and disposal of the waste for safety purposes and all understand this |  |
| Progressively remove the waste to prevent build up and ensure ground conditions are maintained as required |  |
| Entry and exit points to be clean and clear of demolition waste and not stacked near the excavation site or entry/exit points |  |
| Ensure vehicle and trucks and material loads are accessing with safety measures in place for the people and vehicle movements and material loading/offloading. |  |
| Always ensure the underground services are not affected |  |
| Excavated material should be placed in such a manner that it channels rainwater and any other run-off water away from the excavation site. |  |
| Type of Waste (Add rows as necessary) | Construction material waste |
| Identify waste bins , rubbish or recycling bins and dedicated waste placement points |  |
| Ensure you identify waste areas for collection, movement, storage and disposal of the waste for safety purposes and all understand this |  |
| Progressively remove the waste to prevent build up |  |
| Entry and exit points to be clean and clear of materials and waste |  |
| Ensure construction materials not allowed to fall freely unless in a confined space eg) chute |  |
| Ensure the bins are emptied and replaced as required |  |

| Waste Management Site Fees (Must include types of fees and the cost or the calculation for the cost for each) |  |
| --- | --- |
| Type of Fee | Cost |
| Construction waste – skip bins | $690 per bin |
| Construction waste - excavation | $80 per tonne |
| Clean fill | free |

Section 4 – Additional Costs

| Project-Specific Statutory Costs (Must include types of fees and the cost or the calculation for the cost for each) |  |
| --- | --- |
| Type of Cost | Cost |
| Residential Contract | $380 |
| Home Warranty insurance | $5142 |
| Qleave and QBCC Insurance | $2840 |
| Engineering fees | $1700 |

| Approval Costs (Must include types of fees and the cost or the calculation for the cost for each) |  |
| --- | --- |
| Type of Cost | Cost |
| Driveway permit | $320 |
| BA and DA and Zoning/Building permits | $5210 |
| Surveyors fees | $8580 |
| Architectural, drawing and specifications | $15900 |
| Pool approval | $500 |
| Administrative costs | $170 |

| Compliance Costs (Must include types of fees and the cost or the calculation for the cost for each) |  |
| --- | --- |
| Type of Cost | Cost |
| Stage inspections comprising of: - excavation – after excavation of foundation material and before the concrete for the footings, or slab, for the building are poured - footings – if the building is to have footings—after the placement of formwork and reinforcement for the footings but before the concrete for the footings is poured - slab – if the building is to have a slab—after the placement of formwork and reinforcement for the slab but before the concrete for the slab is poured - frame – inspection of the frame (note the breakdown of the framing stage under section 44 of the Building Regulation 2021) - final – inspection of any outstanding items and the collection of certificates, such as termite protection, wet area membrane installation, glazing, smoke alarms and energy efficiency, drainage, termite management systems, weatherproofing, fire safety, glazing etc | $2500 |
| Pool inspection as under QDC MP 3.4 | $275 |
| Sediment control for EPA compliance | $545 |
| Noise/acoustic assessment by qualified Engineer | $4800 |
| Traffic control for safe work compliance | $6800 |

END OF ADDITIONAL PROJECT DETAILS IDENTIFICATION FORM