THEORY EXAM INSTRUCTIONS - This exam will occur under standard assessment conditions. - An oral examination may be allowed under certain circumstances. Answers will be documented and FT003 (Oral examination declaration form) must be completed. - Participants must provide enough detail in their responses to demonstrate their knowledge and skills. - Please fill in the details below, providing your name, signature and date of assessment. - Your signature acknowledges that this is your own work. - If a participant initially answers a question incorrectly but modifies their response, they must follow the instructions below: - If a participant changes their response in writing, they must put their initials next to the written change. - If a participant changes their response verbally, this must be noted on the exam paper by the assessor.

| Participant name |  |
| --- | --- |
| Participant signature |

| Assessor name |  |
| --- | --- |
| Assessor signature |

Date

| Number of questions | Number of questions answered correctly | Satisfactory? |  |
| --- | --- | --- | --- |
| 45 |  | Yes | No |
| Assessor Comments |  |  |  |  |

- Compliance documentation and reporting:

- Give one (1) example of each of the following types of compliance documentation relating to earthworks construction:

- Legislation.

- Organisational requirements.

- Client requirements.

- Site operational requirements.

- What are two (2) government agencies or statutory authorities you may have to report to?

| Answer | - Legislation - Australian Standards, e.g. AS 3798 – 2007 guidelines on - earthworks for commercial and residential developments. Organisational requirements - Workplace relations. Client requirements - Planned outcomes of the earthworks construction project Site operational requirements - Operational procedures, e.g. work instructions, safe operating procedures (SOPs) and other site and equipment safety requirements |
| --- | --- |

- Administrative documentation and project quality management:

- List three (3) types of administrative documentation that you may have to complete.

- What are three (3) factors of project quality management?

| Answer | - daily activities - ordering and reordering - workplace incedent and accident management - Dimensions and tolerances of tasks. - Material standards. - Work standards. |
| --- | --- |

- What is the aim of OHS/WHS legislation?

| Answer | responsibility to ensure that the PCBU is doing everything it should to ensure health and safety. |
| --- | --- |

- List three (3) requirements that may be covered by the environmental management plan for your worksite?

| Answer | Dust monitoring and dust suppression measures to be put in place, i.e. water carts. Stability or structural reports on buildings in the area to monitor vibration results. Noise management requiring hearing protection, or strict work times, e.g. no work before 7am or after 6pm. |
| --- | --- |

- What are the three (3) plans that outline the requirements and procedures for controlling traffic and people around and through the work zone?

| Answer | Traffic Management Plan Vehicle Management Plan Site Isolation Plan |
| --- | --- |

- Why is it important to consult and communicate with others during the risk management process?

| Answer | This will help ensure that risks and hazards are not only effectively identified but that those involved with controlling and treating them are clear of their role and responsibilities in the risk management process. |
| --- | --- |

- What should you do if you don’t understand the meaning of a particular term, phrase or word used on your worksite?

| Answer | clarify with a content expert, ie project manager, engineer etc. |
| --- | --- |

- Geotechnical, geological, survey and engineering data:

- List three (3) ways that geotechnical data is used in earthworks.

- List three (3) uses of soil or geological data.

- What is survey and engineering information used for?

| Answer | - Identify soil and rock types and characteristics. - Check conditions and materials on and under the - ground surface. - Determine their physical/mechanical and chemical - properties. - Defining material types. - Identifying wet and dry - areas. - Identifying broken - ground, faults and joints. - Floor heights. - Bench heights and widths. - Floor and bench grades. |
| --- | --- |

- Hydrological and meteorological data:

- Why is hydrological data important?

- List three (3) factors that may be included in meteorological data.

| Answer | - Hydrological data helps to determine the properties, - distribution and effects of water, moisture content and - exposures on the project/site soil, rocks and atmosphere. - rainfall - humidity - temperature |
| --- | --- |

- Project plans and specifications:

- What are three (3) things provided in project plans and drawings?

- What is the purpose of project and task specifications?

| Answer | - Location of the site and earthworks in relation to the - surrounding area. - The position of structures, roads, access areas. - Layout of drainage lines. - to detail how activities will be completed |
| --- | --- |

- Drainage and dewatering:

- List two (2) examples of drainage and dewatering requirements.

- Why must any drainage and dewatering issues be managed promptly and effectively once you have identified them?

| Answer | - Any form of erosion or sediment controls. - Pumping out of sumps or pits. - so that no dirty or - contaminated water or pollution can leave the site. |
| --- | --- |

- Why is it important that earthworks construction activities are scheduled within a job plan?

| Answer | to make the most efficient use of the resources on site and meet all task requirements. |
| --- | --- |

- Soil density and testing:

- Why do you need to test the relationship between soil density and moisture when scheduling activities?

- What does ‘plasticity index’ mean?

- Why is it important for materials to have the right balance of particle sizes?

| Answer | - To gain adequate compaction - The plasticity index (PI) determines how the materials can be - shaped and will relate to the clay content within the material. - to ensure the correct construction of layers |
| --- | --- |

- Earthwork calculations:

- How do you calculate the following in earthworks construction:

- Grade or slope.

- Volume.

- Area.

- What is percentage or cross fall?

- How do you work out the rates of resource consumption?

| Answer | - rise/run - LxWxD - WxL |
| --- | --- |

- Sequencing of civil works task:

- How does an understanding of the relationship between the different areas of civil works help in planning work?

- What is the sequence of tasks for drainage works?

| Answer | so that things that need to be completed prior to the commencement of other activities can be done. - 1. Forming drain lines. - 2. Forming out the work area, that is building the frame to - hold the drain system. - 3. Laying pipes, box drains or culverts. - 4. Backfilling. - 5. Landscaping as appropriate. |
| --- | --- |

- Job planning and administration:

- List four (4) requirements that should be included when drafting a job plan.

- What are three (3) examples of administration procedures and requirements?

| Answer | - Construction materials, including quality and delivery. - Occupational Health & Safety/Work Health & Safety (OHS/WHS). - Sub-contractor support. Communication and coordination. - Activity scheduling. Material delivery. - Risk management. - Staff requirements such as leave timesheets, wages or - other documentation. - Reporting achieved targets or productivity. - Procurement. |
| --- | --- |

- Interpret requirements and provide estimates:

- Why is it important to correctly interpret the requirements of the project contract?

- What are two (2) factors you may have to provide estimates for, and how do you go about it?

| Answer | - to ensure that all earthworks plans, client requirements and quality standards are known, understood and achieved. cost of completing a task - add the cost of production to the cost of materials. completion time of a job work out how long each individual activity should take from start to finish. You then determine how many activities need to be completed. The total of these two figures should give an estimate of the total hours. |
| --- | --- |

- List four (4) types of resources that will need to be available and ready for earthworks construction tasks.

| Answer | labour plant equipment and tools material supply vehicles construction materials |
| --- | --- |

- How can you be sure that you have followed the relevant resource requirements and procedures?

| Answer | by checking the project and task documentation. |
| --- | --- |

- How do you confirm that plant and equipment is suitable for the task?

| Answer | refer to the machine specifications and other details listed in the individual operator’s manuals or your site documentation. |
| --- | --- |

- Provide instructions and supervise team members:

- List three (3) ways you can provide clear and timely instructions to the work team.

- List three (3) team members and others who may be involved in the earthworks activities you are supervising.

| Answer | - Two-way radios. - Mobile phones. - Landline phones - Personnel/Workers directly involved in the task. - Other members of the organisation's management - team. - Supplier’s representatives |
| --- | --- |

- List two (2) factors that may be detailed in your worksite communication requirements and procedures?

| Answer | Documentation requirements, e.g. verbal or written reports. Protocols for using communications equipment. |
| --- | --- |

- Prepare messages and instructions:

- Why is it important to prepare for and conduct briefings, toolbox and site meetings?

- What should you keep in mind when preparing and delivering short messages or instructions?

| Answer | so that information cab be provided both before the work begins and during the task or operations. - The level of information that is given out will be suitable to work operations and the people involved |
| --- | --- |

- Why is it important for a leader/supervisor to develop and maintain workplace relationships?

| Answer | to keep communication channels open to encourage productivity and information sharing. |
| --- | --- |

- What factors will determine the leadership style and techniques you adopt?

| Answer | Your style of leadership will depend on your team members, organisational requirements, work procedures, and the type of earthworks construction being undertaken |
| --- | --- |

- Supervisor techniques:

- What are four (4) things you need to be able to do as a supervisor?

- List the five (5) techniques commonly used in mentoring.

| Answer | Communicate. Organise. Plan. Manage people and resources. - accompanying - sowing - catalysing - showing - harvesting |
| --- | --- |

- What do set out requirements and procedures include?

| Answer | Establishing Construction Offsets Establishing Cleared Width Establishing Control Lines Determining Batters |
| --- | --- |

- What are construction offsets and how are they established?

| Answer | Offsets refer to how far from a control line an activity is to take place or an area is to be cleared. A negative number means the offset is to the left of the line, a positive number means the offset is to the right of the line. In establishing construction offsets, quite often survey markers and level pegs are used to define the work area. In many cases these pegs or markers are placed at an offset value to where the works are being conducted to prevent them from being hit or damaged during the work processes. |
| --- | --- |

- List three (3) factors of road geometry.

| Answer | grade or slope crests structures |
| --- | --- |

- What are levelling devices used for in the setting out process?

| Answer | Determining drainage grades. Determining gradients, batters and benches. Measuring offsets. Helping with road geometry. |
| --- | --- |

- What are the four (4) aspects of project and task monitoring that you must carry out?

| Answer | Ongoing risk assessment and management. Engineering survey. Sampling and testing. General supervision, observation and recording. |
| --- | --- |

- Engineering principles and interpret test results:

- Why should earthworks operators be aware of the engineering survey principles for the job?

- Why do you need to interpret material properties and compaction test results?

| Answer | - so they understand the required tolerances and - standards. - The testing results will indicate material variances throughout the works. Test results are also used to ensure levels of compaction are achieved based on specifications |
| --- | --- |

- Monitor earthworks tasks:

- List four (4) criteria that may relate to performance of the tasks you are monitoring.

- What are three (3) methods you might use for monitoring earthworks tasks?

| Answer | - Compaction. - Layer thickness. - Particle size. - Clay content. - Inspection and test plans. - Site inspections that may be daily, weekly, or based on a - particular work area. - Survey and level checks. |
| --- | --- |

- Why should routine inspections of earthworks operations be carried out?

| Answer | to ensure compliance with Occupational Health & Safety/Work Health & Safety (OHS/WHS) legislation, contractual obligations and requirements specific to the project and the worksite. |
| --- | --- |

- List four (4) methods and operational techniques involved in earthworks construction practice.

| Answer | Site preparation. Extraction. Load and haulage. Placement. |
| --- | --- |

- How will you find out the operational techniques for the earthworks job you are working on?

| Answer | it will be outlined in the relevant procedures for the work activity |
| --- | --- |

- In ground surface treatment, what is proof rolling?

| Answer | A plant item such as a roller, loaded truck, or other nominated item may be used to see if there is any movement in the ground underneath. Movement under the wheels could indicate unsuitable or insufficiently compacted material in lower layers. In some cases further testing such as deflection monitoring may need to be carried out or unsuitable material removed before continuing. |
| --- | --- |

- Give two (2) examples of how you should inform operators and other personnel/workers in writing and verbally that changes to the job plan or relevant earthworks construction practice have been initiated?

| Answer | Written communication – plans, sketches and drawings, work instructions or other documentation. Oral communication – briefings, toolbox meetings, verbal instructions, discussions with operators, team meetings, or relevant conversations based on the activity. |
| --- | --- |

- What are four (4) maintenance activities that may be carried out by earthworks construction operators?

| Answer | Servicing and monitoring, e.g. changing filters, topping up oils and hydraulic fluids. Cleaning and checking for damage before and after use. Fitting and removing attachments. Minor repairs, e.g. replacing ripping teeth or boots |
| --- | --- |

- How can you ensure that your team carries out maintenance activities in line with site procedures and the manufacturer’s specifications?

| Answer | Logs, logbooks and diaries – used to track daily and weekly activities. Logbooks are often used to verify the number of hours an operator has worked on a machine, training that has been completed or other task-related information. Job reports. Completion reports. |
| --- | --- |

- List four (4) reasons why you should follow organisational and/or site requirements and procedures for preparing and presenting reports.

| Answer | How and when reports are to be completed. Who completes the reports. What supporting documentation is required for each report. How the report is presented and submitted. |
| --- | --- |

- What are three (3) guidelines you should keep in mind when preparing reports?

| Answer | Objectives of the report. Overview of the situation. Research and information collection methodology used and why this methodology was chosen. |
| --- | --- |

- What are logbooks and diaries used for?

| Answer | Logs, logbooks and diaries – used to track daily and weekly activities. Logbooks are often used to verify the number of hours an operator has worked on a machine, training that has been completed or other task-related information. |
| --- | --- |

- List four (4) things that you need to do when recommending changes or improvements to the safety, effectiveness or efficiency of tasks.

| Answer | Summarise the situation. Clarify why you are providing advice. Outline all the considerations. Explain the risks or hazards. |
| --- | --- |

- How do you develop individual workers or the team as a whole?

| Answer | Team building exercises |
| --- | --- |