BSBWHS431

Develop processes and procedures for controlling hazardous chemicals in the workplace

ASSESSMENT SUMMARY

The assessment activities for BSBWHS431 Develop processes and procedures for controlling hazardous chemicals in the workplace cover all aspects of the unit of competency. For Apprentices/Trainees, this includes supervisor completion of the Apprentice/Trainee Record (ATR).

Participants must read all the information given before starting any assessment task. The assessment tasks are intended to be fair, flexible, valid and reliable. If a Participant has any questions or requires alternative arrangements to be made such requests should be directed to their Trainer/Assessor, who should note such arrangements in the ‘Assessor Comments’ on the affected assessment. If a Participant feels they are not yet ready to be assessed, they should discuss their options with their Trainer/Assessor.

To satisfactorily complete the assessment, the Participant is required to answer all questions correctly and demonstrate their skills to industry standards. If a Participant does not answer all questions or perform all tasks, they will be deemed ‘Not Satisfactory’ for the task and ‘Not Yet Competent’ for the unit of competency.

The Trainer/Assessor can provide the Participant with additional information on interpreting the assessment outcomes and guide the Participant’s options. If a Participant is still deemed ‘Not Satisfactory’ they will have the opportunity to undertake a supplementary assessment or appeal the result as per the process in the Student Handbook.

By signing below, the Participant acknowledges:

- They understand the assessment instructions and requirements, and consent to being assessed.

- The work submitted is to be their own work – with the work of/with others needing to be clearly acknowledged and documented.

- They consent to any photo/video recording of their work that occurs during assessment.

| Participant name | Elise Painter |
| --- | --- |
| Participant signature |  |

Pre-requisite unit(s) to be achieved and authenticated prior to undertaking assessment

Not applicable

| Task | Date completed | Assessment Results |  |
| --- | --- | --- | --- |
| Workbook | Select Date. | Satisfactory | Not Satisfactory |
| Quiz | Select Date. | Satisfactory | Not Satisfactory |
| Practical Assessment | Select Date. | Satisfactory | Not Satisfactory |
| Outcome for BSBWHS431 | Select Date. | Competent | Not Yet Competent |

| Evidence requirements checklist (ticked by assessor and evidence supplied to RTO) |  |
| --- | --- |
| Quiz Workbook Practical Assessment | 2 x Processes and procedures for control hazardous chemical 2 x Email communications with PCBU 2 x Email communications with workers 2 x Presentation materials |

| Assessor name |  |
| --- | --- |
| Assessor signature |  |

BSBWHS431

Develop processes and procedures for controlling hazardous chemicals in the workplace

WORKBOOK

| Participant name | Elise Painter |
| --- | --- |

FOR THE PARTICIPANT

Instructions

- You will complete written activities as outlined on the following pages. You must attempt all questions and provide enough detail in your responses to demonstrate your knowledge. - The timeframe for these tasks will be established by your Assessor or Training Plan, but additional time may be allowed upon negotiation with the Assessor. - You must complete all questions unassisted by the Assessor or other personnel but may refer to reference material as needed. All questions must be answered correctly for a ‘Satisfactory’ assessment outcome to be awarded. - An oral examination may be allowed under certain circumstances, whereby your answers will be documented by another party. - Any written questions that are incomplete, or not answered satisfactorily will need to be completed again after further training support. - If you answer a question incorrectly but modify your response, put your initials next to the written change (or the Assessor must initial a verbal change).

Resources required

Pen

FOR THE ASSESSOR

Instructions

- Where oral examination has occurred, ensure that this is acknowledged in writing (e.g. FT001 and Assessor Comments) - All questions must be satisfactorily demonstrated by the Participant for a Satisfactory determination (indicated by an Assessor tick to the right of the question). - Assessors are to complete marking in pen of a different colour to the Participant. - If the Participant verbally modifies their response, write in the modified response verbatim and put your initials next to the written change.

ASSESSOR COMMENTS

As necessary, outline: - Details on reattempts, alternative arrangements or reasonable adjustments made. - Where a Participant is deemed ‘Not Satisfactory’, provide recommendations for future training.

NOTE

At times, you will be provided with a specific business context to which you must respond, otherwise it is anticipated that you will use a workplace of your choosing (your own, or hypothetical) to provide the context to the task.

At all times, the responses in your assessment must reference Australian (Queensland or Federal) documentation, including legislation, regulations, codes of practice, standards and guidance publications. For example, in Queensland refer to https://www.worksafe.qld.gov.au/

For the following activities, name a workplace that you wish to examine.

| Name of workplace | Kent Construction |
| --- | --- |

Identify the duty holders and their range of duties regarding the control of hazardous chemicals by explaining the legislative requirements for the topics listed below: - Labelling hazardous chemicals - Obtaining and giving access to safety data sheets - Hazardous chemicals register - Management of risk - Safety signs - Spills and damage - Emergency and safety equipment - Storage and handling systems - Health monitoring and records - Induction, information, training and supervision

Labelling hazardous chemicals

Duty Holders: Kent Construction (as the PCBU): Must ensure that all hazardous chemicals used on-site are correctly labelled. Duties: All containers of hazardous chemicals must have compliant GHS (Globally Harmonized System) labels. Decanted chemicals must also be labelled unless used immediately. Workers must be instructed not to use unlabelled chemicals.

Obtaining and giving access to safety data sheets

Duty Holders: Kent Construction: Must ensure that an up-to-date SDS is available for all hazardous chemicals on site. Duties: Obtain SDSs from suppliers or manufacturers for all hazardous chemicals purchased. Provide easy access to the SDS for workers and contractors, either electronically or in hard copy. Train workers on how to locate and understand SDSs.

Hazardous chemicals register

Duty Holders: Kent Construction: Responsible for maintaining a hazardous chemicals register at each construction site. Duties: The register must list all hazardous chemicals present, along with their SDS. It must be updated as chemicals are added or removed. The register must be accessible to workers and safety representatives.

Management of risk

Duty Holders: Kent Construction: Has the primary responsibility for managing risks associated with hazardous chemicals. Duties: Identify chemical hazards and assess risks specific to construction activities (e.g., solvents, adhesives, fuels). Implement controls using the hierarchy of controls: eliminate, substitute, engineer, administrate, and provide PPE. Regularly review and revise control measures based on site conditions and incidents.

Safety signs

Duty Holders: Kent Construction: Must ensure safety signage is present where hazardous chemicals are stored, used, or handled. Duties: Display signs warning of hazards (e.g., "Flammable Materials"). Signs must comply with Australian Standard AS 1319 and be clearly visible to workers and site visitors. Include signs indicating PPE requirements.

Spills and damage

Duty Holders: Kent Construction: Must have procedures and equipment for managing chemical spills and damage. Duties: Maintain spill kits appropriate for the chemicals on site (e.g., absorbents for oils or solvents). Train workers in spill response procedures. Inspect chemical containers regularly for leaks or damage and replace as needed.

Emergency and safety equipment

Duty Holders: Kent Construction: Must provide and maintain emergency equipment for hazardous chemical incidents. Duties: Install and maintain eyewash stations and safety showers near areas where chemicals are handled. Provide fire extinguishers suitable for chemical fires. Ensure emergency plans include chemical incident response procedures and that workers are trained in these procedures.

Storage and handling systems

Duty Holders: Kent Construction: Must ensure hazardous chemicals are stored and handled safely on-site. Duties: Store chemicals in compliance with SDS instructions and legislative requirements. Segregate incompatible chemicals (e.g., acids and bases) and secure storage areas from unauthorized access. Ensure proper ventilation in storage areas to minimize exposure risks.

Health monitoring and records

Duty Holders: Kent Construction: Must provide health monitoring for workers exposed to specific hazardous chemicals. Duties: Conduct health monitoring for chemicals like silica dust or lead, which are common in construction. Maintain records for 30 years and provide them to workers upon request. Act on health monitoring results to reduce exposure risks if adverse findings are identified.

Induction, information, training and supervision

Duty Holders: Kent Construction: Must provide appropriate induction, training, and supervision to workers. Duties: Include hazardous chemical safety in site inductions, covering risks, controls, and emergency response procedures. Provide ongoing training for workers handling chemicals (e.g., proper use of PPE). Supervise workers to ensure compliance with safe work practices.

| Satisfactory Outcome |  |  |
| --- | --- | --- |

Explain the requirements of your organisation regarding the control of hazardous chemicals by identifying the requirements found in the WHSMS (i.e. policies, procedures and processes) for: - Control of hazardous chemicals (including accessing internal and external sources of WHS information and data) - Recording outcomes of monitoring and distributing to required personnel

Control of hazardous chemicals

WHS policy

WHS Policy Hazardous Chemicals Policy: Kent Construction commits to ensuring the safe storage, handling, use, and disposal of hazardous chemicals. Ensures compliance with WHS laws, Codes of Practice (e.g., "Managing Risks of Hazardous Chemicals"), and relevant Australian Standards. Requires the maintenance of a hazardous chemicals register and access to Safety Data Sheets (SDS). Information Access Policy: Mandates that up-to-date SDSs for all hazardous chemicals are obtained from suppliers or manufacturers and made accessible to all workers. Promotes internal access to WHS documentation and external resources like Safe Work Australia’s guidelines.

WHS procedure

Hazard Identification and Risk Assessment: Procedures for identifying hazardous chemicals in the workplace, assessing associated risks, and implementing control measures. Requires regular workplace inspections to identify improperly stored or unlabelled chemicals. Labeling and SDS Management: All chemicals must be labelled in accordance with the Globally Harmonized System (GHS) standards. Procedures for obtaining, reviewing, and storing SDSs (e.g., ensuring all SDSs are updated within five years). Emergency Management: Include hazardous chemical incidents in emergency response plans, specifying procedures for spills, leaks, and exposures. Procedures for training workers in spill response, using spill kits, and reporting incidents.

WHS process

Hazardous Chemicals Register: Maintain an updated register listing all hazardous chemicals used or stored on site. Ensure the register includes the SDS for each chemical and make it accessible to workers. Control Implementation: Apply the hierarchy of control (elimination, substitution, engineering controls, administrative controls, and PPE). Conduct regular reviews and audits of the controls to ensure effectiveness. Access to Information: Maintain both physical and electronic copies of SDSs in designated areas. Establish a system for workers to request and receive updates on hazardous chemical risks.

Recording outcomes of monitoring and distributing to required personnel

WHS policy

WHS Policy Health Monitoring Policy: Requires health monitoring for workers exposed to hazardous chemicals, such as silica, lead, or asbestos. Commits to maintaining confidentiality of health records while sharing necessary outcomes with workers and supervisors. Mandates the secure storage of health records for a minimum of 30 years, as required by WHS regulations.

WHS procedure

WHS Procedure Health Monitoring Procedure: Outlines when health monitoring is required (e.g., regular exposure to hazardous substances). Details the types of monitoring performed, such as air quality testing, blood tests, or lung function assessments. Specifies the process for notifying workers of results, especially when further action is required. Distribution of Results: Procedures for informing affected workers, supervisors, and managers about monitoring outcomes while protecting individual privacy. Defines reporting obligations to regulatory authorities, where required.

WHS process

WHS Process Monitoring and Record-Keeping: Use qualified personnel to conduct workplace exposure assessments and health monitoring. Document results in a centralized record system. Distribution of Results: Provide individual workers with their health monitoring results along with guidance on necessary follow-up actions. Share anonymized monitoring data with relevant site supervisors and managers to inform risk control measures. Incorporate findings into WHS reviews, revising controls as needed.

| Satisfactory Outcome |  |  |
| --- | --- | --- |

Identify one (1) hazardous chemical that may be used in your workplace and respond to the following points. Identify the source of your information for each response (e.g. SDS). - The range of harms caused by the chemical and how they are caused - Identify exposure standards, environmental requirements, and storage specifications - Identify one (1) specialist you may seek advice from about emerging issues and potential risks from exposure to the chemical - Workplace tasks involved with the chemical - The level of risk for the chemical within your workplace - Describe a safe system of work (including risk controls) that may be implemented to address the hazardous chemical and risk assessment results - Consider the hierarchy of control and any legal requirements - Identify who you would seek feedback from when developing processes and procedures - How you advise others of safe systems of work - Identify the individuals and parties who should be notified (i.e. because they are at risk or are required to implement controls) - Describe the process you would undertake to evaluate the implementation of the safe system of work and identify non-compliances - Explain how findings should be recorded - Identify who should be notified of the outcomes of monitoring

Name of chemical

Solvent-Based Paints

The range of harms caused and how they are caused

Source of Information: SDS from the manufacturer or supplier. Range of Harms: Acute Effects: Skin and eye irritation, dizziness, headaches, nausea from inhalation of vapors. Chronic Effects: Prolonged exposure may lead to respiratory issues, liver or kidney damage, and potential central nervous system effects. Environmental Harms: Can contaminate soil and water if spilled or improperly disposed of. How Harms Are Caused: Inhalation: Fumes from volatile organic compounds (VOCs) present in the solvent evaporate and can be inhaled. Skin Contact: Direct contact may cause irritation or dermatitis. Ingestion: Accidental ingestion can cause severe internal damage. Exposure Standards: As per Safe Work Australia, exposure standards for VOCs such as toluene (a common solvent) are 50 ppm (188 mg/m³) TWA and 150 ppm (560 mg/m³) STEL. Environmental Requirements: Store and dispose of in accordance with EPA regulations to prevent contamination. Avoid discharge into waterways. Storage Specifications: Store in a well-ventilated, fireproof cabinet. Keep away from ignition sources and incompatible materials such as strong oxidizers. Specialist Advice: Consult a Hazardous Materials Consultant or Occupational Hygienist for advice on emerging risks and exposure controls.

Workplace tasks

Painting Activities: Application of paints using brushes, rollers, or spray equipment. Surface Preparation: Cleaning and priming surfaces before painting. Clean-up Operations: Using solvents to clean brushes and tools.

Determine the level of risk

Matrix; A: Death or permanent disability; B: Serious injury; C: Lost time injury; D: Casualty Treatment; E: First Aid. 1: Very Likely; E; E; H; H; M. 2: Likely; E; H; H; M; M. 3: Possible; H; H; M; M; L. 4: Unlikely; H; M; M; L; L. 5: Very Unlikely; M; M; L; L; L. E: Extreme; H: High; M: Medium; L: Low. Level of risk: C3 - Medium

Safe system of work

Risk Controls (Hierarchy of Control): Elimination: Use water-based paints where feasible to eliminate VOCs. Substitution: Replace high-VOC solvents with low-VOC alternatives. Engineering Controls: Install ventilation systems, such as extraction fans, in painting areas. Use enclosed spray booths for spray-painting tasks. Administrative Controls: Rotate tasks to minimize individual exposure time. Conduct training on handling and disposal procedures. PPE: Provide respirators with appropriate filters, gloves, safety goggles, and protective clothing. Legal Requirements: Comply with Work Health and Safety Regulations 2011 for hazardous chemical management. Follow Safe Work Australia's Managing Risks of Hazardous Chemicals Code of Practice. Feedback During Development: Seek input from: Workers performing tasks involving the chemical. WHS Officers and Site Supervisors. Chemical suppliers or occupational hygienists.

How you advise others about safe systems of work

Methods to Advise: Training Sessions: Provide induction and task-specific training. Safety Signage: Post clear signage at storage and use areas (e.g., "Flammable Material" and "Use PPE"). Toolbox Talks: Regularly communicate updates and reminders about safe practices. SDS Access: Ensure workers know where to find the SDS for solvent-based paints. Individuals and Parties to Notify: Workers handling or exposed to the chemical. Site supervisors and project managers responsible for task oversight. Contractors or subcontractors working in proximity.

System review

Evaluation Process: Inspections: Conduct routine workplace inspections to check adherence to safe systems of work. Monitoring: Measure air quality to ensure VOC levels are within acceptable exposure standards. Assess workers’ use of PPE and compliance with procedures. Incident Reporting: Investigate any spills, exposures, or near-miss incidents for root causes. Recording Findings: Document findings in a Hazardous Chemicals Compliance Log or incident reporting system. Include details of inspections, monitoring results, and corrective actions taken. Notifying Outcomes: Workers: Provide feedback during toolbox talks or safety briefings. Management: Share reports with site supervisors, project managers, and WHS committee members. Regulators: Notify WorkSafe or equivalent regulatory body of significant incidents or non-compliance as required.

| Satisfactory Outcome |  |  |
| --- | --- | --- |

For the safe system of work identified in Activity 3, outline what training and development opportunities you may provide to assist in compliance with safe systems of work.

Training and development

| Satisfactory Outcome |  |  |
| --- | --- | --- |

BSBWHS431

Develop processes and procedures for controlling hazardous chemicals in the workplace

Quiz

| Participant name | Elise Painter |
| --- | --- |

FOR THE PARTICIPANT

Instructions

- You will complete written questions as outlined on the following pages. You must attempt all questions and provide enough detail in your responses to demonstrate your knowledge. - It is anticipated that you should complete this assessment within one (1) hour, but additional time may be allowed upon negotiation with the Assessor. - You must complete all questions unassisted by the Assessor or other personnel but may refer to reference material as needed. All questions must be answered correctly for a ‘Satisfactory’ assessment outcome to be awarded. - An oral examination may be allowed under certain circumstances, whereby your answers will be documented by another party. - Any written questions that are incomplete, or not answered satisfactorily will need to be completed again after further training support. - If you answer a question incorrectly but modify your response, put your initials next to the written change (or the Assessor must initial a verbal change).

Resources required

Pen

FOR THE ASSESSOR

Instructions

- Where oral examination has occurred, ensure that this is acknowledged in writing (e.g. FT001 and Assessor Comments) - All questions must be satisfactorily demonstrated by the Participant for a Satisfactory determination (indicated by an Assessor tick to the right of the question). - Assessors are to complete marking in pen of a different colour to the Participant. - If the Participant verbally modifies their response, write in the modified response verbatim and put your initials next to the written change.

ASSESSOR COMMENTS

As necessary, outline: - Details on reattempts, alternative arrangements or reasonable adjustments made. - Where a Participant is deemed ‘Not Satisfactory’, provide recommendations for future training.

- Which of the following would need to be recorded on a workplace register for hazardous chemicals? Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Acids |  |
|  |  | Disinfectants |  |
|  |  | Glue |  |
|  |  | Paint |  |
|  |  | Water |  |

- The ‘Managing risks of hazardous chemicals in the workplace – Code of Practice 2013’ describes the types of labels that may be present on chemicals used when working.

Match the following pictograms and their meanings by writing the appropriate letter next to each number in the centre column.

| Answer | Meaning |  |  | Pictograms |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1 | Warning | 1 - C | A |  |  |
|  | 2 | Human health | 2 - D | B |  |  |
|  | 3 | Acute toxicity | 3 - B | C |  |  |
|  | 4 | Corrosive | 4 - E | D |  |  |
|  | 5 | Flammable | 5 - A | E |  |  |

- For chemicals stored indoors, where must safety signs be placed?
Tick the most appropriate response.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Not publicly, to avoid drawing attention (e.g. in manifest) |  |
|  |  | At the entry to the specific room |  |
|  |  | At the main entrance to the workplace |  |

- Which two (2) factors are considered when assessing risks?
Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Consequence |  |
|  |  | Cost of risk control |  |
|  |  | Likelihood of occurrence |  |
|  |  | Observance of legislation |  |

- True or False (Tick your response): Information about hazards and risks can come from both internal sources (e.g. policies and procedures, workplace monitoring results) and external sources (e.g. specialists on emerging issues, training).

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

- Order the elements of the hierarchy of control, from 1 to 6 (i.e. most effective to least effective).

| Answer | Label (1 – 6) | Element |  |
| --- | --- | --- | --- |
|  | 3 | Isolate the hazard |  |
|  | 6 | Provide and use Personal Protective Equipment (PPE) |  |
|  | 1 | Eliminate the hazard |  |
|  | 4 | Control the hazard by engineering means |  |
|  | 5 | Control the hazard by administrative means |  |
|  | 2 | Substitute the hazard |  |

- True or False (Tick your response): In Queensland, the Agricultural Chemicals Distribution Control Act 1966 may require that you hold a commercial operator’s licence if you intend to use ground equipment to distribute herbicides.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

- True or False (Tick your response): You can apply for a permit to allow for the legal use of chemicals in certain ways that are contrary to the label instructions or for the limited use of an unregistered chemical product.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

- Which of the following information can be found in a Safety Data Sheet (SDS)? Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Disposal requirements |  |
|  |  | Handling and processing requirements |  |
|  |  | Transportation and storage limitations |  |
|  |  | Safe use of product |  |

- When monitoring chemicals in the workplace, which of the following information and data should be reported? Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Interpretation of results (e.g. non compliance) |  |
|  |  | The process undertaken |  |
|  |  | Information about equipment that was not used |  |
|  |  | Effects for workers and the workplace |  |

- True or False (Tick your response): All health monitoring reports must be submitted to the regulator.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

BSBWHS431

Develop processes and procedures for controlling hazardous chemicals in the workplace

PRACTICAL ASSESSMENT

| Participant name | Elise Painter |
| --- | --- |

FOR THE PARTICIPANT

Instructions

- The tasks outlined on the following pages will require performance of normal workplace activities, as a result of a structured activity set by an Observer in the workplace or a simulated activity set by your Assessor/Observer. - The timeframe for these tasks will be established by your Assessor/Observer, in consideration of location conditions and industry requirements. - You must complete all tasks unassisted by the Assessor or other personnel, unless the steps note this assistance. - You must attempt all tasks (and itemised steps) for a ‘Satisfactory’ assessment outcome to be awarded. - For any observation conducted that is incomplete, or without satisfactory performance, the observation will need to be completed again after further training support.

FOR THE ASSESSOR/OBSERVER

Instructions

- The following observation record should be used to judge and record your observations of the Participant as they complete the task(s) set. You must record your observations of the Participant’s performance. - For each occasion, indicate the task performed and whether it was a simulation or workplace-based task. - Any simulated assessment must ensure the Participant still performs under workplace conditions which reflect industry standards. - In the comments, provide clear detail regarding the task that the Participant was required to do (e.g. PPE used, materials handled, attachments connected, plant types). - All steps must be satisfactorily demonstrated by the Participant for a ‘Satisfactory’ determination (indicated by an Assessor tick to the right of each itemised step, or an Observer tick to the right of each itemised step, and confirmation by an Assessor in the Assessor Comments). - Where you have determined that one or more task steps have not been satisfactorily demonstrated, provide the Participant with additional support and outline details of such assistance in the Assessor/Observer Comments. Allow the Participant two (2) further opportunities to demonstrate the task again to allow an opportunity for a ‘Satisfactory’ assessment outcome to be achieved. - If after three (3) attempts the Assessor determines that the Participant cannot demonstrate the required steps, a ‘Not Satisfactory’ outcome is to be determined.

| TASK OUTCOME |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Task | Date | Task Type (Tick) | Location | Task outcome (Tick) |  |
|  |  |  |  | Satisfactory | Not Satisfactory |
| Task 1 (Occasion 1) | Select Date. | Workplace Simulation |  |  |  |
| Task 1 (Occasion 2) | Select Date. | Workplace Simulation |  |  |  |

ASSESSOR COMMENTS: GENERAL

As necessary, outline: - Details on reattempts, alternative arrangements or reasonable adjustments made. - Where a Participant is deemed ‘Not Satisfactory’, provide recommendations for future training.

| ASSESSOR COMMENTS: PRACTICAL TASK 1: OCCASION 1 |  |
| --- | --- |
| Individual student (as named on cover page) Multiple students [ All from Attendance Report, Specific students involved – identify students] |  |
| Task | Provide detail |
| 1.1C. 1.1D. 1.1E. 1.3A. | Specify potential non-compliance observed regarding implementation of procedures/processes related to chemicals at the workplace: |
| 1.2B. | Specify () communication/ consultation method applied when consulting and gaining feedback from PCBU: Face to face during meeting Face to face – on-the-job Telephone call/s Email Other (please specify): |
| 1.2D. | Specify how implementation of improved processes/procedures was monitored: Visually observed workers using controls Consultation with workers Monitoring for incidents/near misses Other (please specify): |

| ASSESSOR COMMENTS: PRACTICAL TASK 1: OCCASION 2 |  |
| --- | --- |
| Individual student (as named on cover page) Multiple students [ All from Attendance Report, Specific students involved – identify students] |  |
| Task | Details |
| 1.1C. 1.1D. 1.1E. 1.3A. | Specify potential non-compliance observed regarding implementation of procedures/processes related to chemicals at the workplace: |
| 1.2B. | Specify () communication/ consultation method applied when consulting and gaining feedback from PCBU: Face to face during meeting Face to face – on-the-job Telephone call/s Email Other (please specify): |
| 1.2D. | Specify how implementation of improved processes/procedures was monitored: Visually observed workers using controls Consultation with workers Monitoring for incidents/near misses Other (please specify): |

BSBWHS431

Develop processes and procedures for controlling hazardous chemicals in the workplace

Practical Task 1 (2X)

Description of task: The participant is to consult with the PCBU and review a chemical register to identify two (2) hazardous chemicals in the workplace. These may reflect actual workplace circumstances or the provided chemical register. The participant is to draft, develop and communicate processes and procedures for the control of each chemical. In response to observing non-compliances with the implementation of the processes and procedures, the participant is to develop and deliver a short presentation (e.g. PowerPoint presentation slides with accompanying face-to-face delivery or real-time video, in a simulated environment arranged with your Assessor) to address compliance requirements of workers. The participant is to submit the following evidence with this Practical Assessment sheet. There should be two (2) of each of the following documents submitted (i.e. one for each chemical): - Processes and procedures for control hazardous chemical - Email communications with: - PCBU – identifying hazardous chemicals, and seeking feedback about processes and procedures - Workers – promoting developed processes and procedures - Presentation materials (e.g. PowerPoint) Tasks are at the Assessor’s Discretion, but detail of tasks undertaken must be recorded in the Assessor’s Comments (on the back of the Practical Assessment cover page).

Resources required: Computer, Chemical Register, SDS, Code of Practice.

| When completing the task, did the Participant: | Tick if satisfactorily completed. Provide comments if left unticked. |  |
| --- | --- | --- |
|  | 1st Occasion | 2nd Occasion |
| 1 Hazard and risk identification |  |  |
| Consult with PCBU and chemical register to identify hazardous chemical used in the workplace. |  |  |
| Identify workplace tasks involving identified hazardous chemical. |  |  |
| Refer to legislation, codes of practice and SDS to identify requirements for identified hazardous chemical regarding: - Licences and permits - Safe use, handling, processing and disposal - Exposure standards, and associated monitoring - Transportation and storage (including labelling and signage). |  |  |
| Determine potential worker exposure to identified hazardous chemical – using specialist advice, as required. |  |  |
| Assess risks associated with identified hazardous chemical, reflecting on information from workplace environmental monitoring. |  |  |
| 2 Develop and monitor processes and procedures |  |  |
| In line with organisational requirements, develop safe systems of work, policies and procedures addressing: - Safe work practices - Risk control measures, as per hierarchy of control - Incident response |  |  |
| Seek and integrate PCBU feedback about developed processes and procedures. |  |  |
| Using email, communicate and promote developed processes and procedures to workers. |  |  |
| Monitor and review implementation of control processes and procedures, identifying non-compliances. |  |  |
| 3 Maintaining WHS legislative compliance |  |  |
| Develop a presentation for workers which covers: - Identification of PCBU’s duties regarding identified hazardous chemical - Non-compliances highlighted from workplace monitoring - Recommendations for improvements. |  |  |
| Deliver presentation (in person/via live stream video) and listen to your audience to respond appropriately to any questions. |  |  |

| HAZARDOUS CHEMICAL REGSITER | comments | - Flammable liquid - Aspiration hazard - Skin corrosion/irritation | - Poisons Schedule S6 - Flammable - Corrosive | - Flammable liquid/vapour - Harmful if swallowed | - Organ toxicity - Toxic to aquatic life | - Flammable liquid/vapour - Serious eye irritation | - Highly flammable liquid and vapour |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | location | Workshop cage | Workshop cage | Workshop cage | Outdoor storage area | Office | Indoor kitchen area |
|  | quantitiy | 20L | 500ml | 4L | 1L | 250ml | 500ml |
|  | issue date of sds | 11/08/2016 | 27/06/2017 | 10/12/2018 | 08/05/2019 | 15/02/2020 | 26/03/2020 |
|  | nAME OF CHEMICAL | Mineral Turpentine | Ajax Hydrogen Peroxide 30% 100 VOL | Acetone | Esplanade Herbicide | Sanikleen (Ethyl alcohol) | Rasmsol Disinfectant Spray |