RIIMPO402D

Apply the principles of earthworks construction

THEORY EXAM INSTRUCTIONS - This exam will occur under standard assessment conditions. - An oral examination may be allowed under certain circumstances. Answers will be documented and FT003 (Oral examination declaration form) must be completed. - Participants must provide enough detail in their responses to demonstrate their knowledge and skills. - Please fill in the details below, providing your name, signature and date of assessment. - Your signature acknowledges that this is your own work. - If a participant initially answers a question incorrectly but modifies their response, they must follow the instructions below: - If a participant changes their response in writing, they must put their initials next to the written change. - If a participant changes their response verbally, this must be noted on the exam paper by the assessor.

| Participant name | Scott Jensen |
| --- | --- |
| Participant signature |  |

| Assessor name |  |
| --- | --- |
| Assessor signature |  |

| Date | Click here to enter a date. |
| --- | --- |

| Number of questions | Number of questions answered correctly | Satisfactory? |  |
| --- | --- | --- | --- |
| 45 |  | Yes | No |

Assessor Comments

- Compliance documentation and reporting:

- Give one (1) example of each of the following types of compliance documentation relating to earthworks construction:

- Legislation.

- Organisational requirements.

- Client requirements.

- Site operational requirements.

- What are two (2) government agencies or statutory authorities you may have to report to?

| Answer | - Legislation - Equal employment opportunities. Organisational requirements - Workplace relations. Client requirements - Clients standards and expectations. Site operational requirements - Emergency response and evacuations procedures-developed to meet the specific hazards, risks or environmental considerations of the sire you are working on. B) Workcover , EPA, RTA. |  |
| --- | --- | --- |

- Administrative documentation and project quality management:

- List three (3) types of administrative documentation that you may have to complete.

- What are three (3) factors of project quality management?

| Answer | - Daily activities, Ordering and recording, Workplace incident and accident management - Dimensions and tolerances of tasks. Material standards. Work standards. |  |
| --- | --- | --- |

- What is the aim of OHS/WHS legislation?

| Answer | The aim of the OHS/WHS legislation is to ensure the safety of everyone working on or visiting the site. |  |
| --- | --- | --- |

- List three (3) requirements that may be covered by the environmental management plan for your worksite?

| Answer | An organisational or project environmental management plan. Noise and vibration management. Dust management. |  |
| --- | --- | --- |

- What are the three (3) plans that outline the requirements and procedures for controlling traffic and people around and through the work zone?

| Answer | Traffic Management Plan, Vehicle Management Plan, Site Isolation Plan |  |
| --- | --- | --- |

- Why is it important to consult and communicate with others during the risk management process?

| Answer | This will help ensure that risks and hazards are not only effectively identified but that those involved with controlling and treating them are clear of their role and responsibilities in the risk management process. |  |
| --- | --- | --- |

- What should you do if you don’t understand the meaning of a particular term, phrase or word used on your worksite?

| Answer | You should seek advice from your superior or person in charge or look back at your policies and procedures documentation. You should always make sure that everyone on the worksite is familiar with all terms, specific phrases and words used on the worksite. |  |
| --- | --- | --- |

- Geotechnical, geological, survey and engineering data:

- List three (3) ways that geotechnical data is used in earthworks.

- List three (3) uses of soil or geological data.

- What is survey and engineering information used for?

| Answer | 1. Check conditions and materials on and under the ground surface. 2. Determine their physical/mechanical and chemical properties. 3. Evaluate the stability of natural slopes and man-made soil deposits. - Defining material types. - Identifying wet and dry. - Identifying broken ground, faults and joints. Survey and engineering information is used to define work areas and zones, for example: Floor heights, Floor and bench grades, Road profile requirements. |  |
| --- | --- | --- |

- Hydrological and meteorological data:

- Why is hydrological data important?

- List three (3) factors that may be included in meteorological data.

| Answer | Hydrological data mainly concerns surface water and ground water on the site and it is critical to the success of earthworks operations. - Rain fall, temperature, wind direction and speed |  |
| --- | --- | --- |

- Project plans and specifications:

- What are three (3) things provided in project plans and drawings?

- What is the purpose of project and task specifications?

| Answer | Location of the site and earthworks in relation to the surrounding area. The position of structures, roads, access areas. Layout of drainage lines. - Project and task specifications list the types and characteristics of particular earthworks tasks to be carried out, They may be presented in the quality requirements, project drawings, notes and descriptions, or outlined in work instructions. They also detail how activities will be completed. |  |
| --- | --- | --- |

- Drainage and dewatering:

- List two (2) examples of drainage and dewatering requirements.

- Why must any drainage and dewatering issues be managed promptly and effectively once you have identified them?

| Answer | Any form of erosion or sediment controls. Shifting water from one location to another using the site controls or water transfer devices such as pumps. - The most important part about draining and dewatering a construction site is that no dirty or contaminated water or pollution must leave the site. If this happens, severe penalties can apply that can be imposed by federal, state and local government authorities. |  |
| --- | --- | --- |

- Why is it important that earthworks construction activities are scheduled within a job plan?

| Answer | Scheduling all of these activities will ensure that processes are conducted in the correct order or sequence and that the appropriate resources are available when they are needed. |  |
| --- | --- | --- |

- Soil density and testing:

- Why do you need to test the relationship between soil density and moisture when scheduling activities?

- What does ‘plasticity index’ mean?

- Why is it important for materials to have the right balance of particle sizes?

| Answer | - To gain adequate compaction - It determines how the materials can be shaped and will relate to the clay content within the material. - To ensure the correct construction of layers. |  |
| --- | --- | --- |

- Earthwork calculations:

- How do you calculate the following in earthworks construction:

- Grade or slope.

- Volume.

- Area.

- What is percentage or cross fall?

- How do you work out the rates of resource consumption?

| Answer | - Grade = Rise : Run, Volume = Length x Width x Depth, Area = Width x Depth - This is generally the slope placed on earthworks to ensure water can drain off the work area - You can work volume of delivered material and work out how much material has been used throughout the day by using the calculations. |  |
| --- | --- | --- |

- Sequencing of civil works task:

- How does an understanding of the relationship between the different areas of civil works help in planning work?

- What is the sequence of tasks for drainage works?

| Answer | - So tasks can be completed in the right order. - Forming drain lines; forming out the work area; laying pipes, box drains or culverts; backfilling; landscaping as appropriate. |  |
| --- | --- | --- |

- Job planning and administration:

- List four (4) requirements that should be included when drafting a job plan.

- What are three (3) examples of administration procedures and requirements?

| Answer | - Human resources; sub-contractor support; activity scheduling; risk management. - Staff requirements such as leave timesheets, wages or other documentation; Reporting achieved targets or productivity, procurement. |  |
| --- | --- | --- |

- Interpret requirements and provide estimates:

- Why is it important to correctly interpret the requirements of the project contract?

- What are two (2) factors you may have to provide estimates for, and how do you go about it?

| Answer | - to ensure that all earthworks plans, client requirements and quality standards are known, understood and achieved. - To estimate the cost of completing a task you add the cost of production to the cost of materials and to estimate the completion time of a job, you work out how long each individual activity should take from start to finish., |  |
| --- | --- | --- |

- List four (4) types of resources that will need to be available and ready for earthworks construction tasks.

| Answer | Labour; Plant Equipment and Tools; Material supply vehicles; Construction materials. |  |
| --- | --- | --- |

- How can you be sure that you have followed the relevant resource requirements and procedures?

| Answer | The resources for the activity should be checked and confirmed with the relevant personnel/workers such as supervisor/managers, engineers or project management staff to ensure that resource requirements and procedures have been adhered to. |  |
| --- | --- | --- |

- How do you confirm that plant and equipment is suitable for the task?

| Answer | checking the intended application or use of each item is in line with its capabilities and limitations and you may need to refer to the machine specifications and other details listed in the individual operator’s manuals or your site documentation., |  |
| --- | --- | --- |

- Provide instructions and supervise team members:

- List three (3) ways you can provide clear and timely instructions to the work team.

- List three (3) team members and others who may be involved in the earthworks activities you are supervising.

| Answer | - Two way radios; mobile phones; Landline phones. - Personnel/Workers directly involved in the task; Supplier’s representatives; Sub-contractors representatives. |  |
| --- | --- | --- |

- List two (2) factors that may be detailed in your worksite communication requirements and procedures?

| Answer | Documentation requirements, e.g. verbal or written reports. Protocols for using communications equipment. |  |
| --- | --- | --- |

- Prepare messages and instructions:

- Why is it important to prepare for and conduct briefings, toolbox and site meetings?

- What should you keep in mind when preparing and delivering short messages or instructions?

| Answer | - Briefings, handovers and toolbox, and site meetings are vital in aiding communications on the site, They allow interaction between management, supervisory staff, workers, subcontractors, and any other people involved in the activities. - When preparing messages or instructions, make sure they are appropriate and not confusing for the receiver. It helps to remember the target audience you’re aiming your message at. |  |
| --- | --- | --- |

- Why is it important for a leader/supervisor to develop and maintain workplace relationships?

| Answer | It promotes a good work environment and encourages effective interactions between team members, supervisors, management, and any additional people that may be involved with the work activities. |  |
| --- | --- | --- |

- What factors will determine the leadership style and techniques you adopt?

| Answer | Your style of leadership will depend on your team members, organisational requirements, work procedures, and the type of earthworks construction being undertaken, the leadership techniques you adopt will depend on the nature and dynamics of the group and also the experience level of the people concerned. |  |
| --- | --- | --- |

- Supervisor techniques:

- What are four (4) things you need to be able to do as a supervisor?

- List the five (5) techniques commonly used in mentoring.

| Answer | - Communicate; organise; plan; manage people and resources. - Accompanying; sowing; catalysing; showing; harvesting. |  |
| --- | --- | --- |

- What do set out requirements and procedures include?

| Answer | Establishing control lines; establishing cleared width; determining batters; establishing constructions offsets, |  |
| --- | --- | --- |

- What are construction offsets and how are they established?

| Answer | Offsets refer to how far from a control line an activity is to take place or an area is to be cleared, quite often survey markers and level pegs are used to define the work area. |  |
| --- | --- | --- |

- List three (3) factors of road geometry.

| Answer | Grade or slope; curves and curve radius; crests. |  |
| --- | --- | --- |

- What are levelling devices used for in the setting out process?

| Answer | Determining drainage grades, gradients, batters an benches also measuring offsets and helping with road geometry. |  |
| --- | --- | --- |

- What are the four (4) aspects of project and task monitoring that you must carry out?

| Answer | Ongoing risk assessment and management. Engineering survey. Sampling and testing. General supervision, observation and recording. |  |
| --- | --- | --- |

- Engineering principles and interpret test results:

- Why should earthworks operators be aware of the engineering survey principles for the job?

- Why do you need to interpret material properties and compaction test results?

| Answer | - To ensure earthworks construction is being done correctly, with correct layer formation and within the specified tolerances. - To allow for additional treatments when necessary. |  |
| --- | --- | --- |

- Monitor earthworks tasks:

- List four (4) criteria that may relate to performance of the tasks you are monitoring.

- What are three (3) methods you might use for monitoring earthworks tasks?

| Answer | - Compaction; layer thickness; particle size; clay content. - Field checklists; inspections and test plans; survey and level checks. |  |
| --- | --- | --- |

- Why should routine inspections of earthworks operations be carried out?

| Answer | To ensure compliance with Occupational Health & Safety/Work Health & Safety (OHS/WHS) legislation, contractual obligations and requirements specific to the project and the worksite. |  |
| --- | --- | --- |

- List four (4) methods and operational techniques involved in earthworks construction practice.

| Answer | Site preparation; extraction; load and haulage; placement. |  |
| --- | --- | --- |

- How will you find out the operational techniques for the earthworks job you are working on?

| Answer | Industry standards usually apply across all earthworks construction sites, however some organisations require different operational techniques to be used in certain cases.a |  |
| --- | --- | --- |

- In ground surface treatment, what is proof rolling?

| Answer | A plant item such as a roller, loaded truck, or other nominated item may be used to see if there is any movement in the ground underneath. Movement under the wheels could indicate unsuitable or insufficiently compacted material in lower layers. In some cases further testing such as deflection monitoring may need to be carried out or unsuitable material removed before continuing. |  |
| --- | --- | --- |

- Give two (2) examples of how you should inform operators and other personnel/workers in writing and verbally that changes to the job plan or relevant earthworks construction practice have been initiated?

| Answer | Written communication – plans, sketches and drawings, work instructions or other documentation. Oral communication – briefings, toolbox meetings, verbal instructions, discussions with operators, team meetings, or relevant conversations based on the activity. |  |
| --- | --- | --- |

- What are four (4) maintenance activities that may be carried out by earthworks construction operators?

| Answer | Servicing and monitoring, e.g. changing filters, topping up oils and hydraulic fluids; Cleaning and checking for damage before and after use; Fitting and removing attachments and mainor repairs, e.g. replacing ripping teeth or boots. |  |
| --- | --- | --- |

- How can you ensure that your team carries out maintenance activities in line with site procedures and the manufacturer’s specifications?

| Answer | You may need to create a schedule to ensure that regular times are allocated to your team for cleaning, checking, repairing and carrying out other maintenance tasks. |  |
| --- | --- | --- |

- List four (4) reasons why you should follow organisational and/or site requirements and procedures for preparing and presenting reports.

| Answer | Activity planning; ordering and tracing of materials; budgeting; legal requirements for reporting.a |  |
| --- | --- | --- |

- What are three (3) guidelines you should keep in mind when preparing reports?

| Answer | Keep it clear and concise – use a style of language that is simple enough to be read quickly but technical enough to meet the needs of the audience; Pay attention to your grammar and make sure the information is clear; Proofread it – errors in your report reduce your credibility as well as that of your organisation. |  |
| --- | --- | --- |

- What are logbooks and diaries used for?

| Answer | They are used to track daily and weekly activities. Logbooks are often used to verify the number of hours an operator has worked on a machine, training that has been completed or other task-related information. |  |
| --- | --- | --- |

- List four (4) things that you need to do when recommending changes or improvements to the safety, effectiveness or efficiency of tasks.

| Answer | Summarise the situation; Clarify why you are providing advice; Outline all the considerations and provide details as to how the risks or hazards could be controlled. |  |
| --- | --- | --- |

- How do you develop individual workers or the team as a whole?

| Answer | By listening and considering ideas and suggestions, you as a supervisor are building and developing the individuals and teams in your charge and If you are approachable and open to suggestions, your team will function at a higher level and team morale will be enhanced. |  |
| --- | --- | --- |