RIIMPO402D

Apply the principles of earthworks construction

THEORY EXAM INSTRUCTIONS - This exam will occur under standard assessment conditions. - An oral examination may be allowed under certain circumstances. Answers will be documented and FT003 (Oral examination declaration form) must be completed. - Participants must provide enough detail in their responses to demonstrate their knowledge and skills. - Please fill in the details below, providing your name, signature and date of assessment. - Your signature acknowledges that this is your own work. - If a participant initially answers a question incorrectly but modifies their response, they must follow the instructions below: - If a participant changes their response in writing, they must put their initials next to the written change. - If a participant changes their response verbally, this must be noted on the exam paper by the assessor.

| Participant name | Taunaola Kei |
| --- | --- |
| Participant signature |  |

| Assessor name |  |
| --- | --- |
| Assessor signature |  |

| Date | 30/05/2019 |
| --- | --- |

| Number of questions | Number of questions answered correctly | Satisfactory? |  |
| --- | --- | --- | --- |
| 45 |  | Yes | No |

Assessor Comments

- Compliance documentation and reporting:

- Give one (1) example of each of the following types of compliance documentation relating to earthworks construction:

- Legislation.

- Organisational requirements.

- Client requirements.

- Site operational requirements.

- What are two (2) government agencies or statutory authorities you may have to report to?

| Answer | - Legislation - Occupational Health & Safety/Work Health & Safety (OHS/WHS) requirements. Organisational requirements - Workplace relations. Client requirements - Client standards and expectations Site operational requirements - work instructions, safe operating procedures (SOPs) and other site and equipment safety requirements. |  |
| --- | --- | --- |

- Administrative documentation and project quality management:

- List three (3) types of administrative documentation that you may have to complete.

- What are three (3) factors of project quality management?

| Answer | - Daily activities - Ordering and reordering - Workplace incident and accident management - Dimensions and tolerances of tasks. - Material standards. - Work standards. |  |
| --- | --- | --- |

- What is the aim of OHS/WHS legislation?

| Answer | The aim of the legislation is to ensure the safety of everyone working on or visiting the site. |  |
| --- | --- | --- |

- List three (3) requirements that may be covered by the environmental management plan for your worksite?

| Answer | - An organisational or project environmental management plan. - Noise and vibration management. - Dust management. |  |
| --- | --- | --- |

- What are the three (3) plans that outline the requirements and procedures for controlling traffic and people around and through the work zone?

| Answer | - Traffic Management plan - Vehicle management plan - Site isolation plan |  |
| --- | --- | --- |

- Why is it important to consult and communicate with others during the risk management process?

| Answer | This will help ensure that risks and hazards are not only effectively identified but that those involved with controlling and treating them are clear of their role and responsibilities in the risk management process. |  |
| --- | --- | --- |

- What should you do if you don’t understand the meaning of a particular term, phrase or word used on your worksite?

| Answer | Talk to your manager, supervisor and workers |  |
| --- | --- | --- |

- Geotechnical, geological, survey and engineering data:

- List three (3) ways that geotechnical data is used in earthworks.

- List three (3) uses of soil or geological data.

- What is survey and engineering information used for?

| Answer | - Identify soil and rock types and characteristics. - Check conditions and materials on and under the ground surface. - Determine their physical/mechanical and chemical properties. - Defining material types. - Identifying wet and dry areas. - Identifying broken ground, faults and joints. - Floor heights. - Bench heights and widths. - Floor and bench grades. |  |
| --- | --- | --- |

- Hydrological and meteorological data:

- Why is hydrological data important?

- List three (3) factors that may be included in meteorological data.

| Answer | - Hydrological data helps to determine the properties, distribution and effects of water, moisture content and exposures on the project/site soil, rocks and atmosphere. - Rainfall – Humidity - Temperature |  |
| --- | --- | --- |

- Project plans and specifications:

- What are three (3) things provided in project plans and drawings?

- What is the purpose of project and task specifications?

| Answer | - Location of the site and earthworks in relation to the surrounding area. - The position of structures, roads, access areas Layout of drainage lines. - list the types and characteristics of particular earthworks tasks to be carried out. |  |
| --- | --- | --- |

- Drainage and dewatering:

- List two (2) examples of drainage and dewatering requirements.

- Why must any drainage and dewatering issues be managed promptly and effectively once you have identified them?

| Answer | - Any form of erosion or sediment controls. - Pumping out of sumps or pits. - is that no dirty or contaminated water or pollution must leave the site. If this happens, severe penalties can apply that can be imposed by federal, state and local government authorities. |  |
| --- | --- | --- |

- Why is it important that earthworks construction activities are scheduled within a job plan?

| Answer | Scheduling all of these activities will ensure that processes are conducted in the correct order or sequence and that the appropriate resources are available when they are needed. |  |
| --- | --- | --- |

- Soil density and testing:

- Why do you need to test the relationship between soil density and moisture when scheduling activities?

- What does ‘plasticity index’ mean?

- Why is it important for materials to have the right balance of particle sizes?

| Answer | - To gain adequate compaction it is necessary to have the density and moisture in balance. If the material moisture is too high, then it can become difficult to work or simply turn to mud. If the moisture content is too low the material will not bind and therefore will not compact properly. - The plasticity index (PI) determines how the materials can be shaped and will relate to the clay content within the material. High clay content material or high PI may be very difficult to work on its own and may require additives to allow suitable construction - It is important to have the right balance of particle sizes to ensure the correct construction of layers. There must be the correct balance of particle sizes for the materials to meet specification requirements. |  |
| --- | --- | --- |

- Earthwork calculations:

- How do you calculate the following in earthworks construction:

- Grade or slope.

- Volume.

- Area.

- What is percentage or cross fall?

- How do you work out the rates of resource consumption?

| Answer | - Grade = rise / run - Volume = length x width x depth - Area = width x depth - The slope placed on earthworks to ensure water can drain off the work area. For example, a 3% cross fall would equate to a 3 metre rise across a 100 metre area. - Daily production figures will often be used as a guideline to work out consumption rates. Be aware, however, that not all materials are consumed at the same rate and supply rates may differ from worksite to worksite. |  |
| --- | --- | --- |

- Sequencing of civil works task:

- How does an understanding of the relationship between the different areas of civil works help in planning work?

- What is the sequence of tasks for drainage works?

| Answer | - Always ensure that all base activities or sub-tasks are completed. These are the things that need to be completed prior to the commencement of other activities. You will need to consider all of the activities and deadlines on the site. - 1. Forming drain lines. - 2. Forming out the work area, that is building the frame to hold the drain system. - 3. Laying pipes, box drains or culverts. - 4. Backfilling. - 5. Landscaping as appropriate. |  |
| --- | --- | --- |

- Job planning and administration:

- List four (4) requirements that should be included when drafting a job plan.

- What are three (3) examples of administration procedures and requirements?

| Answer | Human resources. Plant and machinery. Construction materials, including quality and delivery. Occupational Health & Safety/Work Health & Safety (OHS/WHS). - Staff requirements such as leave timesheets, wages or other documentation. - Reporting achieved targets or productivity. - Procurement. - Supply agreements |  |
| --- | --- | --- |

- Interpret requirements and provide estimates:

- Why is it important to correctly interpret the requirements of the project contract?

- What are two (2) factors you may have to provide estimates for, and how do you go about it?

| Answer | - To ensure that all earthworks plans, client requirements and quality standards are known, understood and achieved. - To estimate the completion time of a job, you work out how long each individual activity should take from start to finish. - You then determine how many activities need to be completed. The total of these two figures should give an estimate of the total hours. |  |
| --- | --- | --- |

- List four (4) types of resources that will need to be available and ready for earthworks construction tasks.

| Answer | - Labour - Plant, equipment and tools - Material supply vehicles - Construction materials |  |
| --- | --- | --- |

- How can you be sure that you have followed the relevant resource requirements and procedures?

| Answer | The resources for the activity should be checked and confirmed with the relevant personnel/workers such as supervisor/managers, engineers or project management staff to ensure that resource requirements and procedures have been adhered to. |  |
| --- | --- | --- |

- How do you confirm that plant and equipment is suitable for the task?

| Answer | Checking the intended application or use of each item is in line with its capabilities and limitations. You may need to refer to the machine specifications and other details listed in the individual operator’s manuals or your site documentation. Prior to work starting, make sure that personnel/workers know how to use each tool and piece of equipment or plant correctly, in line with site and equipment safety requirements. |  |
| --- | --- | --- |

- Provide instructions and supervise team members:

- List three (3) ways you can provide clear and timely instructions to the work team.

- List three (3) team members and others who may be involved in the earthworks activities you are supervising.

| Answer | - Toolbox meetings - Handovers - Site meetings - Personnel/Workers directly involved in the task - Other members of the organisation's management team. - Supplier’s representatives. |  |
| --- | --- | --- |

- List two (2) factors that may be detailed in your worksite communication requirements and procedures?

| Answer | Documentation requirements, e.g. verbal or written reports Protocols for using communications equipment. |  |
| --- | --- | --- |

- Prepare messages and instructions:

- Why is it important to prepare for and conduct briefings, toolbox and site meetings?

- What should you keep in mind when preparing and delivering short messages or instructions?

| Answer | - It’s important that the work team or teams are kept informed of issues that may relate to safety, quality, environment, traffic management, or general work activities. The level of information that is given out will be suitable to work operations and the people involved. - Make sure they are appropriate and not confusing for the receiver. It helps to remember the target audience you’re aiming your message at. - Always give the receiver the opportunity to confirm the instruction or message and to ask questions if necessary. |  |
| --- | --- | --- |

- Why is it important for a leader/supervisor to develop and maintain workplace relationships?

| Answer | It promotes a good work environment and encourages effective interactions between team members, supervisors, management, and any additional people that may be involved with the work activities. It is important to make sure that everyone observes organisational and site requirements for working and interacting with others. |  |
| --- | --- | --- |

- What factors will determine the leadership style and techniques you adopt?

| Answer | Your style of leadership will depend on your team members, organisational requirements, work procedures, and the type of earthworks construction being undertaken. |  |
| --- | --- | --- |

- Supervisor techniques:

- What are four (4) things you need to be able to do as a supervisor?

- List the five (5) techniques commonly used in mentoring.

| Answer | - Communicate. - Organise. - Plan. - Manage people and resources. - Liaise with both management and your work team. - Accompanying - Sowing - Catalysing - Showing - Harvesting |  |
| --- | --- | --- |

- What do set out requirements and procedures include?

| Answer | - Establishing Control Lines Control lines are the base line for measuring from. They vary from site to site but could comprise boundary lines, fences, string lines, defined work areas, or other details as indicated in plans or drawings. - Establishing Cleared Width The cleared width generally indicates the level of clearing that must be undertaken for the earthwork’s tasks. It ensures that the work stays within a defined footprint and aims to minimise environmental damage that may occur. - Determining Batters Batters are the slopes that may be naturally formed as part of the work location, for example in a cut area or embankment fill areas. Batters often have specific set out requirements to meet slope and grade dimensions. - Establishing Construction Offsets Before any project begins, it is important that the boundaries or clearance zones are determined and marked out. |  |
| --- | --- | --- |

- What are construction offsets and how are they established?

| Answer | Offsets refer to how far from a control line an activity is to take place, or an area is to be cleared. A negative number means the offset is to the left of the line, a positive number means the offset is to the right of the line. |  |
| --- | --- | --- |

- List three (3) factors of road geometry.

| Answer | Grade or slope Curves and curve radius Crests |  |
| --- | --- | --- |

- What are levelling devices used for in the setting out process?

| Answer |  Determining drainage grades.  Determining gradients, batters and benches.  Measuring offsets.  Helping with road geometry. |  |
| --- | --- | --- |

- What are the four (4) aspects of project and task monitoring that you must carry out?

| Answer | Ongoing risk assessment and management. Engineering survey. Sampling and testing. General supervision, observation and recording. |  |
| --- | --- | --- |

- Engineering principles and interpret test results:

- Why should earthworks operators be aware of the engineering survey principles for the job?

- Why do you need to interpret material properties and compaction test results?

| Answer | - The principles may be relatively simple or quite complex depending on the tasks being undertaken. They will be defined in the project specifications and organisational procedures. - The testing results will indicate material variances throughout the works. Test results are also used to ensure levels of compaction are achieved based on specifications. |  |
| --- | --- | --- |

- Monitor earthworks tasks:

- List four (4) criteria that may relate to performance of the tasks you are monitoring.

- What are three (3) methods you might use for monitoring earthworks tasks?

| Answer | - Compaction. - Layer thickness. - Particle size. - Clay content. - Field checklists. - Inspection and test plans. - Site inspections that may be daily, weekly, or based on a particular work area - Survey and level checks |  |
| --- | --- | --- |

- Why should routine inspections of earthworks operations be carried out?

| Answer | To ensure compliance with Occupational Health & Safety/Work Health & Safety (OHS/WHS) legislation, contractual obligations and requirements specific to the project and the worksite. |  |
| --- | --- | --- |

- List four (4) methods and operational techniques involved in earthworks construction practice.

| Answer |  Site preparation.  Extraction.  Load and haulage.  Placement.  Distribution.  Surface finishing. |  |
| --- | --- | --- |

- How will you find out the operational techniques for the earthworks job you are working on?

| Answer | Specification requirements or contract documentation. |  |
| --- | --- | --- |

- In ground surface treatment, what is proof rolling?

| Answer | A plant item such as a roller, loaded truck, or other nominated item may be used to see if there is any movement in the ground underneath. Movement under the wheels could indicate unsuitable or insufficiently compacted material in lower layers. In some cases further testing such as deflection monitoring may need to be carried out or unsuitable material removed before continuing. |  |
| --- | --- | --- |

- Give two (2) examples of how you should inform operators and other personnel/workers in writing and verbally that changes to the job plan or relevant earthworks construction practice have been initiated?

| Answer | Written communication – plans, sketches and drawings, work instructions or other documentation. Oral communication – briefings, toolbox meetings, verbal instructions, discussions with operators, team meetings, or relevant conversations based on the activity. |  |
| --- | --- | --- |

- What are four (4) maintenance activities that may be carried out by earthworks construction operators?

| Answer | Type of plant, machinery, tool or equipment. Manufacturer’s specifications and instructions. Servicing requirements. Level of maintenance training required. |  |
| --- | --- | --- |

- How can you ensure that your team carries out maintenance activities in line with site procedures and the manufacturer’s specifications?

| Answer | Fitting and removing attachments. Minor repairs, e.g. replacing ripping teeth or boots. Helping the mechanics with maintenance and repairs. Recording and reporting of damage, defects or faults. |  |
| --- | --- | --- |

- List four (4) reasons why you should follow organisational and/or site requirements and procedures for preparing and presenting reports.

| Answer | Activity planning Ordering and tracing of materials Budgeting Legal requirements for reporting |  |
| --- | --- | --- |

- What are three (3) guidelines you should keep in mind when preparing reports?

| Answer |  Keep it clear and concise – use a style of language that is simple enough to be read quickly but technical enough to meet the needs of the audience.  Pay attention to your grammar and make sure the information is clear.  Proofread it – errors in your report reduce your credibility as well as that of your organisation. |  |
| --- | --- | --- |

- What are logbooks and diaries used for?

| Answer | Used to track daily and weekly activities. Logbooks are often used to verify the number of hours an operator has worked on a machine, training that has been completed or other task-related information |  |
| --- | --- | --- |

- List four (4) things that you need to do when recommending changes or improvements to the safety, effectiveness or efficiency of tasks.

| Answer | Summarise the situation. Clarify why you are providing advice. Outline all the considerations. Explain the risks or hazards. |  |
| --- | --- | --- |

- How do you develop individual workers or the team as a whole?

| Answer | By listening and considering ideas and suggestions, you as a supervisor are building and developing the individuals and teams in your charge. If you are approachable and open to suggestions, your team will function at a higher level and team morale will be enhanced. |  |
| --- | --- | --- |