# Hazard Identification Form Template

| Project name | Maroochydore Private Hospital |
| --- | --- |
| Prepared by | Ric |
| Date completed | 19/02/2024 |

Table Format

|  |  |  | Control measure |  |  |
| --- | --- | --- | --- | --- | --- |
| Hazard | Affected personnel | Associated risks | Control measure for implementation | Personnel who will implement | Deadline for implementation |
| Mobile Plant | All Workers | Crush/struck by | Pos coms/exclusion zones | Plant operator and Spotter | Prior to commencement of task- 9:am 19/02/2024 |
| Vehicle access to site | All workers | Plant interaction Public/pedestrian interaction, vehicle roll ways | Construction traffic to follow designated route via controlled site access points | Traffic management | Prior to commencement of task- 9:am 19/02/2024 |
| House keeping | All workers | Poor Housekeeping (Slips, trips, falls) | Is adequate to allow persons who are not employees to move safely within the place of work and facilitates safe access to and egress from the place of work, including emergency exits Daily weekly cleans bins provided | All workers | During and After completed works |
| PPE selection, supply and use | All workers | Not meeting Australian standards | Work activities to be employed that eliminate the requirement for extra PPE other than site mandatory PPE where possible Employer to provide compliant PPE as per SWMS | All workers | Prior to work commencing each day |
|  |  |  | Control measure |  |  |
| Hazard | Affected personnel | Associated risks | Control measure for implementation | Personnel who will implement | Deadline for implementation |
| Fire protection / detection operation during works | All workers | Unarmed Set off due to work activity | All fire sensors to be identified before work commences and protected during works During hot works a Fire blanket is to be placed between work area and sensor Complete Hot Works Permit and identify all fire heads and sensors for protection | McNab Subcontractor (PCBU) | Prior to work commencing each day |
| Workplace Violence | All workers | Intimidation, Bullying | Provide counselling for those involved During Site Induction, identify that bullying and harassment is unacceptable on site | McNab Subcontractor (PCBU) | At all times |
| Mobile phone use | All workers | Personal injury | No mobile phones are to be used when operating plant Persons at the workplace should never make or receive calls when working in the vicinity of operating plant If phone needs to be answered or calls need to be made, check surroundings or move to a safe area. Persons using phone on site are to isolate themselves from moving plant Persons using phone are not to walk around the site whilst using the mobile phone | McNab Subcontractor (PCBU) | At all times |
| Workplace Stress | All workers | Work hours, work environment | Where possible, employees not to be exposed to excessive heat or cold, irritant noise, poor lighting or physical hazards Ensure regular consultation with employees through tool box talks Working hours not to exceed those stipulated in award. Welfare and trauma counselling made available to all staff in the event of a serious incident on site | McNab Subcontractor (PCBU) | At all times |
| Dust | All workers | Dust | Offsite cutting and prefabrication should be used as much as possible Erect cutting rooms or designated cutting areas to capture dust and with adequate ventilation Use methods to suppress dust i.e. water spray, dust barriers Regular clean ups to be completed on site Equipment to be regularly cleaned | McNab Subcontractor (PCBU) | At all times |
| Noise / vibration | All workers | Noise / vibration | Works to be completed within DA conditional hours. Where there is a potential for Noise and vibration exposure limited to noise in excess of: 85 dB(A) continuously for eight hours or a daily noise dose equivalent to or above 85dB(A) in a shorter time frame, silencers on plant to be considered and implemented where possible. Hearing Protection to be worn at all times. Vibration exposure to arms/ hands from tools for greater than 4 hours in a 24 hour period to be used to manufacturers specification and duration. Anti vibration gloves to be worn. Rotation schedule to be established. Exposure to whole body vibration must not exceed the exposure levels nominated for machinery or plant by the manufacturer. Use tools and methods that create less noise and vibration where possible Noise producing activities to be isolated where ever possible, Subcontractor to address the control of noise during their activities Wear appropriate ear protection and take note of signage and restricted areas Plant and equipment to be maintained i.e. exhausts ; Community complaints register to be maintained by McNab | McNab Subcontractor (PCBU) | At all times |

End of Hazard Identification Form Template