CPCCBC4012B

Read and interpret plans and specifications

THEORY EXAM INSTRUCTIONS - This exam will occur under standard assessment conditions. - An oral examination may be allowed under certain circumstances. Answers will be documented and FT003 (Oral examination declaration form) must be completed. - Participants must provide enough detail in their responses to demonstrate their knowledge and skills. - Please fill in the details below, providing your name, signature and date of assessment. - Your signature acknowledges that this is your own work. - If a participant initially answers a question incorrectly but modifies their response, they must follow the instructions below: - If a participant changes their response in writing, they must put their initials next to the written change. - If a participant changes their response verbally, this must be noted on the exam paper by the assessor.

| Participant name | Kayla Watson |
| --- | --- |
| Participant signature |  |

| Assessor name |  |
| --- | --- |
| Assessor signature |  |

| Date | Click here to enter a date. |
| --- | --- |

| Number of questions | Number of questions answered correctly | Satisfactory? |  |
| --- | --- | --- | --- |
| 87 |  | Yes | No |

Assessor Comments

Plans and specifications are used to inform which of the following processes? Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Building contracts |  |
|  |  | Development applications |  |
|  |  | Estimation |  |
|  |  | Planning |  |
|  |  | Supervisory tasks |  |
|  |  | Tenders |  |

Which compass direction is always shown on plans and drawings?

| Answer | True North |  |
| --- | --- | --- |

What information is provided on a location plan?

| Answer | The Lot number/s Deposit plan numbers North Point Scale (typically 1:5000) |  |
| --- | --- | --- |

What detail does a site or block plan contain?

| Answer | Site boundaries showing location of proposed building and existing dwellings Street frontage names Major geological and topographical features (trees) Datum points, contours, and slopes Driveways, stormwater drainage, easements and right of carriage Soil and surface water drains, pipe sizes Service runs from the house to mains – utility services location (sewer, water, gas, electricity) Details of paving and retaining walls |  |
| --- | --- | --- |

What do floor plans identify?

| Answer | Floor plan is a scaled drawing showing the above view of an area. Shows detail for the proposed structure for rooms, positions of fixtures, floor coverings. There are typically dimensions shown on the floor plan to show distances between walls, room sizes, wall lengths etc. |  |
| --- | --- | --- |

What is a landscape plan and what information might it provide?

| Answer | Landscape plan is like a floor plan but for outdoor areas. Shows details of plants, paths, garden layouts and retaining walls. They are used to plan the layout for the outdoor area. |  |
| --- | --- | --- |

What are elevation views? What do they provide to the plan reader?

| Answer | Elevation is a view you see when standing in front of that wall . Most councils will require an elevation from each side of the proposed development and the view is indicated by north, south, east, or west e.g. if the wall is on the west side of the house, it is the west elevation. Elevation plans provide the finish to external walls and the size of windows, doors, and balconies. |  |
| --- | --- | --- |

What is a section?

| Answer | A section is a vertical view through the building The direction of section lines or arrows are usually shown on the plan to help interpret the sectional view Typicall feature height of the building, position of piers, beams, & openings, floor to ceiling heights, distance between finished floor and existing/proposed ground levels, and construction details. |  |
| --- | --- | --- |

What are detailed drawings?

| Answer | A detailed drawing is a detailed description of a structure. They are large-scale drawings that show in detail parts that may be included in less detail on other plans. They show compliance with regulations and other requirements, the assembly and junctions between components, construction details etc. |  |
| --- | --- | --- |

What are specifications?

| Answer | Specifications set out the minimum standards or codes required for all structural components and provide details of building materials and fittings e.g. windows, frame material and type of glass, doors, size, material and swing etc. . |  |
| --- | --- | --- |

Explain the meaning of the abbreviations in the table below.

| Answer | Abbreviation | Meaning |  |
| --- | --- | --- | --- |
|  | ACRYL | Acrylic |  |
|  | AS | Australian Standard |  |
|  | ASSD | Assumed Datum |  |
|  | B | Basin |  |
|  | BN | Bull Nose |  |
|  | BTH | Bath |  |
|  | BWK | Brickwork |  |
|  | CB | Concrete Block |  |
|  | CF | Concrete Floor |  |
|  | CG | Clear Glass |  |
|  | CJ | Ceiling Joist |  |
|  | CORR | Corrugated |  |
|  | CWT | Cold Water Tank |  |
|  | DG | Double Glazing |  |
|  | DH | Double Hung |  |
|  | DPC | Damp Pool Course |  |
|  | DSB | Distribution switch board |  |
|  | DW | Dishwasher |  |
|  | EJ | Effluent |  |
|  | FPBD | Fibrous Plaster Board |  |
|  | FFL | Finished Floor Level |  |
|  | FHR | Fire Hose Reell |  |
|  | GPO | General Purpose Outlet |  |
|  | HBD | Hardboard |  |
|  | HTR | Heater |  |
|  | HWD | Hardwood |  |

Name each of the drawing symbol images shown in the table below.

| Answer | Name | Image |  |
| --- | --- | --- | --- |
|  | Cavery Brick Wall |  |  |
|  | Brick Veneer Wall |  |  |
|  | Timber Stud Wall |  |  |
|  | Window in Cavity Wall |  |  |
|  | Window in Single Wall |  |  |
|  | Single swing Door |  |  |
|  | Double swing Door |  |  |
|  | Sliding Door in a pocket |  |  |
|  | Sliding door exposed on wall |  |  |
|  | Socket Outlet (power) - general |  |  |

Identify what the following window symbols represent on elevations.

| Answer | Item | Drawing |  |
| --- | --- | --- | --- |
|  | Hinged at Side |  |  |
|  | Hinged at Top |  |  |
|  | Sliding Horizontally |  |  |
|  | Fixed |  |  |

What does a clouded area on a plan indicate?

| Answer | A revision or amendment has been made to that area |  |
| --- | --- | --- |

An amendment to specifications could relate to which of the following? Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Contact details of all construction crew members |  |
|  |  | Materials or products used |  |
|  |  | Methods of carrying out specified work |  |

Refer to the title block below to respond to Q16. to Q18.

Source: http://www.dtwd.wa.gov.au/sites/default/files/teachingproducts/BC1934_CCBY.PDF

On what date was an amendment made to the window details?

| Answer | 5/10/2011 |  |
| --- | --- | --- |

True or False (Tick your response): Details of the window in Q16. should also be reflected in the specifications and confirmed with the client.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

What amendment occurred on 08/11/2011?

| Answer | Bed 3 robe deleted |  |
| --- | --- | --- |

Refer to the site plan below to respond to Q19. to Q26.

What structure is being built on this site? Tick your response.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Residence |  |
|  |  | Road |  |
|  |  | Shed |  |
|  |  | Water meter |  |

What is the Lot number of this site?

| Answer | Lot 16 |  |
| --- | --- | --- |

What is the size of this lot?

| Answer | 3653m2 |  |
| --- | --- | --- |

Is the shed at the north-east of the proposed structure to be removed or retained? Tick your response.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Removed |  |
|  |  | Retained |  |

True or False (Tick your response): The sewer is at the eastern boundary of the property.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

True or False (Tick your response): The driveway is located to the south of the proposed property.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

What size is the easement on the eastern boundary of the site?

| Answer | 5m |  |
| --- | --- | --- |

True or False (Tick your response): The stormwater drain is constructed from PVC pipe.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

Refer to the contour plan below to respond to Q27. to Q28.

What is the level at the northernmost point of the site?

| Answer | 3.94 |  |
| --- | --- | --- |

Which end of the site has the lowest level? Tick your response.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | East |  |
|  |  | West |  |

Refer to the sectional drawing below to respond to Q29. to Q49.

Give a brief description about the building from the detail available in the sectional drawing. Is it the same structure proposed in Q19. to Q26. ?

| Answer | The building will be a double story residential house. It will have 4 bedrooms, a veranda at the front and back of the house and a large family room downstairs. Yes, the building is the same structure in Q19-Q26. |  |
| --- | --- | --- |

Who is the client?

| Answer | H&M Smith |  |
| --- | --- | --- |

Where is this project to be built?

| Answer | Lot 16 Glenvalley Road, Richmond |  |
| --- | --- | --- |

What are the contact details for the draftsman?

| Answer | Joe Foreman Phone: 03 9514 2311 Mobile: 0413 113 113 18 Fairview Road, Richmond 3216 |  |
| --- | --- | --- |

What sheet number is this drawing?

| Answer | 5 of 7 |  |
| --- | --- | --- |

What timber is required for loading and non-load bearing walls?

| Answer | 45x90 for load bearing and 35x90 for non-load bearing |  |
| --- | --- | --- |

What spacing is recommended for common studs?

| Answer | 600mm max spacing |  |
| --- | --- | --- |

What type of roof truss is to be installed?

| Answer | Gangnail roof trusses |  |
| --- | --- | --- |

What centres are the trusses positioned at?

| Answer | 900 spacing |  |
| --- | --- | --- |

What size are the veranda posts?

| Answer | 90x90 |  |
| --- | --- | --- |

What timber species are the veranda posts to come from?

| Answer | Oregon |  |
| --- | --- | --- |

What pitch is the roof?

| Answer | 30° |  |
| --- | --- | --- |

What size are the noggings and at what spacing?

| Answer | 35x70 with 1350 max spacing |  |
| --- | --- | --- |

What size are the ceiling joists?

| Answer | 240mm |  |
| --- | --- | --- |

What timber is used for roof battens and what spacing?

| Answer | Pine Battens with 900mm max spacing |  |
| --- | --- | --- |

What thickness is the plasterboard sheeting?

| Answer | 10mm |  |
| --- | --- | --- |

What type of flooring is to be installed?

| Answer | 15mm Plywood floorings |  |
| --- | --- | --- |

What is the roof cladding material?

| Answer | Colourbond Corrugated metal roofing |  |
| --- | --- | --- |

Where will you find information about the footings?

| Answer | Engineers drawing |  |
| --- | --- | --- |

What do the letters ‘DAR’ refer when timber ordering?

| Answer | Dressed-All-Round – smoothed on all sides and edges generally rounded |  |
| --- | --- | --- |

Explain what ‘240 x 63 LVL’ refers to on the sectional drawing. What do the letters ‘LVL’ refer to?

| Answer | Refers to the length and width of the Starting joist LVL is the type of material to be used – LVL timber refers to laminated veneer lumber |  |
| --- | --- | --- |

Refer to the drawing below to respond to Q50. to Q65.

What is the number of the drawing?

| Answer | 2 |  |
| --- | --- | --- |

What is the Job Number?

| Answer | 00R-112 |  |
| --- | --- | --- |

What is the scale of the drawing?

| Answer | 1:100 |  |
| --- | --- | --- |

What does the drawing show?

| Answer | Various elevations of a house, specifically the north, south, east, and west sides of the building. |  |
| --- | --- | --- |

Which view shows the front of the house?

| Answer | East Elevation |  |
| --- | --- | --- |

How many motor vehicles can be parked in the garage?

| Answer | 2 |  |
| --- | --- | --- |

What are the window and doors made from?

| Answer | Aluminium Framed windows and timber doors |  |
| --- | --- | --- |

What is the size of the eaves?

| Answer | 450 |  |
| --- | --- | --- |

What are the eaves lined with?

| Answer | Hardflex |  |
| --- | --- | --- |

What type of roof structure is the roof built from?

| Answer | Timber framed roof trusses |  |
| --- | --- | --- |

Which Clause of the NCC should be read for more detail about roof structure in Q59. ?

| Answer | Part 3.4 |  |
| --- | --- | --- |

Which Australian Standard should be read for more detail about the roof structure in Q59. ?

| Answer | AS 1684 |  |
| --- | --- | --- |

What is the roof cladding materials made from?

| Answer | Concrete roof tiles |  |
| --- | --- | --- |

Which Clause of the NCC should be read for more detail about the roof cladding materials in Q62. ?

| Answer | Part 3.5.1 |  |
| --- | --- | --- |

Which Australian Standard should be read for more detail about the roof cladding materials in Q62. ?

| Answer | AS 1562 |  |
| --- | --- | --- |

What is the roof pitch?

| Answer | 22.5° |  |
| --- | --- | --- |

Identify one (1) regulatory body you may need to submit plans to for approval?

| Answer | City Council e.g. Brisbane City Council |  |
| --- | --- | --- |

Briefly describe the approval process and timeframes for an approval through the regulatory body in Q66.

| Answer | Buiilding approvals require assessment against the Building Code of Australia and Qld’s building and plumbing regulation. You can lodge an application online, in person, or through the mail. From there the application is reviewed. Then the application is sent by the applicant to all relevant external referral agencies to seek advice, approvals or permits. Council may request further information is necessary. |  |
| --- | --- | --- |

What is a prime cost (PC) item? Give an example.

| Answer | A PC item is an amount of money included in a contract sum to purchase a specified item like tiles, taps, doors, bathroom fittings etc. An agreed estimated amount is included at contract signing but specific products are not selected until a later stage. A builder must estimate the cost of such items at or above the lowest amount these items could reasonably cost. E.g. plumbing contractor allows for basins in the home. If the client specifies a more expensive basin than allowed for in the PC items schedule in the contract, then a variation will be required to cover the cost difference e.g. a $200 basin is specified instead of the $100 basin allowed for in the PC. |  |
| --- | --- | --- |

What is a provisional sum (PS) item? Give an example.

| Answer | A Provision Sum is an amount of money included in the contract sum to cover work and/or materials, not detailed when entering the contract. Builders will typically include a PS for site work costs. The builder is legally required to take reasonable steps to ensure an accurate site works cost estimate but unforeseen events mean that the provisional sum is exceeded which is added to the contract sum. E.g. The site appears to be sandy and clean with minimal siteworks required, however there may be large rocks concealed below the surface that only become apparent once siteworks commence. |  |
| --- | --- | --- |

Refer to the Bill of Quantities below to respond to Q70. to Q76.

Source: http://organizedbuilders.com.au/sites/default/files/BOQ-Summary.pdf

True or False (Tick your response): This Bill of Quantities represents a job for a two-storey house.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

Identify two (2) materials being used for the floors?

| Answer | Carpet and tiles |  |
| --- | --- | --- |

True or False (Tick your response): A landscaper has been contracted for this job.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

How much does the contracted plasterer cost?

| Answer | $12,900 |  |
| --- | --- | --- |

How much do the kitchen appliances cost?

| Answer | $2,700 |  |
| --- | --- | --- |

What percentage of the total cost does the fixing stage represent of the building contract?

| Answer | 23% |  |
| --- | --- | --- |

What percentage of the total cost does the completion stage represent of the building contract?

| Answer | 17% |  |
| --- | --- | --- |

Refer to the Purchase Order below to respond to Q77. to Q82.

Source: http://organizedbuilders.com.au/sites/default/files/Databuild-Bill-of-Quantities.pdf

How many Pine F5 90 x 45 studs (2.4m long) are required?

| Answer | 100 |  |
| --- | --- | --- |

If the material in Q77. increased in cost to $2.75 each, what would be the total cost for this material?

| Answer | $275 |  |
| --- | --- | --- |

How many sheet of ply are required in total (i.e. all sizes)?

| Answer | 13 Sheet in total 2400x1200 4mm Ply x3 sheets 2700 x 1200 4mm Ply x6 sheets 300 x 1200 4mm Ply x4 sheets |  |
| --- | --- | --- |

What material has the highest total cost?

| Answer | Pine 90x45 mgp10 |  |
| --- | --- | --- |

How many metres are required of the material in Q80. ?

| Answer | 322.5m |  |
| --- | --- | --- |

What beam is identified as being a better option than F17?

| Answer | E14 |  |
| --- | --- | --- |

Refer to the labour chart below to respond to Q83. to Q86.

Source: https://www.archsd.gov.hk/media/89170/sor2013_vol1_builders_works_2013_11_04.pdf

Based on the hourly rates given, which is the least expensive trade to contract?

| Answer | Asphalter (road construction) |  |
| --- | --- | --- |

If the trade in Q83. were to be contracted for two (2) hours’ work, how much would this cost?

| Answer | $214.80 |  |
| --- | --- | --- |

Based on the hourly rates given, which is the most expensive trade to contract?

| Answer | Bar bender and fixer |  |
| --- | --- | --- |

If the trade in Q85. were to be contracted for six (6) hours’ work, how much would this cost?

| Answer | $1285.80 |  |
| --- | --- | --- |

Name the building legislation (Act and Regulations) that apply within your jurisdiction and should be referenced when reading plans and building contracts.

| Answer | Building Act 1974 Building Regulation 2006 Also refer to the National Construction Code (NCC) |  |
| --- | --- | --- |