Non Conformance for 009 - 4857 - CEDAR RD & MACARTHUR AVE NCR-000001

| NCR Number: | NCR-000001 |
| --- | --- |
| Status: | Submitted |
| Location: | 009 - 4857 - CEDAR RD & MACARTHUR AVE |
| Date Open: | 08-Oct-2021 |
| Date Closed: |  |
| Approved By: |  |
| Owner: | Kyle Carlson (Site Engineer) |
| Area: | CED - Cedar Rd |
| Type: | Minor |
| Specification Clause / Reference: | Kerb on Cedar Rd being poured out of tolerance for levels |
| Lot Number: | ROW-KERB-ES-0001 |
| Title: | 4857 CMNH - Kerb NCR - Cedar Rd - CH30 to CH210 |
| Description: | We’ve had an issue on site with survey which has resulted in a portion of kerb on Cedar Rd being poured out of tolerance for levels. We’ve had surveyre-check the pickup and it has come back the same, so we have eliminated apick-up error. We’ve highlightedthe areas on the below snapshots which are the worst - generally the kerb islower than design. It doesnâ€™t appear to be ponding so the grade seems okay. I propose that weadjust the crown/verge heights to match in with the as-constructed heights. If you are in agreeance, I will have our surveyor create the model and provideback to you for review. I will raise an NCR in our system for this, but I just wanted to ensure You’re okay with the proposal first. |

NCR Recipients

To:

| Name | Company | Title |
| --- | --- | --- |

CC:

| Name | Company | Title |
| --- | --- | --- |

Share Point Documents

| Name | Title / Description | Revision |
| --- | --- | --- |

Attachments

| Name | Attached | Size |
| --- | --- | --- |

Corrective or Improvement Actions

| Number | Description | Raised By | Date Raised | Status |
| --- | --- | --- | --- | --- |