Cost Estimation Sheet

| Participant name | Lucy Muston |
| --- | --- |
| Date | 08/04/2025 |

Details of Building and Construction Project

| Name of project | Calvary Christian College – New Prep Building |
| --- | --- |
| Type of project | Residential building Commercial building |
| State/Territory | Queensland |
| Plans accessed for material estimates | Yes |

Identify internal linings, external cladding and roof materials from plans and specification

| Item | Description |
| --- | --- |
| Internal linings | ACP Item: Acoustic Ceiling Panel Product Name: Quiet Space Panel Brand: Autex Material: Thermally Bonded Polyester Thickness: 25mm Colour: Nude White Sheet Size: 2400 x 1200 (cut into 1200 x 1200 panels) Description: The Acoustic Ceiling Panels will be directly fixed to the ceiling lining in accordance with the manufacturer’s recommendations. This product helps in sound absorption, contributing to a quieter indoor environment. CLT-01 Item: Ceiling Lining – Type 01 Product Name: Plasterboard – Flush Set Brand: Gyprock or Equivalent Material: Standard Grade Commercial Plasterboard Thickness: 13mm Finish: Painted Sheet Size: To suit application Description: The plasterboard will be installed with a Level 5 finish. Junctions between plasterboard surfaces will be square set, and Rondo P50 shadow line junctions will be used for connections to non-plasterboard surfaces. CLT-02 Item: Ceiling Lining – Type 02 Wet Areas Product Name: Aquachek Brand: Gyprock or Equivalent Material: Moisture Resistant Plasterboard Thickness: 13mm Finish: Painted Sheet Size: To suit application Description: The ceiling lining in wet areas will have a Level 5 finish with square set junctions to other plasterboard surfaces and Rondo P50 shadow line junctions to non-plasterboard surfaces for a smooth transition. PBL Item: Plasterboard Wall Lining Product Name: Plasterboard – Flush Set Brand: Gyprock or Equivalent Material: Standard Grade Plasterboard Thickness: 10mm Colour: Painted Sheet Size: To suit application Description: The plasterboard will be finished to a Level 5 finish, with square set junctions to other plasterboard surfaces and Rondo P50 shadow line junctions to non-plasterboard surfaces for seamless integration. VBL Item: Villaboard Wall Lining – Wet Areas Product Name: Villaboard – Flush Set Brand: James Hardie Material: Fibre Cement Thickness: 9mm Colour: Painted Sheet Size: To suit application Description: The Villaboard will be installed with square set junctions to other plasterboard surfaces, providing a durable and water-resistant wall lining in wet areas. |
| External cladding | 9mm FC cladding to external walls on and including battening system. External cladding / battens and flashing to external bulkheads. |
| Roofing materials | Main Roof - 0.48mm Colorbond Ultra Spandeck roof sheeting - Safety wire under roof sheeting - Metal CL4 Colour coded fasteners - Anticon 60mm R1.3 Insulation HD - Thermoseal 733 Sarking - Topspan 40 .75 Battens to Steel Framed Trusses - 0.55mm Colorbond Ultra Expansion Flashing - 0.55mm Colorbond Ultra Flashings to the roof - 250mm Half Round Colorbond Ultra Eaves Gutter, brackets, stop ends and droppers - 150mm dia PVC downpipe offsets to Eaves gutter - dia Galvanised CHS downpipe Shoes to Eaves gutter - Freespan Danpalon 16mm Polycarbonate roof sheeting system - Access equipment and Cranage for roofing installation - 0.48mm Colorbond Ultra Spandeck Vertical Wall Cladding - 0.55mm Colorbond Ultra Flashings to wall cladding - Topspan 40 .75 Battens to wall cladding - Thermoseal 733 Medium Duty Sarking behind wall cladding - Freespan Danpalon 16mm Multicell Polycarbonate wall cladding system Lower Roof - 0.48mm Colorbond Ultra Spandeck roof sheeting - Safety wire under roof sheeting - Metal CL4 Colour coded fasteners - Anticon 60mm R1.3 Insulation HD - Thermoseal 733 Sarking - 0.55mm Colorbond Ultra Flashings to the roof - 150mm Half Round Colorbond Ultra Eaves Gutter, brackets, stop ends and droppers - 100mm dia PVC downpipe offsets to Eaves gutter - 100mm dia Galvanised CHS downpipe Shoes to Eaves gutter - Freespan Danpalon 16mm Polycarbonate roof sheeting system - Access equipment for roofing installation Walkway Roof - .48mm Colorbond Ultra Spandeck roof sheeting - Safety wire under roof sheeting - Metal CL4 Colour coded fasteners - Thermoseal 733 Sarking - .55mm Colorbond Ultra Flashings to the roof - 150mm Half Round Colorbond Ultra Eaves Gutter, brackets, stop ends and droppers - 100mm dia PVC downpipe offsets to Eaves gutter - 100mm dia Galvanised CHS downpipe Shoes to Eaves gutter - Access equipment for roofing installation |

Opinion of costs - for one (1) room identify costs of three (3) materials needed.

| Item | Description (including size, thickness) | Cost per Unit | Quantity | Total cost | Construction method |
| --- | --- | --- | --- | --- | --- |
| 1 | Classroom 1- Flooring - vinyl sheet VIN-01 Tarkett Acczent Excellence 80 – colour Oak Tree Beige, thickness – Height: 10.71 metres. | $35.00 per m2 | 110.4m2 | $3,864.00 | The construction method for Tarkett Acczent Excellence 80 vinyl sheet flooring involves a glue-down installation technique. This method ensures the vinyl sheets are securely adhered to the prepared subfloor using a high-performance adhesive, offering enhanced durability and stability. The seams are typically heat-welded, providing a seamless, watertight finish that improves the flooring’s resistance to moisture and wear. |
| 2 | Classroom 1- Painting - Painting – Type 31, Colour: Winter Orange S11H8 | $30.00 per litre | 212m2 | $6360.00 | The construction method for applying Type 31 Winter Orange S11H8 paint involves surface preparation, including cleaning and repairing any imperfections. A primer is applied, if necessary, followed by two coats of paint using a brush, roller, or spray system. The paint is left to dry between coats to achieve a smooth, durable finish. |
| 3 | Basin – type 03 - Brand Caroma | $507.32 | 1 | $507.32 | The construction method for the installation of the Caroma Type 03 Basin begins with preparing the area by ensuring it is clean and verifying that all required plumbing connections, including water supply and drainage, are correctly positioned. The basin is then mounted using either a wall bracket or pedestal base, ensuring it is securely fixed and level with proper alignment. Following the mounting, the water supply lines are connected to the basin’s inlet valves, and the drain is sealed to prevent any leaks. Once the basin is installed, a thorough leak test is conducted to ensure proper functioning of all connections. Finally, the installation area is cleaned, and the basin is left ready for use. |

Identify two (2) Prime Cost (PC) items and two Provisional Sum items

| Prime Cost items | Provisional Sum items |
| --- | --- |
| Nil | Building Signage - $20,000.00 |
|  | Fixed playground equipment and soft fall base layer - $ 100,000.00 |
|  | Irrigation - $ 32,000.00 |