CPCCBC4029B

Apply construction information to the sales process

ASSESSMENT SUMMARY

The assessment activities for CPCCBC4029B Apply construction information to the sales process cover all aspects of the unit of competency required. To demonstrate competence in this unit, a participant must undertake all tasks and complete them satisfactorily and consistently. For Apprentices and Trainees, this includes employer completion of the Employer Record Book (ERB).

Participants must read all the information given before starting any assessment task. The assessment tasks are intended to be equitable, fair and flexible. If a participant has any questions or requires alternative arrangements to be made such requests should be directed to their Trainer/Assessor, who should note such arrangements in the ‘Assessor Comments’ on the affected assessment. If a participant feels they are not yet ready to be assessed, they should discuss their options with their Trainer/Assessor.

To satisfactorily complete the assessment, the participant is required to answer all questions correctly and demonstrate their skills to industry standards. If a participant does not answer some questions or perform some tasks, they will be deemed ‘Not Satisfactory’ for the task and ‘Not Yet Competent’ for the unit of competency. The Trainer/Assessor is able to provide the participant with additional information on interpreting the assessment outcomes and provide guidance on the participant’s options. Should a participant still be deemed ‘Not Satisfactory’ they will have the opportunity to undertake a supplementary assessment or appeal the result.

By signing the cover page for each assessment item, the participant acknowledges they understand the assessment instructions and requirements and consent to being assessed. The participant also verifies and assures the RTO that the work submitted is their own work.

| Participant Name | ROBBIE MEADOWS |
| --- | --- |

| Task | Assessment Results |  |
| --- | --- | --- |
| Theory Exam | Satisfactory | Not Yet Satisfactory |
| Practical Assessment | Satisfactory | Not Yet Satisfactory |
| Outcome for Unit of Competency | Competent | Not Yet Competent |

| Assessor Name |  |
| --- | --- |
| Assessor Signature |  |
| Unit Competence Determination Date | Click here to enter a date. |

CPCCBC4029B

Apply construction information to the sales process

THEORY EXAM INSTRUCTIONS - This exam will occur under standard assessment conditions. - An oral examination may be allowed under certain circumstances, whereby the Participant’s answers will be documented by a party other than the Participant. This must be acknowledged in writing (e.g. FT001 and Assessor Comments) - Participants must provide enough detail in their responses to demonstrate their knowledge and skills. - Please fill in the details below, providing your name, signature and date of assessment. - Your signature acknowledges that this is your own work. - Assessors are to complete marking in pen of a different colour to the Participant. - If a Participant initially answers a question incorrectly but modifies their response, they must follow the instructions below: - If a Participant changes their response in writing, they must put their initials next to the written change. - If a Participant changes their response verbally, this must be noted on the exam paper by the assessor.

| Participant name | ROBBIE MEADOWS |
| --- | --- |
| Participant signature |  |
| Date | 26/12/2019 |

| Assessor name |  |
| --- | --- |
| Assessor signature |  |
| Date | Click here to enter a date. |

| Number of questions | Number of questions answered correctly | Task outcome (Tick) |  |
| --- | --- | --- | --- |
| 18 |  | Satisfactory | Not Yet Satisfactory |

ASSESSOR Comments

Outline the circumstances in which a building licence must be obtained before carrying out building work.

| Answer | Any building work valued over $3,300 Building work valued over $1,100 where it involves Hydraulic Services Design Building work of any value where it involves: Drainage Plumbing and Drainage Gas Fitting Termite Management – Chemical Fire Protection Completed Residential Building Inspection Building Design – Low Rise, Medium Rise and Open Site Classification |  |
| --- | --- | --- |

Describe the steps involved in organising electricity supply to site. Provide approximate timeframes for this process.

| Answer | You contact your electrical contractor to advise if supply is available Should the premise not have supply available, the electrical contractor can complete an enquiry on the client’s behalf The Distribution Network Service Provider conducts the necessary network investigations, and provide an estimate of potential costs and timeframes within 20 business days. If there is supply available, the electrical contractor will need to enter into a Network Connection Agreement with the Distribution Network Service Provider If the application is not expedited (e.g. by agreeing to the Model Terms and Conditions for Deemed Standard Connection Contracts), the Distribution Network will advise of a Network Connection Offer within 10 business days After agreeing to the connection offer, the electrical contractor will prepare the premises for connection The electrical contractor submits an Electrical Work Request (EWR) to the Distribution Network Service Provider once the work is complete. A copy of the EWR will be sent to the client’s preferred electricity retailer. Once all forms are received, the Distribution Network Service Provider will visit the premises to perform the requested work. The Distribution Network Service Provider must adhere to timeframes for new connections (e.g. Energex will attend the property within 5 business days) |  |
| --- | --- | --- |

True or False (Tick your response): Staged approval is one type of approval that may be sought for the building approval stage.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

True or False (Tick your response): Building provision approvals are issued by the local government (e.g. Brisbane City Council).

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

Describe the steps involved in organising water supply to site. Provide approximate timeframes for this process.

| Answer | Once you have a state government or local council Development Approval, you can apply for a Queensland Urban Utilities Water Approval. Firstly, identify the correct application form for your needs. Ensure a properly made application using the Urban Utilities Check for Completeness guide. Submit and pay for a Standard Connection (e.g. amalgamations, single lot residential developments, or one-into-two lot subdivisions) or Property and Network Connection application (e.g. amalgamations, staged developments and industrial or commercial developments that generate trade waste). Where developers are proposing to connect to water and sewerage services, councils will require evidence that a connection is available. Generally the evidence required, and therefore the relative timing of the two approvals, will depend on the complexity of the proposed development and the likely availability of services. Connections for simple residential developments are usually completed within five (5) days |  |
| --- | --- | --- |

Identify the Volume and Parts of the NCC which outline energy efficiency requirements for Classes 1 and 10, and Classes 2 to 9.

| Answer | Classes 1 and 10 | Classes 2 to 9 |  |
| --- | --- | --- | --- |
|  | NCC Volume Two, Parts 3.12.0 – 3.12.5 | NCC Volume One, Part J1-7 and J10 |  |

Describe the long-term benefits of sustainable building solutions. List one (1) environmental benefit and one (1) benefit for the client.

| Answer | Topics | Definition |  |
| --- | --- | --- | --- |
|  | Environmental benefit | Sustainable solutions can reduce the size of any equipment needed and reduce the amount of energy the occupants use. Consequently, this can reduce the amount of GHG emissions attributable to a dwelling’s artificial heating and cooling. |  |
|  | Client benefit | Client benefits are achieved by having a dwelling designed with greater levels of energy efficiency. The house’s interior is likely to stay warmer in cold weather and cooler in hot weather. When the internal environment conditions are tolerable for the occupants, it should minimise the need for artificial cooling and/or heating, saving money. |  |

Match the insurance types and definitions by writing the appropriate letter next to each number in the centre column.

| Answer | Insurances |  | Answers | Definition |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1 | Builder’s All Risk | 1 -B | A | Covers personal injury or property damage. |  |
|  | 2 | Indemnity | 2 -C | B | Covers accidental risks of physical loss or damage during construction, third party liabilities and advance loss of profits. |  |
|  | 3 | Public Liability | 3 -A | C | Covers costs where a contractor has caused building errors or omissions. |  |
|  | 4 | Workers’ Compensation | 4 -D | D | Covers death, injury or illness of employees. |  |

True or False (Tick your response): Insurance should be taken out by the contractor prior to starting construction.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

Select and describe five (5) trades involved in the construction process.

| Answer | Trade | Description |  |
| --- | --- | --- | --- |
|  | Electrician | Arranges electricity to be connected; places wiring for lighting and power cables; Fixes switches, installs power points, wires electrical systems |  |
|  | Gas fitting | Places gas pipes to appliances |  |
|  | Bricklaying | Installs bricks, including sub base and piers, and brick walls |  |
|  | Carpenters | Installs frames, trusses, soffits, doors and mouldings |  |
|  | Painters | Paint walls, ceilings, mouldings etc. |  |

Order the following steps from 1 to 10 to identify an appropriate construction sequence.

| Answer | Step Number | Steps |  |
| --- | --- | --- | --- |
|  | 3 | Floor frame |  |
|  | 5 | Windows and external door frames |  |
|  | 7 | Wall insulation |  |
|  | 1 | Set out |  |
|  | 9 | Painting |  |
|  | 6 | Electrical wiring |  |
|  | 2 | Plumbing of waste pipes |  |
|  | 8 | Internal linings |  |
|  | 10 | Landscaping |  |
|  | 4 | Wall frames |  |

For each of the main stages of construction listed below, identify the main trades involved and an approximate duration of each stage for a standard.

| Answer | Stage | Description |  |  |
| --- | --- | --- | --- | --- |
|  | 1: Clearing of the site | Trades | Earthmovers |  |
|  |  | Duration | Varies, but 1 day for a clear site |  |
|  | 2: Slab or base stage | Trades | Concretor |  |
|  |  | Duration | 1 – 2 weeks |  |
|  | 3: Frame stage | Trades | Carpenters |  |
|  |  | Duration | 3-4 weeks |  |
|  | 4: Lockup stage | Trades | Carpenters, Electricians, Plumbers, Gas Fitters, Joinery Trades, Gyprockers, Plasterers, Roofers |  |
|  |  | Duration | 3-4 Weeks |  |
|  | 5: Fit-out or fixing stage | Trades | Carpenters, Joiners, Tilers, Waterproofers |  |
|  |  | Duration | 5-6 Weeks |  |
|  | 6: Practical completion stage | Trades | Painters, Silicone Installer, Carpenters, Plumbers, Electricians, Gas Fitter |  |
|  |  | Duration | 7-8 Weeks |  |

Identify five (5) factors that can affect the duration of each construction stage.

| Answer | Availablity of materials Availability of trades Client variations Site conditions Inclement weather |  |
| --- | --- | --- |

Describe how each of the following factors can affect construction times.

| Answer | Factor | Effect |  |
| --- | --- | --- | --- |
|  | Changed conditions | If the requirements of the job are altered, this will change the initially projected timeframes, especially where additional materials and labour must be organised. |  |
|  | Labour availability | Most tasks that are not completed on time push back the trades scheduled to follow, so when labour is unavailable the schedule will be pushed out. |  |
|  | Material availability | Without the required material, tasks can’t be completed. |  |
|  | Weather | Inclement weather will impact trades ability to complete tasks, whether it’s rain, wind or heat. |  |

Describe the site establishment requirements for each of the points listed.

| Answer | Requirement | Description |  |
| --- | --- | --- | --- |
|  | Earthworks | Contact Dial Before You Dig before undertaking any excavation to ensure that services will not be damaged during construction. |  |
|  | Electricity | Electricity may need to be disconnected from work zones prior to construction and temporary utilities installed for the duration of the construction process. |  |
|  | Neighbours | Provide advance notice to neighbours of intended start dates and site contact numbers |  |
|  | Waste disposal | Set up a designated area for rubbish bins and waste removal. Check if permits are required to place bins on the street |  |
|  | Water connection | Book in a plumber to perform any required disconnections prior to any demolition taking place |  |

Identify the section of the Work Health and Safety Regulations that outlines the site establishment requirements listed.

| Answer | Regulation section | Requirement |  |
| --- | --- | --- | --- |
|  | Part 3.2, Division 2, Section 40a | The layout of the work area must allow for persons to enter, exit and move about without risk to health and safety |  |
|  | Part 3.2, Division 2, Section 41(1) | Adequate facilities must be provided for workers, including toilets, drinking water, washing facilities and eating facilities. |  |
|  | Part 6.4, Section 308 A and B | A principal contractor for a construction project must install signs outside the work area that shows their name and telephone contact numbers and site office address. |  |

True or False (Tick your response): The owner is able to access the site at any time.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

Which of the following parties should the client contact to access the site? Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Contractor |  |
|  |  | Local Government |  |
|  |  | Industry Regulator (e.g. Queensland Building and Construction Commission) |  |
|  |  | Subcontractors |  |

CPCCBC4029B

Apply construction information to the sales process

PRACTICAL ASSESSMENT INSTRUCTIONS - The following practical assessment will require the Participant to complete work related activities. - The Assessor will conduct observations of the Participant’s work to determine whether they display sufficient practical skills and knowledge. - Please fill in the details below – providing your name, signature and date of assessment. - The Participant’s signature acknowledges that the Assessor provided clear instructions and the Participant understood what was required to complete this assessment. - The Assessor may substitute a Practical Task with an alternate task of equal value. - Alternate tasks must be fully documented by the assessor in the Assessor Comments area.

| Participant name | ROBBIE MEADOWS |
| --- | --- |
| Participant signature |  |
| Date | 26/12/2019 |

| Assessor name |  |
| --- | --- |
| Assessor signature |  |
| Date | Click here to enter a date. |

| Task | Task outcome (Tick) |  |
| --- | --- | --- |
|  | Satisfactory | Not Yet Satisfactory |
| Task 1 (Occasion 1) |  |  |
| Task 1 (Occasion 2) |  |  |

Assessor Comments

CPCCBC4029B

Apply construction information to the sales process

| Practical Task 1 (2X) |  |  |  |
| --- | --- | --- | --- |
| Description of task: On two (2) occasions, a suitable observer is to verify the participant’s ability to apply construction information during the sales process in order to develop the customer’s knowledge of construction processes and the effects on the contract and building timelines. The participant is to gather evidence of how they have completed each component of the Practical Task, which is to be submitted with this Practical Assessment sheet. Evidence to be submitted may include: - RFI and relevant correspondence with client - A variation, price and acceptance by the client The observer is to provide detailed comments on the tasks completed by the participant. The assessor is to verify the observer’s comments in the Assessor Comments section, and provide the Practical Assessment outcome on the front cover of this document. |  |  |  |
| Resources required: Two (2) sales contracts. |  |  |  |
| Assessment of task |  |  |  |
| Name of participant: | ROBBIE MEADOWS |  |  |
| Name of observer: | ROSE-ANNE RACITI |  |  |
| Role of observer (Confirm suitability to sign) |  |  |  |
| Email address of observer | allieoopsie@outlook.com |  |  |
| Signature of observer: |  |  |  |
| Date: | 26/12/2019 |  |  |
| When completing the task, did the participant: |  | Tick if satisfactorily completed. Provide comments if left unticked. |  |
|  |  | 1st Occasion | 2nd Occasion |
| - Identify and explain to the customer the approvals required before starting construction, including: - Statutory approvals - Sustainability requirements - Insurance covers |  |  |  |
| - Identify and explain to the customer the stages and timeframes of construction, including: - Main trade components - Construction process sequence (including approximate duration) |  |  |  |
| - Identify and explain to the customer potential industry conditions that can affect construction times. |  |  |  |
| - Identify and explain to the customer site establishment requirements before starting construction. |  |  |  |
| - Establish site access arrangements with the client. |  |  |  |
| - Effectively communicate using the following methods: - Verbally using language/concepts appropriate to cultural differences - Non-verbal communication - Active listening and responding to client questions |  |  |  |
| The participant’s performance was: Satisfactory Not Satisfactory |  |  |  |

observer Comments