CPCCBC4012B

Read and interpret plans and specifications

THEORY EXAM INSTRUCTIONS - This exam will occur under standard assessment conditions. - An oral examination may be allowed under certain circumstances. Answers will be documented and FT003 (Oral examination declaration form) must be completed. - Participants must provide enough detail in their responses to demonstrate their knowledge and skills. - Please fill in the details below, providing your name, signature and date of assessment. - Your signature acknowledges that this is your own work. - If a participant initially answers a question incorrectly but modifies their response, they must follow the instructions below: - If a participant changes their response in writing, they must put their initials next to the written change. - If a participant changes their response verbally, this must be noted on the exam paper by the assessor.

| Participant name | Debbie Cremer |
| --- | --- |
| Participant signature |  |

| Assessor name |  |
| --- | --- |
| Assessor signature |  |

| Date | 8/04/2019 |
| --- | --- |

| Number of questions | Number of questions answered correctly | Satisfactory? |  |
| --- | --- | --- | --- |
| 87 |  | Yes | No |

Assessor Comments

Plans and specifications are used to inform which of the following processes? Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Building contracts |  |
|  |  | Development applications |  |
|  |  | Estimation |  |
|  |  | Planning |  |
|  |  | Supervisory tasks |  |
|  |  | Tenders |  |

Which compass direction is always shown on plans and drawings?

| Answer | North |  |
| --- | --- | --- |

What information is provided on a location plan?

| Answer | Shows the site and where it is located on the street |  |
| --- | --- | --- |

What detail does a site or block plan contain?

| Answer | Shows specific detail of the block on which the building will be carried out |  |
| --- | --- | --- |

What do floor plans identify?

| Answer | Shows specific detail of the building, e.g. separate rooms, fixtures, floor coverings, windows etc |  |
| --- | --- | --- |

What is a landscape plan and what information might it provide?

| Answer | Landscape plan is a plan showing the outside landscape which is to be constructed. It would show details of plants, pathways, garden beds, walls and fencing etc |  |
| --- | --- | --- |

What are elevation views? What do they provide to the plan reader?

| Answer | Elevation views are drawings of the north, south, east and west faces of the structure. They provide basic details of the façade, e.g. cladding, window and doors |  |
| --- | --- | --- |

What is a section?

| Answer | A section is a cross section of the structure showing things like heights and thicknesses of components. It will show the relationship between spaces. i |  |
| --- | --- | --- |

What are detailed drawings?

| Answer | Detailed drawings show the particular method of construction of finish. More detailed that the other drawings. |  |
| --- | --- | --- |

What are specifications?

| Answer | Specifications are the details of all the different components and processes of the build. They will set out details of the materials and techniques to be used and the standards and codes to be observed Specifications will be read in conjunction with drawings, plans and any other building documents |  |
| --- | --- | --- |

Explain the meaning of the abbreviations in the table below.

| Answer | Abbreviation | Meaning |  |
| --- | --- | --- | --- |
|  | ACRYL | Acrylic |  |
|  | AS | Australian Standard |  |
|  | ASSD | Assumed Datum |  |
|  | B | Basin |  |
|  | BN | Bullnose |  |
|  | BTH | Bath |  |
|  | BWK | Brickwork |  |
|  | CB | Concrete Block |  |
|  | CF | Concrete Floor |  |
|  | CG | Clear Glass |  |
|  | CJ | Ceiling Joist |  |
|  | CORR | Corrugated |  |
|  | CWT | Cold Water Tank |  |
|  | DG | Double Glazing |  |
|  | DH | Double Hung |  |
|  | DPC | Damp Proof Course |  |
|  | DSB | Distribution Switchboard |  |
|  | DW | Dishwasher |  |
|  | EJ | Expansion Joint |  |
|  | FPBD | Fibrous Plaster Board |  |
|  | FFL | Finished Floor Level |  |
|  | FHR | Fire Hose Reel |  |
|  | GPO | General Purpose Outlet |  |
|  | HBD | Hardboard |  |
|  | HTR | Heater |  |
|  | HWD | Hardwood |  |

Name each of the drawing symbol images shown in the table below.

| Answer | Name | Image |  |
| --- | --- | --- | --- |
|  | Cavity Brick |  |  |
|  | Brick Veneer |  |  |
|  | Timber Stud |  |  |
|  | Window in Cavity Wall |  |  |
|  | Window in Single Wall |  |  |
|  | Single Door |  |  |
|  | Double Door |  |  |
|  | Sliding Cavity Door |  |  |
|  | Sliding Exposed Door |  |  |
|  | Power Socket Outlet |  |  |

Identify what the following window symbols represent on elevations.

| Answer | Item | Drawing |  |
| --- | --- | --- | --- |
|  | Side Hinging Window |  |  |
|  | Top Hinging Window |  |  |
|  | Horizontal Sliding Window |  |  |
|  | Fixed Winow |  |  |

What does a clouded area on a plan indicate?

| Answer | The clouded area on a new revision of a drawing indicates the change or amendment |  |
| --- | --- | --- |

An amendment to specifications could relate to which of the following? Tick all that apply.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Contact details of all construction crew members |  |
|  |  | Materials or products used |  |
|  |  | Methods of carrying out specified work |  |

Refer to the title block below to respond to Q16. to Q18.

Source: http://www.dtwd.wa.gov.au/sites/default/files/teachingproducts/BC1934_CCBY.PDF

On what date was an amendment made to the window details?

| Answer | 5/10/2011 |  |
| --- | --- | --- |

True or False (Tick your response): Details of the window in Q16. should also be reflected in the specifications and confirmed with the client.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

What amendment occurred on 08/11/2011?

| Answer | Bed 3 robe deleted |  |
| --- | --- | --- |

Refer to the site plan below to respond to Q19. to Q26.

What structure is being built on this site? Tick your response.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Residence |  |
|  |  | Road |  |
|  |  | Shed |  |
|  |  | Water meter |  |

What is the Lot number of this site?

| Answer | 16 |  |
| --- | --- | --- |

What is the size of this lot?

| Answer | 3653 square metres |  |
| --- | --- | --- |

Is the shed at the north-east of the proposed structure to be removed or retained? Tick your response.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | Removed |  |
|  |  | Retained |  |

True or False (Tick your response): The sewer is at the eastern boundary of the property.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

True or False (Tick your response): The driveway is located to the south of the proposed property.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

What size is the easement on the eastern boundary of the site?

| Answer | 5 metres |  |
| --- | --- | --- |

True or False (Tick your response): The stormwater drain is constructed from PVC pipe.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

Refer to the contour plan below to respond to Q27. to Q28.

What is the level at the northernmost point of the site?

| Answer | 3.94 |  |
| --- | --- | --- |

Which end of the site has the lowest level? Tick your response.

| Answer | Choice | Possible Answers |  |
| --- | --- | --- | --- |
|  |  | East |  |
|  |  | West |  |

Refer to the sectional drawing below to respond to Q29. to Q49.

Give a brief description about the building from the detail available in the sectional drawing. Is it the same structure proposed in Q19. to Q26.?

| Answer | The building is a two level, timber weatherboard residence with colorbond corrugated iron roof and verandahs. It is the same residence as Q19 to Q26. |  |
| --- | --- | --- |

Who is the client?

| Answer | H & M Smith |  |
| --- | --- | --- |

Where is this project to be built?

| Answer | Lot 16 Glenvalley Rd Richmond |  |
| --- | --- | --- |

What are the contact details for the draftsman?

| Answer | Joe Foreman Ph: 03 9514 2311 Mob: 0413 113 113 18 Fairview Rd Richmond 3216 |  |
| --- | --- | --- |

What sheet number is this drawing?

| Answer | Sheet number 5 of 7 sheets |  |
| --- | --- | --- |

What timber is required for loading and non-load bearing walls?

| Answer | Load bearing walls: F5 45x90 Non-load bearing walls: F5 35x90 |  |
| --- | --- | --- |

What spacing is recommended for common studs?

| Answer | 600 max spacing |  |
| --- | --- | --- |

What type of roof truss is to be installed?

| Answer | Scissor roof trusses |  |
| --- | --- | --- |

What centres are the trusses positioned at?

| Answer | 900 spacing |  |
| --- | --- | --- |

What size are the veranda posts?

| Answer | 90x90 |  |
| --- | --- | --- |

What timber species are the veranda posts to come from?

| Answer | Oregon |  |
| --- | --- | --- |

What pitch is the roof?

| Answer | 30 degrees |  |
| --- | --- | --- |

What size are the noggings and at what spacing?

| Answer | Size: 35x70 Spacing: 1350 max |  |
| --- | --- | --- |

What size are the ceiling joists?

| Answer | 240 |  |
| --- | --- | --- |

What timber is used for roof battens and what spacing?

| Answer | Timber: Pine Spacing: 900 |  |
| --- | --- | --- |

What thickness is the plasterboard sheeting?

| Answer | 10mm |  |
| --- | --- | --- |

What type of flooring is to be installed?

| Answer | 15mm Plywood flooring |  |
| --- | --- | --- |

What is the roof cladding material?

| Answer | Colorbond corrugated metal roofing |  |
| --- | --- | --- |

Where will you find information about the footings?

| Answer | Engineers drawings |  |
| --- | --- | --- |

What do the letters ‘DAR’ refer when timber ordering?

| Answer | DAR is “Dressed all Round” timber meaning it has been pained smooth on all surfaces. |  |
| --- | --- | --- |

Explain what ‘240 x 63 LVL’ refers to on the sectional drawing. What do the letters ‘LVL’ refer to?

| Answer | Refers to the starting joist on the verandah. LVL means “Laminated Veneer Lumber” which is slices of thin wood bonded together to produce an engineered product that is as strong as hardwood. |  |
| --- | --- | --- |

Refer to the drawing below to respond to Q50. to Q65.

What is the number of the drawing?

| Answer | 2 |  |
| --- | --- | --- |

What is the Job Number?

| Answer | 00R-112 |  |
| --- | --- | --- |

What is the scale of the drawing?

| Answer | 1:100 |  |
| --- | --- | --- |

What does the drawing show?

| Answer | Elevations of new residence |  |
| --- | --- | --- |

Which view shows the front of the house?

| Answer | East elevation |  |
| --- | --- | --- |

How many motor vehicles can be parked in the garage?

| Answer | 2 |  |
| --- | --- | --- |

What are the window and doors made from?

| Answer | Windows: aluminium frames Doors: timber Sliding door: aluminium frame |  |
| --- | --- | --- |

What is the size of the eaves?

| Answer | 450 |  |
| --- | --- | --- |

What are the eaves lined with?

| Answer | Hardiflex |  |
| --- | --- | --- |

What type of roof structure is the roof built from?

| Answer | Timber framed roof trusses |  |
| --- | --- | --- |

Which Clause of the NCC should be read for more detail about roof structure in Q59. ?

| Answer | Part 3.4.3 |  |
| --- | --- | --- |

Which Australian Standard should be read for more detail about the roof structure in Q59. ?

| Answer | AS 1684 – National Timber Framing |  |
| --- | --- | --- |

What is the roof cladding materials made from?

| Answer | Concrete roof tiles |  |
| --- | --- | --- |

Which Clause of the NCC should be read for more detail about the roof cladding materials in Q62. ?

| Answer | Part 3.5.1 |  |
| --- | --- | --- |

Which Australian Standard should be read for more detail about the roof cladding materials in Q62. ?

| Answer | AS 4055 Wind Loads for Housing |  |
| --- | --- | --- |

What is the roof pitch?

| Answer | 22.5 degrees |  |
| --- | --- | --- |

Identify one (1) regulatory body you may need to submit plans to for approval?

| Answer | Local City Council |  |
| --- | --- | --- |

Briefly describe the approval process and timeframes for an approval through the regulatory body in Q66.

| Answer | Sunshine Coast Council Development application process - Lodge development application form “DA Form 2 and pay fee - Within 10 days, the assessment Manager determines if it is a “properly made application” and issues a conformation notice to the applicant (or an action notice if it is deemed to be not properly made) - Application is assessed under the Sustainable Planning Act 2009 - A decision must be made by the Assessment Manager within 35 days of a properly made application being received - Decision notice is issued |  |
| --- | --- | --- |

What is a prime cost (PC) item? Give an example.

| Answer | An estimated amount of money added to the contract value for an item or items where the actual cost will not be known until a later stage in the build. The contract value will be adjusted via a variation order when the actual cost of the prime cost item has been finalised. An example of a Prime Cost Item is tiles. The client will select the tiles they prefer at a later stage and the cost will be adjusted if required. If the prime cost amount is $10,000 in the contract and the client chooses more expensive tiles which amount to $16,000 then a variation is raised for the difference of $6,000. |  |
| --- | --- | --- |

What is a provisional sum (PS) item? Give an example.

| Answer | An estimated amount of money added to the contract value for a component of work or materials where the conditions are not known until after the work starts therefore an exact value cannot be applied. The contract value will be adjusted via a variation order when the actual cost of the provisional sum work has been finalised An example of Provisional Sum is site works. The underground conditions cannot be known until the digging starts. If rock or unfavourable material is found that increase the cost then a variation must be raised for the difference. |  |
| --- | --- | --- |

Refer to the Bill of Quantities below to respond to Q70. to Q76.

Source: http://organizedbuilders.com.au/sites/default/files/BOQ-Summary.pdf

True or False (Tick your response): This Bill of Quantities represents a job for a two-storey house.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

Identify two (2) materials being used for the floors?

| Answer | Carpet Vinyl |  |
| --- | --- | --- |

True or False (Tick your response): A landscaper has been contracted for this job.

| Answer |  | True |  |  |
| --- | --- | --- | --- | --- |
|  |  | False |  |  |

How much does the contracted plasterer cost?

| Answer | $12,900.00 |  |
| --- | --- | --- |

How much do the kitchen appliances cost?

| Answer | $2,700.00 |  |
| --- | --- | --- |

What percentage of the total cost does the fixing stage represent of the building contract?

| Answer | 23% |  |
| --- | --- | --- |

What percentage of the total cost does the completion stage represent of the building contract?

| Answer | 17% |  |
| --- | --- | --- |

Refer to the Purchase Order below to respond to Q77. to Q82.

Source: http://organizedbuilders.com.au/sites/default/files/Databuild-Bill-of-Quantities.pdf

How many Pine F5 90 x 45 studs (2.4m long) are required?

| Answer | 100 |  |
| --- | --- | --- |

If the material in Q77. increased in cost to $2.75 each, what would be the total cost for this material?

| Answer | $275 |  |
| --- | --- | --- |

How many sheet of ply are required in total (i.e. all sizes)?

| Answer | 13 sheets |  |
| --- | --- | --- |

What material has the highest total cost?

| Answer | Pine 90 x 45 mgp10 (Long Lengths) |  |
| --- | --- | --- |

How many metres are required of the material in Q80. ?

| Answer | 322.5mt |  |
| --- | --- | --- |

What beam is identified as being a better option than F17?

| Answer | E14 beam |  |
| --- | --- | --- |

Refer to the labour chart below to respond to Q83. to Q86.

Source: https://www.archsd.gov.hk/media/89170/sor2013_vol1_builders_works_2013_11_04.pdf

Based on the hourly rates given, which is the least expensive trade to contract?

| Answer | Asphalter (Road construction) |  |
| --- | --- | --- |

If the trade in Q83. were to be contracted for two (2) hours’ work, how much would this cost?

| Answer | $214.80 |  |
| --- | --- | --- |

Based on the hourly rates given, which is the most expensive trade to contract?

| Answer | Bar bender and fixer |  |
| --- | --- | --- |

If the trade in Q85. were to be contracted for six (6) hours’ work, how much would this cost?

| Answer | $1,285.80 |  |
| --- | --- | --- |

Name the building legislation (Act and Regulations) that apply within your jurisdiction and should be referenced when reading plans and building contracts.

| Answer | Building Act 1975 Queensland Building and Construction Commission Act-1991-098 Queensland Building and Construction Commission Regulation 2018 |  |
| --- | --- | --- |