Job Scopes and specifications

Before work commences pool to be covered with soft and hard cover

Deck Board Type - Brite Deck Titanium in Willow Oak 140mm x 29mm  x 6 m in length

All Steel work as per engineering drawings  (Hot dip Galvanised Cyclonic Rated)

Pool Fencing Aluminium Powder coated - single rail top and bottom (50 x 50 Post)

F14 H4 Structure Timber

Work Outline

Main Pool Section (Before the walk path)

Demolition Work

Decking Boards

Screening located around toilets and pool pump

Pool fencing

Framing

Posts

Bearer & Joists

Remove deck and under structure & Repack the ground

Cart away removed materials and debris from site

WHS Standard fence and compliance

Plumbing and Electrical Works

BBQ Pool Pump, filter and heater removed, stored and later re-instated.

Street lamp removed and re-instated or worked around and re-instated.

Main Pool Work Scopes

Decking

Supply and install Decking board in Brite Deck Titanium in Willow Oak 140mm x 29mm  x 6 m in length
Decking board to picture frame the exterior top edge of the deck

Concealed T clip system to secure the decking boards to the frame

Matching Fascia Boards to conceal the ends of joists

Include removable access panel through deck to storm water manhole where marked on drawings.

Joist/Bearer tape to be applied to all top edges of horizontal members including joists, bearers for moisture protection before decking is installed.

Install conduit piping under decking boards for future use to carry cables – to run from store room in toilet block to lamp post at pool gate on north side (Maryvale Street) and to run from store room in toilet block to lamp post  at pool gate on south side (Jephson Street).

Under-structure of Deck to be built as per engineers drawings

Deck frame to be supported with 75 x 75 x 4 mm SHS posts as per engineering drawings

New  pool fencing, gates and steps

Aluminium Powder Coated Green Pool Fence 1200mm high Panels and  maximum of 2450 mm long

Pool fence posts 50 x 50mm

Posts to be bolted to side of deck frame to suit the 1200 m high panels.

All brackets attached to posts and panels attached to bracket with short tek screws

Seven pool gates as per drawings 1200mm high with tensioned hinges and pool safe magna lock

New steps near pool hut to garden area - hardwood timber treads on propriety hot dipped galvanised steel stringers

Suitable hand rail/balustrade for steps to match pool fencing

Steps near pool equipment to be re-instated

Privacy Screens

Powder  coated aluminium privacy screens to screen toilets and pool equipment

Privacy screens to be horizontal slatted - 65mm x 16.5mm  “surf mist” colour (match pool hut)

Aluminium posts for screening to have a receiving channel to finish off ends.

Monel pop-rivets to be used.

Pool Hut – Shade Structure - Pergola

Supply and install pool hut/shade structure as shown on plans

Materials for pool hut/shade structure as per quote from Delta but adjusted to accord with plans

Four back posts of pool hut /shade structure part of the under-structure  - to run from ground to roof -  to be painted Cottage Green - as per engineer drawings - SC2 75x75x 4SHS Hot dipped galvanised.

Roof of pool hut/shade structure to be Delta Orb  with 75mm insulation

Insulated roof panel 75mm thick 4500mm long x 1000m wide top side Delta Orb profile surfmist -

under side of roof - flat profile colour surfmist installed to manufacturers specifications.

Fascia, 125mm Squareline Gutter and Trims  - to be painted Cottage Green

Roof beam to manufacturer's specifications.

Bracing to engineers specifications.  Diagonal  bracing to be galvanised hot dipped steel round bar.

Insulated wall panels behind the BBQ to be insulated flat panels to a height of 1500

Above wall panel - Aluminium powder coated panel slats - horizontal slatted - 65mm x 16.5mm - “surf mist” colour.

Aluminium posts for screening to have a receiving channel to finish off ends.

Monel pop-rivets to be used.

Behind BBQ - tiled splashback area using off-white 600h x 300w tiles

Lighting for pool hut/ shade structure as per drawings

1200 Twin LED tube batten lighting (with replaceable LED Tubes - WEATHERPROOF

All wiring concealed - to be contained in ceiling panels

3 x Light Switch as per drawings

2 x double GPO - WEATHERPROOF- 1100 mm above floor level

Re-instate  BBQ in  new pool hut/shade structure

Re-instate pool heater,  pool pump and filter

Re-instate Street Lamps

Re-Board Existing Walk Paths

New decking board - Brite Deck Titanium Willow

Matching Fascia Boards to conceal the ends of joists

Concealed T clip system to secure the decking boards to the frame

Supply and install new fencing (to match  pool fencing) posts to be bolted to the side of bearers

Fence to be affixed to cream posts and small brick wall in 2 locations as indicated on drawings

Small wooden access gate to be replaced with powder coated aluminium gate to match fencing.

STANDARD CERTIFICATION PERMIT (if required)

Certification including 1 inspection and a Form 21 on completion.